3GPP TSG-RAN2#127bis 	R2-2409093

Hefei, China, 14th – 18th Oct, 2024

Agenda item:		8.12.2 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Consideration on RAN2 impact in MIMO phase 5

Document for:	Discussion and Decision

1.	Introduction

Based on the Rel-19 MIMO WID [1], RAN1 started to discuss the item and made some agreements. In this contribution, we analyse RAN2 impacts based on RAN1 agreements.

2.	Discussion

According to [1], one of objectives for Rel-19 MIMO WI is to support UE-initiated/event-driven beam management (UEI beam management) and RAN1 made following agreements for this aspect.

[RAN1#116] Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources Other procedure(s) as required

Based on the above agreement, we can see that UEI beam management is modeled in two steps, i.e., Step 1) event detection/trigger and Step 2) beam reporting transmission.

For Step 1) event detection/trigger, some points need to be determined: Which event triggers beam reporting, Which layer initiates beam reporting, Which RS is used to detect event(s), How to measure RS and Which RS/parameter needs to be configured/updated.

Regarding Which event triggers beam reporting, Which RS is used to detect event(s) and How to measure RS, we think that this is RAN1 scope, and RAN1 already make some progress. Thus, RAN2 don’t need to discuss these points again.

Therefore, from RAN2 point of view, it is enough to discuss which layer initiates beam reporting and which RS/parameter needs to be configured/updated.

Regarding which layer initiates beam reporting, this is still remained as FFS in RAN1. However, RAN1 already agreed that UEI beam reporting is transmitted using the UCI. Thus, it would be desirable that RAN2 assume that UEI beam reporting is initiated by the physical layer because the UCI is L1 layer signaling, and RAN2 don’t need to discuss whether MAC or RRC initiates UEI beam reporting without clear RAN1 request.

Proposal 1. RAN2 assume that UEI beam reporting is initiated by the physical layer and UEI beam reporting initiated by MAC/RRC is not discussed in RAN2 until there is RAN1 input.

For the configuration/update of RS/parameter for beam reporting, RAN1 made some agreements but many things are still under discussion. For example, RAN1 agreed to configure RS for new beam measurement explicitly, but whether to update the RS by MAC CE is FFS and whether to configure additional RS is not concluded.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement. Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken.

Anyway, after RAN1 decision, RAN1 may send LS to introduce RRC parameter list (i.e., Consolidated higher layer parameters list) and a new MAC CE if needed. We think that it is sufficient to start RAN2 discussion based on RAN1 LS at that time.

Proposal 2. RAN2 do not start the discussion on the configuration/update of RS/parameter for beam reporting, until there is RAN1 input.

For Step 2) beam reporting transmission, RAN1 agreed to support Mode-A and Mode-B as follows.

[RAN1#116bis] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode-A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This option is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode-B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all options For above procedures, cross-CC beam reporting is supported for both options. - FFS: Details.

RAN1 made further agreements in the past several meetings, and to summarize RAN1 agreements, Mode-A and Model-B are modeled as follows

- Mode-A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH to request a resource for a second UL channel to carry beam report. (FFS: dedicated SR or a new UCI, FFS: one-bit or multi-bit)

  - Step 2: UE detects the UL-grant DCI format (at least DCI format 0_1/0_2) to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel - PUSCH. (FFS: the second UL channel is PUCCH).

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH notifying a second UL channel to carry beam report. (FFS: dedicated SR or a new UCI, one-bit or multi-bit)

  - Step 2: UE transmits the beam report in the second UL channel - Type 1 CG PUSCH. (FFS: the second UL channel is PUCCH, FFS: whether Type 1 CG PUSCH can carry UL-SCH or is dedicated to beam reporting)

For Step 1 of Mode-A and Mode-B, if RAN1 agreed to support dedicated SR, there is RAN2 impact to introduce SR procedure in the MAC specification. However, this is still FFS, so RAN2 don’t need to start the discussion until there is RAN1 input.

Observation 1. For beam reporting transmission, depending on whether the first PUCCH is dedicated SR or not, there is RAN2 impact to introduce SR procedure, but RAN2 don’t need to start the discuss until there is RAN1 input.

For Step 2 and Step 3 of Mode-A, the UE receives a DCI to indicate to transmit beam reporting, then the UE transmits beam reporting. This is similar to the legacy aperiodic CSI reporting and we think there is no RAN2 impact, except RRC configuration.

Observation 2. For Step 2 and Mode 3 of Mode-A, there is no RAN2 impact except RRC configuration.

For Step 2 of Mode-B, if RAN1 agreed to support Type 1 CG PUSCH dedicated to beam reporting, there may be RAN2 impact. For example, RAN2 need to discuss how to check whether CG occasion can be used for UL-SCH or for beam reporting only and how to handle the overlapping case between the dedicated CG PUSCH and other PUSCH/PUCCH. However, this is still FFS, so RAN2 don’t need to start the discussion until there is RAN1 input.

Observation 3. For Step 2 of Mode-B, depending on whether Type 1 CG PUSCH is dedicated to beam reporting or not, there is RAN2 impact, but RAN2 don’t need to start discussion until there is RAN1 input.

Based on Observation 1, 2 and 3, we propose that RAN2 don’t start to discuss beam reporting transmission until there is RAN1 input

Proposal 3. RAN2 do not start the discussion on beam reporting transmission until there is RAN1 input.

Meanwhile, according to [1], another objective for Rel-19 MIMO is the enhancement for asymmetric DL sTRP/UL mTRP and RAN1 made following agreements.

[RAN1#117] Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). [RAN1#118] Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB. - FFS: Extending the range to lower than -10dB - Step size is 4dB

According to RAN1 agreement, RAN1 agreed to support to associate joint/UL TCI state and PL offset. When a joint/UL TCI state associated with a PL offset is applied for PUSCH/PUCCH/SRS transmission, the UE Tx power for PUSCH/PUCCH/SRS is calculated based on PL offset associated with the joint/UL TCI state. The association between PL offset and a joint/UL TCI state is configured by RRC and the PL offset value is updated by a MAC CE.

For the association between PL offset and joint/UL TCI state, RAN2 need to discuss RRC configuration to associate PL offset and joint/UL TCI state. For example, whether to reuse the legacy joint/UL TCI-state list or to introduce new joint/new TCI-state list.

Proposal 4. RAN2 discuss RRC configuration to associate PL offset and joint/UL TCI state.

In addition, for the update of PL offset, RAN2 need to discuss a MAC CE format to update PL offset associated with one joint/UL TCI state. For example, whether to introduce a new MAC CE to update PL offset only or to enhance the legacy TCI state MAC CE.

Proposal 5. RAN2 discuss MAC CE format to update PL offset associated with one joint/UL TCI state.

In addition, RAN1 also agreed to support applying PL offset on PDCCH-order based PRACH. When one of joint/UL TCI state is indicated by PDCCH-order, a PL offset associated with the indicated joint/UL TCI state is applied on PDCCH order PRACH transmission and Tx power is calculated based on the PL offset.

For PL offset on PDCCH-order based PRACH, from RAN2 point of view, we think no further RAN2 work is needed except RRC configuration to associate PL offset with joint/UL TCI state, proposed in P5.

Observation 4. No further RAN2 work is needed for PL offset on PDCCH-order based PRACH.

3.	Conclusion

In this document, we analysis RAN2 impacts based on RAN1 agreements for MIMO Phase 5, and made observations and proposals as follows.

Observation 1. For beam reporting transmission, depending on whether the first PUCCH is dedicated SR or not, there is RAN2 impact to introduce SR procedure, but RAN2 don’t need to start the discuss until there is RAN1 input.

Observation 2. For Step 2 and Mode 3 of Mode-A, there is no RAN2 impact except RRC configuration.

Observation 3. For Step 2 of Mode-B, depending on whether Type 1 CG PUSCH is dedicated to beam reporting or not, there is RAN2 impact, but RAN2 don’t need to start discussion until there is RAN1 input.

Observation 4. No further RAN2 work is needed for PL offset on PDCCH-order based PRACH.

Proposal 1. RAN2 assume that UEI beam reporting is initiated by the physical layer and UEI beam reporting initiated by MAC/RRC is not discussed in RAN2 until there is RAN1 input.

Proposal 2. RAN2 do not start the discussion on the configuration/update of RS/parameter for beam reporting, until there is RAN1 input.

Proposal 3. RAN2 do not start the discussion on beam reporting transmission until there is RAN1 input.

Proposal 4. RAN2 discuss RRC configuration to associate PL offset and joint/UL TCI state.

Proposal 5. RAN2 discuss MAC CE format to update PL offset associated with one joint/UL TCI state.

4.	Reference

- RP-242394	WID revision: NR MIMO Phase 5	Samsung