3GPP TSG-RAN WG2 Meeting #127bis	R2-2409096

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:	8.6.3

Source:	Samsung

Title:	Support of Event Triggered L1 Measurement Reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

During RAN2#126 and RAN2#127 meetings, RAN2 reached the following agreements for event-triggered L1 measurement enhancements for LTM.

RAN2#126 Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2#127 Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

RAN2#127 Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we will further address event triggered L1 measurement reporting in detail.

Discussion

Introduction

Issue 1: How to associate the LTM measurement resource configuration and the LTM event triggered report configuration

According to the RAN2 agreements, LTM measurement resource configuration is provided in LTM-Config and the event triggered report configuration is provided in the serving cell configuration i.e. same with the Rel-18 LTM CSI resource configuration and report configuration. For detail RRC configuration, how to configure/associate the LTM measurement resource configuration and event triggered report configuration is the remaining issue. For the LTM CSI resource configuration, we think the legacy LTM-CSI-ResourceConfig-r18 could be reused (or slightly updated) for the Rel-19 LTM measurement resource configuration for the event triggered report. It means the new IE for the separate LTM CSI measurement resource configuration is not really required.

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

The differentiation of reporting L1 measurement between Rel-18 L1 measurement report and the Rel-19 event triggered measurement report is achieved by the report configurations. Rel-18 LTM CSI report configuration already link the report configuration using the association of LTM CSI resource configuration (i.e. report configuration refers the index of the LTM measurement resource configuration). We think the same structure could be used for Rel-19 event triggered report configuration. For the detail RRC signalling for the event triggered L1 reporting, we think there are two options:

- Use the legacy LTM-CSI-ReportConfig-r18 and add the new configuration for the LTM event triggered measurement report by the NCE manner.

- Introduce the new IE for the LTM event triggered measurement report (e.g. LTM-CSI-EventReportConfig-r19)

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

We think there are no problems if we just use the legacy LTM-CSI-ReportConfig-r18 and add the new configuration for the LTM event triggered measurement report by the NCE manner. This structure is very similar with the Layer3 ReportConfigNR so the only need to consider is introduce the new events and relevant parameters (thresholds, offset, hysteresis, timeToTrigger, etc.) in LTM-CSI-ReportConfig-r18.

We provide the TP as an example in Annex below.

Proposal 1: LTM measurement resource configuration for the event triggered report is provided in LTM-CSI-ResourceConfig-r18.

Proposal 2: LTM event triggered measurement report configuration is provided in the LTM-CSI-ReportConfig-r18 i.e. add the new events and relevant parameters (thresholds, offset, hysteresis, timeToTrigger, etc.) in LTM-CSI-ReportConfig-r18.

Issue 2: Contents of the event triggered L1 measurement reporting

We think the limitation of the size of L1 measurement reporting is now reduced because RAN2 agreed to use the new MAC CE for the event triggered L1 measurement reporting. We also think that the MAC CE could carry many information rather than the UCI but even the event triggered L1 measurement reporting is reported by the new MAC CE, we think RAN2 first needs to consider the essential parameters. For the contents of the event based L1 measurement reporting, we think the multiple beams of the neighbouring cell could be reported based on the network configuration along with the current beam. In addition, the information to associate the measurement event ID could be added in order to identify for the NW to know what event is triggered. We think the additional information could be added and further discussed, but we prefer to add only the essential information is enough. The legacy CSI measurement report (i.e. legacy L1 report and LTM report) includes the L1 RSRP measurement report for the configured CSI resources but the event L1 measurement report don’t need to report all configured RSes. UE only need to report the L1 measurement report with some selected CSI resources (i.e. RSes which meet the event condition). We think the below information could be the essential and could be the baseline.

- Index of the LTM CSI report for the event triggered reporting

- L1 RSRP of the current beam

- L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

- DL RS resource index (SSB or CSI-RS) and the associated LTM candidate ID

RAN2 could discuss on other information of the event triggered measurement report as below but we think these information are not essential, so we don’t think any further information are needed unless there are good reason to have it from the network perspective.

- Other types of RS for the measurement report (e.g. L1-SINR)

Proposal 3: Event triggered L1 measurement reporting for LTM includes following information:

- Index of the LTM CSI report for the event triggered reporting

- L1 RSRP of the current beam

- L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

- DL RS resource index (SSB or CSI-RS) and the associated LTM candidate ID

Issue 3: Need of measurement reporting once leaving condition is met

In the legacy L3 measurement reporting, measurement report once the leaving condition is met is supported and it is very helpful for the network to operate the mobility. However, we think the L1 measurement report is not L3 filtered information and the quality of the best beam(s) could be changed dynamically even though the additional methods (e.g. TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset) will be introduced. Frequent ping-pong L1 measurement report is one of the concerns to introduce the event triggered L1 measurement report. We think the frequent L1 measurement report will be estimated if we introduce the “leaving condition” for the additional report.

Proposal 4: Do not support the event triggered L1 measurement reporting once leaving condition is met to avoid the frequent L1 measurement report.

Issue 4: Need of support periodic L1 measurement reports after the event has been triggered

Based on the RAN2 agreements, UE report the event triggered L1 measurement reporting when any beam in candidate RS configuration meets the LTM events. Then, the next question could be if this event triggered measurement report is one-shot report or multiple report which can be achieved by configuring the period and the reported numbers. From our understanding, the only one event triggered measurement report is enough because we think this event triggered measurement report already have the L1 filtering and other anti ping-pong methods (e.g. TTT, hysteresis, etc.). It means this event triggered measurement report itself seems stable enough, and this event triggered measurement report MAC CE could get the reliability through HARQ. In addition, network has no idea when this report is triggered so providing the grant for this purpose create the waste of resources. There is semi-persistent CSI reporting. NW can use it if it needs to activate semi-persistent reporting whenever it needs.

Proposal 5: Do not support the periodic L1 measurement reports after the event has been triggered.

Issue 5: What is the current beam for the LTM measurement event evaluation

RAN2 agreed to the current beam indicated by DCI but we think the current beam would be indicated beam for PDCCH or PDSCH, so it is not clear which indicated beam is used for the current beam.

From our understanding, UE could be indicated the beam by unified TCI state MAC CE but this indicated beam(s) would be different applied for each channel (PDCCH/PDSCH/PUCCH/PUSCH). Please see the below Figure 1 and Table 1 how the TCI state is applied on the different channels. In addition, we think the Rel-19 LTM is also configured with multiple TRPs, then the problem is more complicate.

Figure 1. Operation on Unified TCI framework

Table 2. TCI selection scheme for the different channels

| Channel/signal | TCI selection scheme |
| --- | --- |
| PDCCH | RRC config. (1st/2nd/both/non) per CORESET |
| PDSCH by DCI format 1_1/1_2 | Determining by TCI selection field in the DCI |
| PDSCH by DCI format 1_0 | RRC config. (1st/2nd/both) |
| DG, Type-2 CG PUSCH by DCI format 0_1/0_2 | Determining by SRS resource set indicator in the DCI |
| DG, Type-2 CG PUSCH by DCI format 0_0 | Applying 1st indicated TCI state |
| Type-1 CG PUSCH | RRC config. (1st/2nd/both) per Type-1 CG config |
| PUCCH | RRC config. (1st/2nd/both) per PUCCH resource/resource group |
| AP CSI-RS for CSI/BM | RRC config. (1st/2nd) per CSI-RS resource or per resource set |
| P/SP/AP SRS for CB/NCB/AS & AP SRS for BM | RRC config. (1st/2nd) per SRS resource set |

For example, considering the mTRP case, the unified TCI state MAC CE could activate the two TCI states (one for TRP1, the other for TRP2). In addition, RRC configures the applied TCI state per TRP (first/second/both/none)

-	PDCCH could configured by first TCI state

-	PDSCH could be configured by second TCI state

In that case, it is not clear which TCI state is applied for the current beam.

NOTE: R19 MIMO only focus on the sTRP scenario but we think R19 mobility WI doesn’t have this restriction. In addition, R18 LTM already support the mTRP with LTM so it is reasonable the Rel-19 LTM also support the mTRP with LTM.

So, we think at least initial RAN2 discussion is needed, the candidate way forward from my side is that:

- RAN2 first discuss if mTRP with inter-CU LTM is supported, here, mTRP means the source cell is configured with mTRP and two TCI state could be activated.

- If yes, we need to ask it to RAN1 and which beam is used for the current beam (i.e. either indicated PDCCH beam or PDSCH beam if both beams are different)

- If no, RAN capture the restricted scenario as an agreement.

Or, Like Rel-19 MIMO WI, it would be reasonable for Rel-19 Mob WI also need to focus only for sTRP case i.e. mTRP with Rel-19 Mob enhancements are not supported. In that case, the only thing to clarify is that which beam of the channel is used for the current beam and it could be answered by RAN1.

Proposal 6-1: RAN2 to discuss whether sTRP is prioritized or mTRP is also considered for Rel-19.

Proposal 6-2: RAN2 ask to RAN1 which channel beam is used for the current beam for the event triggered measurement report i.e. what is the indicated beam for the current beam.

Issue 6: How to align the type of beams used for the LTM measurement event evaluation

RAN2 agreed to support the both SSB and CSI-RS for L1 measurement resource configuration in LTM configuration. Then, the next question is how to guarantee the same type of RS between the current beam and the new beam for the candidate cells. There are the possibilities UE could configured the different type of the beams between the current beam and the candidate beams because the indicated TCI state is dynamically changed. We think some tricky network configuration solve the issues, see the below options.

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

- Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement.

- Option 4: Rely on network configuration, i.e. same RS type is configured.

We think Option 1 achieves the better performance than Option 2 but only have small signalling overhead than Option 2. For Option 1, UE determines the type of the target beam based on the current UE beam i.e. if the indicated beam is SSB then UE compare the SSB of the target beam from both configured beams. In addition, RAN2 need to ask to RAN4 if the Option 3 is feasible or not.

Proposal 7: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: LTM measurement resource configuration for the event triggered report is provided in LTM-CSI-ResourceConfig-r18.

Proposal 2: LTM event triggered measurement report configuration is provided in the LTM-CSI-ReportConfig-r18 i.e. add the new events and relevant parameters (thresholds, offset, hysteresis, timeToTrigger, etc.) in LTM-CSI-ReportConfig-r18.

Proposal 3: Event triggered L1 measurement reporting for LTM includes following information:

- Index of the LTM CSI report for the event triggered reporting

- L1 RSRP of the current beam

- L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

- DL RS resource index (SSB or CSI-RS) and the associated LTM candidate ID

Proposal 4: Do not support the event triggered L1 measurement reporting once leaving condition is met to avoid the frequent L1 measurement report.

Proposal 5: Do not support the periodic L1 measurement reports after the event has been triggered.

Proposal 6-1: RAN2 to discuss whether sTRP is prioritized or mTRP is also considered for Rel-19.

Proposal 6-2: RAN2 ask to RAN1 which channel beam is used for the current beam for the event triggered measurement report i.e. what is the indicated beam for the current beam.

Proposal 7: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

Annex: TP for LTM measurement resource and event triggered report configurations

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure report on the cell in which the LTM-CSI-ReportConfig is included.

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

[[

eventTriggered-r19                        SEQUENCE {

eventId                                     CHOICE {

eventLTM2                                     SEQUENCE {

a2-Threshold                                LTM-MeasTriggerQuantity,

hysteresis                                  LTM-Hysteresis,

timeToTrigger                               LTM-TimeToTrigger

},

eventLTM3                                     SEQUENCE {

a3-Offset                                   LTM-MeasTriggerQuantityOffset,

a3-Threshold                                LTM-MeasTriggerQuantity,

hysteresis                                  LTM-Hysteresis,

timeToTrigger                               LTM-TimeToTrigger

},

eventLTM4                                     SEQUENCE {

a4-Threshold                                LTM-MeasTriggerQuantity,

hysteresis                                  LTM-Hysteresis,

timeToTrigger                               LTM-TimeToTrigger

},

eventLTM5                                     SEQUENCE {

a5-Threshold1                               LTM-MeasTriggerQuantity,

a5-Threshold2                               LTM-MeasTriggerQuantity,

hysteresis                                  LTM-Hysteresis,

timeToTrigger                               LTM-TimeToTrigger

},

...

},

}

]]

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP