3GPP TSG-RAN WG2 #127bis	R2-2409102

# Hefei, China, Oct 14th – 18th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5078 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the RLM and BFD relaxation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The relaxed measurement criterion of good serving cell quality for RLM is fulfilled when the downlink radio link quality on the configured RLM-RS resource is evaluated to be better than the threshold Qin+XdB. However, Qin is a value in percentage (BLER) but X is a value in dB, we should not sum up these two values directly. Similarly, the relaxed measurement criterion of good serving cell quality for BFD also has a similar problem that needs to be fixed. === TS 38.133 === 8.1.1 Introduction … The threshold Qin is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout and shall correspond to the in-sync block error rate (BLERin) as defined in Table 8.1.1-1. For SSB based radio link monitoring… |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the “relaxed measurement criterion for good serving cell quality” is determined based on the downlink radio link quality (e.g., SINR) of Qin, not BLER. Impact Analysis Impacted 5G architecture options: NR-SA Impacted functionality: NR RLM and BFD Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior regarding when to perform the RLM/BFD measurement relaxation under the ‘good serving cell quality’ condition is ambigious. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the Changes

### 5.7.13	RLM/BFD relaxation

#### 5.7.13.0	General

In case both low mobility criterion and good serving cell criterion are configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when both relaxed measurement criterion for low mobility and relaxed measurement criterion for good serving cell quality are met.

In case only the good serving cell quality criterion is configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when the relaxed measurement criterion for good serving cell quality is met.

When the short DRX cycle is smaller than or equal to 80 ms and the long DRX cycle is larger than 80 ms then the UE is allowed to perform RLM and/or BFD relaxation while drx-ShortCycleTimer is running and the relaxed measurement criteria are met. In this case the UE does not report change of its relaxation state for RLM and/or BFD relaxation due to start or stop of drx-ShortCycleTimer, if configured.

#### 5.7.13.1	Relaxed measurement criterion for low mobility

The relaxed measurement criterion for UE with low mobility in RRC_CONNECTED is fulfilled when:

-	(SS-RSRPRef – SS-RSRP) < SSearchDeltaP-Connected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the SpCell based on SSB (dB).

-	SS-RSRPRef = reference L3 RSRP measurement of the SpCell based on SSB (dB), set as follows:

-	After receiving low mobility criterion configuration, or

-	After MAC of the CG successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig of the CG while low mobility criterion is configured, or

-	upon an indication from lower layers that a RACH-less handover or an LTM cell switch is successfully completed for the CG while low mobility criterion is configured, or

-	If (SS-RSRP - SS-RSRPRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Connected:

-	The UE shall set the value of SS-RSRPRef to the current SS-RSRP value of the SpCell.

#### 5.7.13.2	Relaxed measurement criterion for good serving cell quality

The relaxed measurement criterion of good serving cell quality for RLM is fulfilled when the downlink radio link quality on the configured RLM-RS resource is evaluated to be XdB better than the downlink radio link quality corresponding to the threshold Qin, wherein

-	Qin is specified in clause 8.1 of TS 38.133 [14].

-	X is the parameter offset in goodServingCellEvaluationRLM for the evaluated serving cell.

The relaxed measurement criterion of good serving cell quality for BFD is fulfilled when the downlink radio link quality on the configured BFD-RS resource is evaluated to be XdB better than the downlink radio link quality corresponding to the threshold Qin, wherein

-	Q in is specified in clause 8.1 of TS 38.133 [14].

-	X is the parameter offset in goodServingCellEvaluationBFD for the evaluated serving cell.

End of the Changes