3GPP TSG-RAN WG2 Meeting #127bis										      R2-2409107

Hefei, China, Oct 14th – 18th, 2024

Source: 			CEWiT

Title: 	Discussion on Functionality-based LCM of UE-sided Model for Beam Management Use Cases

Agenda item:    		8.1.2.2

Document for: 		Discussion and Decision

# Introduction

In Rel 18 the application of AI/ML techniques to NR air interface has been studied and captured in TR 38.843 [1]. Based on this normative phase is agreed in Rel 19 with following objective.

Provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management – DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, LCM-related aspects of UE-sided models for beam management use cases are proposed to continue the discussion in RAN 2.

# Discussion

The following agreements were made during RAN2#127 meeting on LCM for the UE-sided model focusing on the inference procedures.

Agreements on definitions

Supported functionalities refer to functionalities that UE can 	indicate by using UE capability information (via RRC/LPP signalling).

Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

Activated functionalities refers to functionalities already enabled for performing inference.

Agreements on procedures

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

“Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

“Step 4”: UE reports applicable functionality in the following scenarios

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality.

We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

  - Functionality Identification

According to TR 38.843, the functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. The UE capability message shall indicate the list of supported functionalities by the UE. The supported functionalities can be indicated with a functionality identifier. Whether the network or the UE assigns the functionality ID can be discussed further.

Configured functionalities refer to the functionalities that the network has configured to the UE. However, getting configured by the network doesn’t imply that the corresponding functionalities are applicable. The network is expected to maintain the corresponding functionality identifiers while configuring the respective functionalities.

Applicable functionalities refer to the functionalities that the UE can use for inference. Applicable functionalities can be considered as candidates for functionality activation/switching/deactivation. Applicable functionality is determined based on the scenarios, locations, configuration, and deployments, among other factors, and the UE reports it to the network either reactively or proactively. In reactive reporting of UE-sided applicable functionality(es), the network may request for the applicable functionality(es) and the UE provides the relevant information in response via UAI message. In proactive reporting, the UE reports its applicable AI/ML functionalities unsolicited via UAI message. While the UE reports the applicable functionalities, it is expected to use the corresponding functionality identifiers to maintain consistency.

Applicable functionalities are a subset of the configured functionalities. Suppose a UE is configured with functionalities that are not applicable under some conditions but may be applicable under some other conditions. In that case, the functionality activating/switching/deactivating feature can be seamlessly performed using the configured functionality list. The UE need not wait for the configurations from the network to activate the newly applicable functionality. However, it is up to the network whether the configuration shall be provided to the UE before determining the applicable functionalities. In such scenarios, the applicable functionalities and the configured functionalities would be the same.

Activated functionalities refer to the functionalities that the UE is using for inference. Activated functionalities are a subset of applicable functionalities. The UE shall indicate the currently activated functionalities to the network and if any switching/deactivation is performed, it shall be informed to the network. The corresponding functionality identifiers are used for any reporting or configurations.

The model availability is not mandated until the stage of inference. Once the applicable functionalities are decided at the UE end, the UE can opt for means to obtain the required model(s) from the OTT server, gNB, CN, or OAM, if not already available with the UE. If UE has multiple applicable models available or transferred from external entities, it can decide which one to use for inference.

Available functionalities refer to the functionalities for which the UE already has model(s) with it. A model available does not imply it is configured/applicable/activated. It shall just indicate the respective functionality is supported but not vice versa. If a functionality has multiple models, model identifiers are used to uniquely identify them. Similarly, if a model can be associated with multiple functionalities, functionality identifiers can be used.

Proposal 1: The functionalities shall be associated with a corresponding functionality identifier. FFS which entity assigns the functionality IDs.

Proposal 2: Consistency of the functionality IDs shall be maintained across different levels of functionalities. FFS which entity manages the functionality IDs.

Proposal 3: Configured functionalities refer to the functionalities that the gNB/LMF has configured to the UE.

Proposal 4: Applicable functionalities are a subset of the configured functionalities.

Proposal 5: Available functionalities refer to the functionalities for which the UE already has model(s) with it.

  - Model Training

Since RAN2 is awaiting RAN1 reply to the LS related to functionality granularity, additional conditions, applicable functionality reporting, inference configurations, etc., this document focuses on the model training aspects of the LCM framework.

For the UE-sided model, the model training shall be performed at the network-side (gnB, CN, or OAM) or the UE-side OTT server. The model training runs periodically or intermittently in the background. The training process shall be initiated either at the UE-side or the network-side. The network shall configure the UE for the training procedures. The measurement reports such as CSI, RSRP, SINR, etc., shall be pushed to the corresponding model training entity.

Figures 1 and 2 illustrate the UE-side and the network-side training procedures.

When the model training is at the UE-side OTT server, the UE shall initiate the training procedures by requesting for the training configurations from the network using the RRC measurement configuration request message. The network shall provide the training configuration to the UE via the RRC measurement configuration message. This enables the UE to perform the necessary measurements for the beam management use case and push the measurement data to the OTT server. The OTT server shall use the measurement data to train and store the model.

When the model training is at the network-side, the network shall configure the UE to perform measurements for beam management via the RRC measurement configuration message. Once the UE is done with the measurements, the UE shall send the RRC measurement report to the network. The network shall update the model training based on the latest measurement report from the UE.

Figure 1: UE-side Model Training

Figure 2: Network-side Model Training

Proposal 6: For the UE-sided inference model, RAN2 shall consider two approaches for the model training:

Option 1: UE-side model training

Option 2: NW-side model training

Proposal 7: For UE-side model training, Figure 1 shall be captured for the model training signaling procedures.

Proposal 8: For NW-side model training, Figure 2 shall be captured for the model training signaling procedures.

Proposal 9: The start/stop of the training procedure for the UE-side model training is up to the UE implementation.

Proposal 10: The start/stop of the training procedure for the NW-side model training is decided by the network.

# Conclusion

This contribution provides the following proposals for functionality-based LCM of the UE-sided model for the beam management use case.

Proposal 1: The functionalities shall be associated with a corresponding functionality identifier. FFS which entity assigns the functionality IDs.

Proposal 2: Consistency of the functionality IDs shall be maintained across different levels of functionalities. FFS which entity manages the functionality IDs.

Proposal 3: Configured functionalities refer to the functionalities that the gNB/LMF has configured to the UE.

Proposal 4: Applicable functionalities are a subset of the configured functionalities.

Proposal 5: Available functionalities refer to the functionalities for which the UE already has model(s) with it.

Proposal 6: For the UE-sided model, RAN2 shall consider two approaches for the model training:

Option 1: UE-side model training

Option 2: NW-side model training

Proposal 7: For UE-side model training, Figure 1 shall be captured for the model training signaling procedures.

Proposal 8: For NW-side model training, Figure 2 shall be captured for the model training signaling procedures.

Proposal 9: The start/stop of the training procedure for the UE-side model training is up to the UE implementation.

Proposal 10: The start/stop of the training procedure for the NW-side model training is decided by the network.

# Reference

[1] TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[2] RAN2#127 Chair Notes