3GPP TSG-RAN WG2 Meeting #127bis										      R2-2409108

Hefei, China, Oct 14th – 18th, 2024

Source: 			CEWiT

Title: 	Discussion on Functionality-based LCM for Positioning Use Case

Agenda item:    		8.1.2.3

Document for: 		Discussion and Decision

# Introduction

In Rel 18 the application of AI/ML techniques to NR air interface has been studied and captured in TR 38.843 [1]. Based on this normative phase is agreed in Rel 19 with following objective.

Provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management – DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, LCM-related aspects of UE-sided models for positioning use cases are proposed to continue the discussion in RAN 2.

# Discussion

The following agreements were made during RAN2#125bis, RAN2#126, and RAN2#127 meetings on the LCM for the positioning use case.

RAN2#125bis

Agreements

For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs.

For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality.

FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed).

RAN2#126

Agreements for NW-side model

RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements for UE-side model

The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and 	understanding of the functionality.

Wait for RAN1 for associate ID discussion.

At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements.

RAN2#127

Agreements on definitions

Supported functionalities refer to functionalities that UE can 	indicate by using UE capability information (via RRC/LPP signalling).

Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

Activated functionalities refers to functionalities already enabled for performing inference.

This document focuses on case 1 – UE-based positioning with UE-side model, direct AI/ML positioning.

  - Inference Phase

Figure 1 illustrates the signaling involved during the inference operation for a UE-sided model.

Figure 1: Generic Signaling for Inference Configurations

Figure 2: Inference Configurations Signaling for UE-based Positioning with UE-side Model

The LMF shall initiate capability transfer which involves requesting the UE capability using the RequestCapabilities LPP message. This can also be an optional message where the UE shall indicate the capabilities to the LMF unsolicitedly. The capability request seeks the supported functionalities of the UE.  The UE shall indicate its capabilities to the LMF via ProvideCapabilities LPP message. This includes either the entire list of supported functionalities or the specific ones as requested by the LMF. The UE shall optionally indicate the model availability status corresponding to the supported functionalities to the LMF.

Once the LMF obtains the list of supported functionalities, the LMF shall assist the UE in determining the applicable functionalities. To do that, it may require knowledge of both the UE-side and the NW-side additional conditions. These additional conditions are the conditions apart from those included in the supported functionalities. The UE shall either request for NW-side additional conditions using RequestAssistanceData LPP message or the LMF shall provide the NW-side additional conditions unsolicitedly using ProvideAssistanceData LPP message or solicited as response to the request from the UE. For the UE-side model, the decision to select the applicable functionalities is taken at the UE end. Based on the NW-side and the UE-side additional conditions, the UE shall report the applicable functionalities to the LMF using ProvideAssistanceData LPP message unsolicitedly or as response to the LMF, if the LMF sends RequestAssistanceData LPP message seeking applicable functionalities report.

Once the model is ready at the UE end, the LMF shall configure the UE for inference procedure using RequestLocationInformation LPP message. This message may include the positioning reference signal (PRS) configurations from the serving gNB and neighboring gNB(s), if applicable, which the UE shall use to measure and estimate the channel conditions. This message shall also include the activation status for the functionalities configured for inference. The UE shall perform the inference on the model activated and reports the results to the LMF using ProvideLocationInformation LPP message, if applicable.

Proposal 1: The UE shall indicate the supported functionalities to the LMF solicitedly or unsolicitedly.

Proposal 2: The LMF shall configure the applicable functionalities to the UE including the NW-side additional conditions using the LPP message solicitedly or unsolicitedly.

Proposal 3: The UE shall request for NW-side additional conditions from the LMF using the LPP message.

Proposal 4: The LMF shall configure the inference configurations, if not already configured, or update the inference configurations, if needed, using the LPP message.

Proposal 5: The UE shall send the inference results to the LMF using the LPP message, if applicable.

  - Training Phase

For the UE-sided model, the model training shall be performed at the LMF-side or the UE-side OTT server. The model training runs periodically or intermittently in the background. The training process shall be initiated either at the UE-side or the LMF-side. The LMF shall configure the UE for the training procedures. The measurement reports such as CSI, RSRP, SINR, etc., shall be pushed to the corresponding model training entity.

When the model training is at the UE-side OTT server, the UE shall initiate the training procedures by requesting for the training configurations from the LMF using the LPP message. The LMF shall provide the training configuration to the UE via the LPP message. This enables the UE to perform the necessary measurements from the serving gNB/TRP and/or the neighboring gNB(s)/TRP(s) for the positioning use case and push the measurement data to the OTT server. The OTT server shall use the measurement data to train and store the model.

When the model training is at the LMF-side, the LMF shall configure the UE to perform the measurements for positioning via the unsolicited LPP message. Once the UE is done with the measurements from the serving gNB/TRP and/or the neighboring gNB(s)/TRP(s), the UE shall send the measurement report to the LMF using the unsolicited LPP message. The LMF shall update the model training based on the latest measurement report from the UE.

Proposal 6: For the UE-sided inference model, RAN2 shall consider two approaches for the model training:

Option 1: UE-side model training

Option 2: NW-side model training

Proposal 7: The UE shall request the training configurations from the LMF using the LPP message.

Proposal 8: The LMF shall configure the UE for training purposes using the LPP message solicitedly or unsolicitedly.

Proposal 9: For the LMF-side model training, the UE shall send the measurements results to the LMF using the LPP message.

Proposal 10: The start/stop of the training procedure for the UE-side model training is up to the UE implementation.

Proposal 11: The start/stop of the training procedure for the LMF-side model training is decided by the LMF.

# Conclusion

This contribution provides the following proposals for functionality-based LCM of the UE-sided model for the positioning use case.

Proposal 1: The UE shall indicate the supported functionalities to the LMF solicitedly or unsolicitedly.

Proposal 2: The LMF shall configure the applicable functionalities to the UE including the NW-side additional conditions using the LPP message solicitedly or unsolicitedly.

Proposal 3: The UE shall request for NW-side additional conditions from the LMF using the LPP message.

Proposal 4: The LMF shall configure the inference configurations, if not already configured, or update the inference configurations, if needed, using the LPP message.

Proposal 5: The UE shall send the inference results to the LMF using the LPP message, if applicable.

Proposal 6: For the UE-sided inference model, RAN2 shall consider two approaches for the model training:

Option 1: UE-side model training

Option 2: NW-side model training

Proposal 7: The UE shall request the training configurations from the LMF using the LPP message.

Proposal 8: The LMF shall configure the UE for training purposes using the LPP message solicitedly or unsolicitedly.

Proposal 9: For the LMF-side model training, the UE shall send the measurements results to the LMF using the LPP message.

Proposal 10: The start/stop of the training procedure for the UE-side model training is up to the UE implementation.

Proposal 11: The start/stop of the training procedure for the LMF-side model training is decided by the LMF.

# Reference

[1] TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[2] RAN2#125bis Chair Notes

[3] RAN2#126 Chair Notes

[4] RAN2#127 Chair Notes