3GPP TSG-RAN WG2 Meeting #127bis	R2-2409113

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:		8.8.4 Support of broadcast service

Title:		Remaining issues for the support of broadcast service in NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this paper, we would like to provide our views on the highlighted aspects on support of broadcast service in NTN.

# 2. Discussion

During the 127th meeting of RAN WG2, significant progress was made in supporting MBS broadcast services in NTN. The remaining issues that still need to be addressed are as follows:

- How to provide MBS broadcast intended service area details to UE: SIB20 vs MCCH.

- How to associate the service area information with MBS sessions.

- The impact of the intended service area concept on legacy MBS service continuity mechanisms.

2.1	How to provide the intended service area info.: SIB20 vs MCCH

During the 127th meeting, RAN WG2 discussed two potential options for providing the intended service area information to UEs: SIB20 vs MBSBroadcastConfiguration in MCCH.

- SIB20: Contains information required to acquire the MCCH/MTCH configuration for MBS broadcast

- MBSBroadcastConfiguration: Control information applicable for MBS broadcast services transmitted via broadcast MRB

Before discussing about which of the options is more appropriate, we should make consensus on how much details of the intended service area information should be included in SIB20 or MCCH. The following options are under considerations:

- Option 1: Provide all intended service area details, including Service Area IDs and detailed information for each area (circles or polygons).

- Option 2: Provide only Service Area IDs, with detailed area information provided elsewhere (e.g., USD or a new SIB).

Proposal 1: RAN WG2 should first reach consensus on the level of detail for intended service area information included in SIB20 or MCCH (All information vs. only Service Area IDs).

ETRI believes that if Option 1 is selected, SIB20 may not be suitable due to its limited payload size (e.g., 2976 bits), considering that a lot of service areas could be defined within a wide satellite beam coverage area and a few tens to hundreds of bits might be required for the details of each service area in a format of circle or polygons. Additionally, SIB20 does not currently include MBS session IDs, whereas MCCH already contains this information. Therefore, considering the intended service area ID should be associated with MBS session ID, MCCH would be more appropriate.

Observation 1: SIB20 may not be suitable due to limited payload size if full details for intended service area information should be included in SIB20 or MCCH.

Additionally, Additionally, given the legacy purpose of each SIB, ETRI suggests that providing the intended service area information via the MBSBroadcastConfiguration in MCCH is more appropriate. Because the intended service area can be regarded as part of the control information for providing area-specific MBS broadcast services for each MBS session.

Observation 2: The intended service area information can be regarded as part of the control information for providing area-specific broadcast service for each MBS session.

During the 127th meeting of RAN WG2, it was discussed and understood that the expected UE behavior is such that when the UE is not located in any intended service area for its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH information. If the intended service area information is provided via SIB20, the UE could avoid unnecessary (re)acquisition of MBSBroadcastConfiguration messages when it is outside of the intended service areas after receiving SIB20.

On the other hand, if the intended service area information is provided via MCCH, the UE can store this information, which may have been received through a previous MCCH or USD message, and then determine whether or not it needs to (re)acquire the up-to-date MCCH information based on its current location and the stored intended service areas.

Therefore, regardless of whether the intended service area information is provided via SIB20 or MCCH, the UE can be implemented in a way that allows it to avoid (re)acquiring up-to-date MCCH information when it is not located in any intended service area for its interested broadcast services.

Observation 3: Regardless of the method (SIB20 or MCCH) used to provide the intended service area information, UE behavior can be optimized to avoid unnecessary (re)acquisition of MCCH when it is not located within the intended service areas for the relevant broadcast services.

Based on the observations discussed above, ETRI believes that providing the intended service area information via MCCH is the more appropriate option. This is due to the limited payload size of SIB20, the legacy purpose of SIB20, and the fact that there is no significant benefit to UE power consumption when using SIB20 for this purpose.

Proposal 2: Considering the limited payload size of SIB20, the legacy purpose of SIB20 and MCCH, and the lack of power-saving benefits from using SIB20, RAN WG2 to decide to provide the intended service area information via MCCH.

2.2	Association between MBS broadcast intended service areas and MBS sessions

The location-dependent MBS service enables the distribution of different content data to different MBS service areas. For the support of location-dependent broadcast services, similar to the legacy MBS mechanism, the MBS broadcast intended service area ID can be used in combination with the MBS session ID to uniquely identify the service area-specific part of the MBS service within 5GC. Therefore, ETRI believes that for association with MBS sessions, the intended service area IDs should be associated with MBS session IDs with the following possibilities:

- Option 1: One intended service area ID mapped to multiple MBS session IDs.

- Option 2: Multiple intended service area IDs mapped to one MBS session ID.

Considering flexibility for the configuration of independent intended service area per each MBS session, in our view, Option 2 is preferred.

Observation 4: The intended service area ID should be associated with the MBS session ID to uniquely identify service areas within the 5GC.

Proposal 3: RAN2 to support N-to-1 mapping between multiple intended service area IDs and one MBS session ID.

Therefore, to indicate the details of the MBS broadcast intended service area associated with the MBS session ID to the UE, the following information needs to be provided via one or more signalling messages:

- The list of MBS session IDs

- The list of corresponding MBS broadcast intended service area IDs per each MBS session ID

- The service area details per each MBS intended service area ID in a format of circle(s) or polygon(s)

Proposal 4: A UE can be configured with one or more signalling messages including the following information:

- The list of MBS session IDs

  - Via reuse of legacy MCCH or addition in SIB20

- The list of corresponding intended service area IDs per each MBS session ID

  - Via addition in MCCH or SIB20

- The service area details per each MBS intended service area ID

  - Via addition in MCCH, SIB20 or USD, or definition of new SIB

2.3	Service Continuity

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

- 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

- 2) Either:

  - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

  - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

  - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

Considering that SIB21 contains the mapping relationship between the current and/or neighbouring carrier frequencies and MBS FSAI, this legacy mechanism for MBS service continuity assumes that the MBS service area is mapped onto one or more cells. However, in NTN systems, this may not work effectively because the UE may not consider that frequency as the highest priority if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session that the UE is receiving or interested in, even if the above conditions 1 and 2 are met.

Therefore, ETRI believes that enhancements to the legacy mechanism for MBS service continuity are necessary, including the following:

- To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs, and include the following information in SIB21:

  - Carrier frequency list (already existing in SIB21)

  - MBS intended service area ID list per each carrier frequency

- To update existing conditions for UE to perform MBS broadcast service cell reselection based on MBS broadcast intended service area information as followings.

  - If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the three following conditions are fulfilled:

    - 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

    - 2) Either:

      - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

      - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

      - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

    - 3) one or more MBS broadcast intended service area ID(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS broadcast intended service area ID(s) is also indicated for this MBS broadcast service session.

Observation 5: In NTN systems, the legacy MBS service continuity mechanism may not work effectively due to the lack of intended service area prioritization.

Proposal 6: RAN2 to consider the following enhancements to the legacy mechanism for MBS service continuity.

- To define a new mapping relationship between carrier frequencies and MBS intended service area IDs in SIB21

- To update the UE's cell reselection behavior based on MBS intended service area information.

# 3. Conclusions

Proposal 1: RAN WG2 should first reach consensus on the level of detail for intended service area information included in SIB20 or MCCH (All information vs. only Service Area IDs).

Observation 1: SIB20 may not be suitable due to limited payload size if full details for intended service area information should be included in SIB20 or MCCH.

Observation 2: The intended service area information can be regarded as part of the control information for providing area-specific broadcast service for each MBS session.

Observation 3: Regardless of the method (SIB20 or MCCH) used to provide the intended service area information, UE behavior can be optimized to avoid unnecessary (re)acquisition of MCCH when it is not located within the intended service areas for the relevant broadcast services.

Proposal 2: Considering the limited payload size of SIB20, the legacy purpose of SIB20 and MCCH, and the lack of power-saving benefits from using SIB20, RAN WG2 to decide to provide the intended service area information via MCCH.

Observation 4: The intended service area ID should be associated with the MBS session ID to uniquely identify service areas within the 5GC.

Proposal 3: RAN2 to support N-to-1 mapping between multiple intended service area IDs and one MBS session ID.

Proposal 4: A UE can be configured with one or more signalling messages including the following information:

- The list of MBS session IDs

- The list of corresponding intended service area IDs per each MBS session ID

- The service area details per each MBS intended service area ID

Observation 5: In NTN systems, the legacy MBS service continuity mechanism may not work effectively due to the lack of intended service area prioritization.

Proposal 6: RAN2 to consider the following enhancements to the legacy mechanism for MBS service continuity.

- To define a new mapping relationship between carrier frequencies and MBS intended service area IDs in SIB21

- To update the UE's cell reselection behavior based on MBS intended service area information.