3GPP TSG-RAN WG2 Meeting #127bis	R2-2409115

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.7.5

Source:	OPPO, China Telecom, Sharp, Samsung, HONOR, CMCC, KDDI, Apple, Intel,

vivo, MediaTek, ZTE

Title:	Discussion on Avoiding Unnecessary RLC Retransmissions

Document for:	Discussion, Agreement

Introduction

To avoid unnecessary retransmissions, the three candidate approaches identified by R2 are listed as follows:

Table 1 Conclusions for Avoiding Unnecessary Retransmissions

| Issues | Details |
| --- | --- |
| Tx initiated approach | - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side |
| Rx initiated approach | - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. |
| Combined approach | - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach |

This paper will further discuss the enhancement of avoiding unnecessary retransmission and the detailed pros and cons of each approach.

Discussion

To avoid unnecessary retransmissions, the key metric is whether they can be avoided accurately and promptly. And essentially, it is about whether and how to identify the SDU which is not useful to PDCP.

- PDU type: For PDCP control PDUs, they should not be considered as “unnecessary retransmission” since they are related to critical PDCP functions.

- Inter-PDU dependency: As explained in SA4 LS [1] and the study of Quality Evaluation Methods for Media and XR Services in TR 26.926 [2], there are dependencies between PDUs, e.g., the decoding of PDU B depends on the content of PDU A, so the discarding decision should be an overall judgment based on implementations of individual applications.

[SA4 LS] In addition, for video, data included in NAL Units are typically spatially and/or temporarily predicted from video data in other NAL Units, so some sort of cross-dependency of NAL units exists. Based on this and other potential scenarios, even mapping multiple NAL units to a single PDU Set may not be excluded to be considered as a viable setup. Generally speaking, packet losses in video applications typically result in some sort of impacted video quality. [TR 26.926] A decoding model now takes into account the S’-Trace as well as the V-Trace in order to generate a decoding model. For this, coding units in a video frame may be viewed individually and get assigned a state: - A coding unit spanning a certain area of the video frame is correct if and only if it is received correctly and it predicts for a non-damaged coding unit. Predicting from non-damaged units means that both spatial prediction as well as temporal prediction coding units are correct - If any of the two conditions do not hold, the coding unit state is damaged. Note that only non-predicted coding units will eventually remove damaged areas. The resulting trace documenting correct and damaged coding units is referred to as V’-Trace, see clause 5.6.4 for more details.

For the identification of PDCP control PDU and inter-PDU dependency, it can be achieved by Tx initiated approach since the obsolescence of SDU at Tx RLC can be determined by the Tx PDCP. However, it doesn’t work in Rx initiated approach and combined approach since Rx RLC has no knowledge of control PDU or inter-PDU dependencies.

For Tx initiated approach, since Tx side has full knowledge of PDU type, the inter-PDU dependency unnecessary retransmission can be decided accurately.

In other words, timer-based Rx initiated approach and combined approach, it may mistakenly discard PDCP control PDUs or PDUs that have dependencies with other received PDUs.

Besides control PDU and inter-PDU dependencies, if considering the delay budget of each independent data PDU, RLC layer should be able to identify the SDUs that (will be) abandoned by PDCP. Otherwise, if the discarding of packets is not aligned between RLC and PDCP, either the unnecessary retransmissions cannot be avoided, or necessary retransmissions are discarded by mistake, causing additional data loss. For Tx initiated approach, the obsolescence of SDU at Tx RLC can be determined by the Tx PDCP, which is aligned with the current mechanism.

For the Rx initiated approach and combined approach, the discard decision is made by the Rx RLC based on a pre-configured timer. Although ideally, the discard should be done upon running out of PDU delay budget, this cannot be achieved by network configuration since the start time point of the timer at Rx RLC cannot synchronize with the data delay budget:

- If the timer at Rx RLC expires before running out of the PDU delay budget, the necessary retransmission is wrongly discarded, causing data loss.

- If the timer at Rx RLC is still running after running out of PDU delay budget, unnecessary retransmission cannot be avoided.

For timer-based Rx initiated approach and combined approach, there would be either packet loss due to too-short timer length or unnecessary retransmission due to too-long timer length, which cannot be solved by network implementation of timer length setting.

Assuming the obsolete PDU is identified, the next question is whether the unnecessary retransmission of the obsolete PDU can be avoided.

- Tx initiated approach can fully avoid unnecessary retransmissions, since it is the Tx entity who performs the retransmission.

- For the Rx initiated approach, since it relies on the status report or a new indication from the Rx to Tx to indicate that retransmission is not needed, and the status report may not be sent timely (which is the motivation for timely retransmission), unnecessary retransmission cannot be avoided promptly.

Tx initiated approach can fully avoid unnecessary retransmission of obsolete PDUs, whereas unnecessary retransmission cannot be fully avoided by Rx initiated approach due to the delay of indication from Rx to Tx side.

Additionally, it is not clear how the Rx timer works. For example:

Whether the timer runs for one missing PDU or multiple missing PDUs, it makes the Rx timer

- Either wait too long (i.e., needs to wait another timer length for each PDU if it runs per missing PDU)

- Or cannot differentiate different packets (i.e., the different status of multiple-PDU is not reflected if it runs for multiple PDUs altogether).

Whether the timer starts upon RLC SN gap or PDCP SN gap, it causes

- Either if it starts once there is an RLC SN gap, then this timer may be started before PDCP reordering timer, a necessary PDU may be discarded even though the PDCP Rx entity is still waiting for it;

- Or if it starts once there is a PDCP SN gap, then it has the same starting point as t-reordering, so we can just rely on t-reordering.

For the Rx-timer, the detailed solution is not clear: 1) the selection of timer granularity may cause either additional latency or mis-discard, and 2) the selection of timer start condition may cause either mis-discard or make the new Rx timer useless.

At last, R2 #127 concluded on a general principle for all the approaches, i.e., the Tx side and Rx side shall remain in sync. This principle can be also secured by Tx initiated approach: Even if Tx window advances first, the windows at Tx side and Rx side are still always synced with each other, as long as the SN gap indication is received successfully at the Rx side (which is used to advance the Rx window) before the delivery of PDUs within the advanced window, which can be secured by SR report from Rx side. And please note that this SN gap based Tx window advance operation does not affect the delivery of PDUs located in the intersection of Tx/Rx window.

The sync of windows at Tx side and Rx side can be secured in Tx initiated approach.

In conclusion, Tx initiated approach is preferred since it solves the “unnecessary retransmission” issue more cleanly.

For avoiding unnecessary RLC AM retransmissions, RAN2 to adopt the Tx initiated approach.

Conclusion

We have the following observations:

Observation 1	For Tx initiated approach, since Tx side has full knowledge of PDU type, the inter-PDU dependency unnecessary retransmission can be decided accurately.

Observation 2	In other words, timer-based Rx initiated approach and combined approach, it may mistakenly discard PDCP control PDUs or PDUs that have dependencies with other received PDUs.

Observation 3	For timer-based Rx initiated approach and combined approach, there would be either packet loss due to too-short timer length or unnecessary retransmission due to too-long timer length, which cannot be solved by network implementation of timer length setting.

Observation 4	Tx initiated approach can fully avoid unnecessary retransmission of obsolete PDUs, whereas unnecessary retransmission cannot be fully avoided by Rx initiated approach due to the delay of indication from Rx to Tx side.

Observation 5	For the Rx-timer, the detailed solution is not clear: 1) the selection of timer granularity may cause either additional latency or mis-discard, and 2) the selection of timer start condition may cause either mis-discard or make the new Rx timer useless.

Observation 6	The sync of windows at Tx side and Rx side can be secured in Tx initiated approach.

We have the following proposals:

Proposal 1	For avoiding unnecessary RLC AM retransmissions, RAN2 to adopt the Tx initiated approach.

Reference

- S4-220505, LS Rely on QoS support with PDU set granularity, May 2022.

- TR 26.926 “Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems; (Release 18)”