3GPP TSG-RAN WG2 #127bis	R2-2409116

Hefei, China, August 2024

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on Measurement Gap enhancements

Document for:	Discussion, Agreement

Introduction

This paper will discuss the R2 impact regarding the following objective on measurement gap enhancements:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

Discussion

Currently, the UE shall not perform transmission or reception in measurement gaps or some specific occasions due to RRM requirements. While on the other hand, to support the XR service high system capacity is needed. Therefore, the enhancement to enable transmission/reception in gaps/restrictions that are caused by RRM measurements is to achieve a balance between RRM performance and system capacity.

Generally, the following two aspects for RAN2 to consider:

- Whether/how UAI helps the measurement gap skipping scheme;

- What can be transmitted in the MG/scheduling restriction occasions, e.g., whether additional LCP restriction is needed.

Issue-1: UAI

Firstly, on how to enable the transmission and reception in MG/scheduling restriction occasions, RAN1 has agreed on the following solution based on dynamic indication from the network

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

As shown in the above agreements, it is network who decides whether to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements, and there is a discussion in RAN1 that whether the assistance information from the UE is needed for network’s decision. There is no consensus in RAN1, one LS[1] is sent to RAN4 and CC RAN2. In the LS, the candidate assistance information is listed as follows

RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information.

The main motivation of assistance information from the UE is whether/how much the RRM performance is impacted from skipping measurement occasions. This relates to how the RRM requirement is defined for measurement gap skipping scenario. I.e.,

- If reusing the existing RRM requirement, the assistance information may be helpful for the network to know whether/which measurement gap to skip without impacting the RRM requirement;

- If defining new RRM requirement for measurement gap skipping scenario that is fully compatible with the skipping behavior, there seems less need for the assistance information.

This should be evaluated by RAN4, and RAN4 has begun their discussion.

On the RRM requirements

Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed.

And the need of UAI

## Issue 2-1: General on UAI <Way forward>: - Discuss if UAI is needed and which aspects to consider.

Whether the assistance information from the UE is necessary depends on RAN4 evaluation on the RRM requirement for measurement gap skipping scenario, e.g., whether new RRM requirement is defined.

For measurement gap skipping, RAN2 wait for RAN4 evaluation before working on the UE assistance information message design.

If RAN4 confirms the assistance information is needed, for the detailed information to be carried, the following candidates are discussed in RAN1

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

Among the above information, the first FFS on assistance information related to measurement occasions, it is more of RAN4 scope, and the last three FFS related to channel quality and traffic are more of RAN2 scope which can be discussed in RAN2.

Within the candidates of UAI that R1 identified, the first one is more related to measurement occasions is in RAN4 scope, while the others are more in RAN2 scope.

But anyway, the discussion at RAN2 can happen after seeing RAN4 LS reply.

Issue-2: LCP

For what can be transmitted in the gaps/restrictions that are caused by RRM measurements, RAN1 has made the following agreement:

- From RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements. - FFS: Whether or not/How to support of the case where an occasion(s) of gap/restrictions that are caused by RRM measurements are cancelled/skipped partially

As agreed in RAN1, the transmission/reception in gaps/restrictions should be same as there is no gap/restriction, which means there is no restriction on specific signal/channel to be transmitted. And our understanding is RAN2 should not introduce any enhancement (e.g., LCP) to restrict the data/channel can be transmitted in the gaps/restrictions.

As agreed in RAN1, the transmission/reception in gaps/restrictions UE skipped should be same as there is no gaps/restrictions.

RAN2 to confirm when UL transmission is enabled in particular gap/restriction (i.e., when gap/restriction is skipped/cancelled), no additional LCP impact is needed.

- Conclusion

We have the following observations:

Observation 1	Whether the assistance information from the UE is necessary depends on RAN4 evaluation on the RRM requirement for measurement gap skipping scenario, e.g., whether new RRM requirement is defined.

Observation 2	Within the candidates of UAI that R1 identified, the first one is more related to measurement occasions is in RAN4 scope, while the others are more in RAN2 scope.

Observation 3	As agreed in RAN1, the transmission/reception in gaps/restrictions UE skipped should be same as there is no gaps/restrictions.

We have the following proposals:

Proposal 1	For measurement gap skipping, RAN2 wait for RAN4 evaluation before working on the UE assistance information message design.

Proposal 2	RAN2 to confirm when UL transmission is enabled in particular gap/restriction (i.e., when gap/restriction is skipped/cancelled), no additional LCP impact is needed.

Reference

- R1-2405736, LS on UE assistance information