# 3GPP TSG-RAN WG2 Meeting #127bis	                               R2-2409122
Hefei, China, Oct 14th – 18th, 2024

Agenda Item	: 8.2.4 (FS_Ambient_IoWT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

During [POST127][033] email discussion, it was discussed how to support random access procedure for A-IOT. However, since there is no consensus on some issues, we show our views on them. In addition, we point out the issues not handled in the email discussion.

Discussion

  - Data transmission procedure

    - Feedback indication from a reader for D2R transmission

During email discussion, it was identified that there is a case where the reader does not have the subsequent R2D data to be transmitted to the device after reception of D2R message. In other words, from the A-IOT device point of view, if the A-IOT device does not receive the subsequent R2D transmission after performing the D2R transmission, there is no clue to identify whether the D2R data is successfully transmitted or not. Considering that, there may be a case where the A-IOT device considers that D2R transmission is successfully completed but the reader does not receive the D2R transmission. In order to handle this case, the following options were proposed.

- Option 1. Re-initiates the same service request only for the A-IOT devices, which the D2R transmission is not successfully completed, by the CN.

- Option 2. Introduce the feedback indication for D2R transmission from the reader in order to perform the re-access procedure according to the feedback indication.

For the inventory for multiple unidentified A-IOT devices case, the CN may not know which A-IOT device suffers the D2R transmission failure. In this case, Option 1 cannot work. Moreover, Option 1 may cause a delay of the completion of the service request because the re-initiation of the same service request will be requested after finishing the previous service request. It may degrade the QoS satisfaction and efficiency.

In addition, since the majority view is that the A-IOT device performs the re-access procedure if the access failure, e.g., contention resolution failure, happens, we do not see the need for a different handling method between the contention resolution failure and D2R transmission failure.

Proposal 1. Introduce the feedback indication from a reader for D2R transmission.

If the above proposal is agreeable, RAN2 need to further discuss what feedback indication is needed. For this, RAN2 need to discuss what is the A-IOT behaviour if the success or failure feedback indication is received first.

In our understanding, the failure feedback indication can be used to indicate that the A-IOT device performs the re-access procedure. However, even if the success feedback indication is received, the A-IOT device just knows that the D2R transmission is successfully completed and there is no expected A-IOT device behaviour after reception of the success feedback indication.

Observation 1. Even if A-IOT device receives the success feedback indication for the D2R transmission, there is no expected A-IOT device behaviour after reception of the success feedback indication.

In addition, even if the success feedback is not indicated, the A-IOT device can consider that the D2R transmission is successfully transmitted if there is no more R2D transmission within a certain time. Thus, we think that the A-IOT device considers that D2R transmission failure happens only when the failure feedback is indicated from the reader and the A-IOT device considers that the D2R transmission is successfully performed if the failure feedback is not indicated.

Proposal 2. The A-IOT device considers that D2R transmission failure happens only when the failure feedback is indicated from the reader and the A-IOT device considers that the D2R transmission is successfully performed if the failure feedback is not indicated.

    - AS scheduling ID

As stated in email discussion, the potential purposes of the AS scheduling ID are as follows.

- For D2R scheduling, the AS scheduling ID is associated with specific device for this D2R scheduling.

- For R2D reception, the AS scheduling ID indicates that the targeted A-IOT device is supposed to receive/decode its R2D transmission.

In other words, the purpose of the AS scheduling ID is used for A-IOT identifier similar to the C-RNTI in NR. However, it is not concluded which ID is used for the AS scheduling ID.

For CBRA case, the reader may assign a unique AS scheduling ID. However, even if the AS scheduling ID is assigned by the reader, the A-IOT device may not permanently use the unique AS scheduling ID due to limited non-volatile memory size. In other words, if the A-IOT device is out of energy, the unique AS scheduling ID should be configured again after re-charging the energy. In addition, for the inventory for multiple unidentified A-IOT devices case, the CBRA should be performed for multiple unidentified A-IOT devices. Since the multiple unidentified A-IOT devices may not perform the access procedure before, the reader cannot assign a unique AS scheduling ID.

In RAN2#127 meeting, it was agreed that the random ID generated by A-IOT device is contained in Msg1. In addition, if the A-IOT device successfully completes the contention resolution within an access occasion, the random ID transmitted in Msg1 is unique within the access occasion. Thus, we propose that in CBRA, the random ID is used for the AS scheduling ID after the completion of the contention resolution.

Proposal 3. In CBRA, the random ID is used for the AS scheduling ID after completion of the contention resolution.

For CFRA, when the A-IOT device performs the CFRA procedure, the A-IOT device transmits the upper layer data (e.g. device ID) in Msg1 and A-IOT device considers that the contention resolution is completed after transmitting Msg1. In other words, during CFRA, the A-IOT device does not transmit the random ID in Msg1.

In order to allocate the AS scheduling ID for CFRA, the simplest way is that the reader contains the AS scheduling ID in the A-IOT paging message. However, in the RAN2#126 meeting, since it was agreed that A-IOT paging message can include multiple A-IOT device IDs to trigger the CFRA procedure for multiple A-IOT devices, containing multiple AS scheduling IDs in A-IOT paging message causes the signalling overhead.

In addition, since the A-IOT device should support low complexity, we think that the unified solution between the CBRA and CFRA should be considered. Thus, it would be better the random ID should be contained in Msg1 during CFRA procedure, and the random ID is used for scheduling purpose.

Proposal 4. During CFRA procedure, the random ID is contained in Msg1, and the random ID is used for scheduling purpose.

  - How to indicate the start of the access occasion

In RAN2#125bis meeting, it was agreed that slotted-ALOHA is the baseline for A-IOT random access. In the slotted-ALOHA, the important point is how the A-IOT device knows the start of the slot in order to synchronize the uplink. Thus, the A-IOT device must know the exact timing for start of access occasion to transmit or receive data.

In order to know the start of the access occasion, there are two options as follows.

- Option 1. The duration of the access occasion is pre-defined and it is indicated by A-IOT paging message.

- Option 2. The reader indicates a start indication for an access occasion to the A-IOT devices using new A-IOT message.

In our understanding, the A-IOT device cannot support the synchronization system. So, Option 1 is feasible only when not many A-IOT devices perform the access procedure within one round. However, since the A-IOT has been discussed for the logistics scenario, RAN2 needs to consider a case where many A-IOT devices should perform the access procedure within one round. Thus, we prefer to go with Option 2.

Proposal 5. Introduce the new A-IOT message to indicate the start of the access occasion.

  - Selection of the access occasion

    - CBRA

During email discussion, it was discussed how to select the access occasion by the A-IOT device, and majority view is that the A-IOT device randomly selects one access occasion within one round to perform the RA procedure. However, even if the A-IOT device can randomly select the access occasion, we think that the energy level of the A-IOT device should be considered.

Since the A-IOT device has small energy storage capability, an A-IOT device may be in a condition that it doesn’t have enough energy to monitor PRDCH for all access occasions. In this case, the A-IOT device may not try to perform the access procedure due to out-of-energy if the A-IOT device selects the last part of the access occasion.

In addition, since the reader does not have information on whether an A-IOT device has enough energy or not, the reader may not control the prioritization of the A-IOT device having not enough energy to perform the access procedure first. Thus, we think that how to select an access occasion based on the A-IOT device’s energy level should be discussed.

It may be argued that the CN can trigger the dedicated A-IOT paging procedure for the A-IOT devices that the access procedure fails due to out-of-energy because the CN knows which A-IOT device does not perform the access procedure. However, since the energy harvesting should be performed for a long time, e.g., a hundred milliseconds or several seconds, it would have an impact on the QoS satisfaction and efficiency.

Proposal 6. How to select an access occasion based on the A-IOT device’s energy level should be studied.

If above proposal is agreeable, how to consider the A-IOT device’s energy level to select the access occasion should be discussed. We think that the simple way is to divide the access occasions into two groups. One group is to use the A-IOT devices having not enough energy and the other group is used to use the A-IOT devices having enough energy. Using this group information, the A-IOT device can select a group based on the A-IOT device’s energy level. Then, the A-IOT device selects one access occasion associated with the selected group.

Proposal 7. The access occasions are divided into two groups, and the A-IOT device selects one group based on the A-IOT device’s energy level.

    - CFRA

In RAN2#125bis meeting, it was agreed that we will study the support for access triggering for a single device, group of devices, or all devices. For the single device case, the RA procedure only for a single A-IOT device would not need CBRA procedure, and the CFRA procedure would be simple and appropriate because the reader requests the performing RA procedure only for a single A-IOT device.

Proposal 8. For RA procedure from a single A-IOT device, the CFRA is used as a baseline.

In addition, in the RAN2#126 meeting, it was agreed that A-IOT paging message can include multiple A-IOT device IDs. We think that if the A-IOT paging message includes multiple A-IOT device IDs, the CFRA can be used based on the order of the A-IOT device IDs.

For example, if the reader transmits the A-IOT paging message containing the first A-IOT device ID and the second A-IOT device ID, only first A-IOT device and the second A-IOT device perform the RA procedure. In this case, if the first and second A-IOT devices select an access occasion according to the order of the A-IOT device ID, the A-IOT device can perform the CFRA procedure.

Proposal 9. When the A-IOT paging message contains multiple A-IOT device IDs, the A-IOT device selects the access occasion according to the order of the A-IOT device ID and each A-IOT device perform the CFRA procedure.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. Introduce the feedback indication from a reader for D2R transmission.

Observation 1. Even if A-IOT device receives the success feedback indication for the D2R transmission, there is no expected A-IOT device behaviour after reception of the success feedback indication.

Proposal 2. The A-IOT device considers that D2R transmission failure happens only when the failure feedback is indicated from the reader and the A-IOT device considers that the D2R transmission is successfully performed if the failure feedback is not indicated.

Proposal 3. In CBRA, the random ID is used for the AS scheduling ID after completion of the contention resolution.

Proposal 4. During CFRA procedure, the random ID is contained in Msg1, and the random ID is used for scheduling purpose.

Proposal 5. Introduce the new A-IOT message to indicate the start of the access occasion.

Proposal 6. How to select an access occasion based on the A-IOT device’s energy level should be studied.

Proposal 7. The access occasions are divided into two groups, and the A-IOT device selects one group based on the A-IOT device’s energy level.

Proposal 8. For RA procedure from a single A-IOT device, the CFRA is used as a baseline.

Proposal 9. When the A-IOT paging message contains multiple A-IOT device IDs, the A-IOT device selects the access occasion according to the order of the A-IOT device ID and each A-IOT device perform the CFRA procedure.