3GPP TSG-RAN WG2 Meeting #127bis	 R2-2409126

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.3.4

Source:	KDDI Corporation

Title:	Discussion on simulation assumption for RLF prediction

SID:	FS_NR_AIML_Mob – Release 19

Document for: 	Discussion and decision

- Introduction

In RAN#103 meeting, a revised Rel-19 SID of ‘AI/ML for mobility in NR’ was approved [1]. The detailed objectives are as follows. In this contribution, we mainly discuss the simulation assumptions for RLF prediction.

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

- Discussion

Motivation of evaluation of AI/ML aided mobility benefits

Traditional mobility mechanisms, based on historical measurement results or measurement events, tend to be reactive. While this approach may work well for low-mobility UEs, it can be problematic in scenarios involving high-mobility UEs especially for Frequency Range 2 (FR2).

When the UE is moving at high speed, the position of the UE may change significantly, potentially causing frequent changes in beam quality. Therefore, in such cases, the impact of delay in beam switching could be significant. The introduction of AI/ML-based mechanisms for mobility provides the potential to transition from reactive to proactive schemes. These can predict the optimal beams or beam pairs over time to improve UE throughput and can also predict when beams/cells might deteriorate, helping to avoid unintended events. Hence, the motivation for evaluating AI/ML aided mobility benefits is to improve network-triggered L3-based handovers and RRM performance through proactive measures, which can lead to better decision-making and avoid unintended events, ultimately enhancing the performance and efficiency of 5G networks.

Observation 1: AI/ML aided mobility can significantly improve the performance of network-triggered L3-based handovers, especially for high-speed UEs in FR2 frequency bands.

Simulation assumption for RLF/HO failure prediction

Agreements on simulation assumption at RAN2 #126 To agree not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8) and blockage (7.6.4) for channel modelling (38.901)

At the meeting, it was agreed that the Blockage model would not be considered as a simulation assumption for RRM measurement prediction evaluation[2]. The agenda of RAN2#127 was written that it should focus on discussing RLF specific methodology and simulation assumptions (addressing the differences or additional aspects from RRM prediction assumptions).

When evaluating the ability to predict unintended events, special attention should be given to the phenomenon of blockage attenuation, particularly in the context of FR2. These higher frequencies are more susceptible to blockage by physical obstacles (e.g., buildings, trees, human bodies), leading to sudden drops in signal quality, known as blockage attenuation. Blockage model can be used in simulations of TS38.901[2].

The followings are the importance of considering blockage attenuation in simulations and evaluations for a couple of key reasons:

High Impact on FR2: Due to their higher frequency, FR2 signals have shorter wavelengths, making them more likely to be blocked by obstacles. This can lead to more frequent and severe signal degradation compared to lower frequency bands (FR1). Thus, accurately predicting when and where blockage attenuation might occur is crucial for maintaining network performance and reliability in FR2 deployments.

Enhancing Predictive Models with AI/ML: Incorporating blockage attenuation into AI/ML models allows these systems to better predict potential unintended events such as sudden signal losses or degradation. By understanding where blockage is likely to occur, the network can proactively adjust, for instance, by switching beams or rerouting connections to minimize the impact on users.

Improving User Experience and Network Efficiency: By accurately predicting and mitigating the effects of blockage attenuation, networks can provide more stable and reliable connections, especially in high-mobility scenarios. This leads to improved user experience through fewer dropped connections and higher data rates. Additionally, network resources can be utilized more efficiently, as the system can pre-emptively address potential issues before they affect users.

In conclusion, it was agreed that the channel model for RRM measurement prediction does not consider blockage. However, in RLF prediction simulations, particularly in FR2, physical obstacles (such as buildings, trees, and human bodies) can cause significant interference, leading to a sudden drop in signal quality known as blocking attenuation. Therefore, blockage is necessary to introduce abrupt changes in the radio link. Considering the evaluation workload, it is acceptable to allow the blockage model as an option.

Proposal 1: RAN2 should consider blockage for channel modelling to evaluate RLF for AI/ML aided mobility benefits particularly in high-mobility scenarios of FR2. Considering the evaluation workload, it is acceptable to allow the blockage model as an option.

Two alternative model (model A and model B ) for the blockage (7.6.4) for channel modelling (38.901)

should be chosen for simulation assumption for RLF to compare the simulation results from companies. Model A is applicable when a generic and computationally efficient blockage modelling is desired. Model B is applicable when a specific and more realistic blocking modelling is desired [3].

Proposal 2: The blockage model A or B is up to companies to choose to allow generalization of the simulation.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1: AI/ML aided mobility can significantly improve the performance of network-triggered L3-based handovers, especially for high-speed UEs in FR2 frequency bands.

Proposal 1: RAN2 should consider blockage for channel modelling to evaluate RLF for AI/ML aided mobility benefits particularly in high-mobility scenarios of FR2. Considering the evaluation workload, it is acceptable to allow the blockage model as an option.

Proposal 2: The blockage model A or B is up to companies to choose to allow generalization of the simulation.

References

[1] RP-240082 	Revised SID on AIML for mobility in NR (OPPO)

[2] R2_126_ChairNotes_24-05-24_13-10_eom

[3] TR 38.901 	Study on channel model for frequencies from 0.5 to 100 GHz.