33GPP TSG-RAN WG2 Meeting #127bis	 R2-2409127

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.3

Source:	KDDI Corporation

Title:	Discussion on L1 event triggered measurement reporting

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN#104 meeting, a revised Rel-19 WID of ‘NR mobility enhancements Phase 4’ was approved [1].

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we will discuss and share our views on event triggered L1 measurement reporting for “Specify necessary components to support event triggered L1 measurement reporting” described in the second bullet of the WID objective above.

- Discussion

Need of MR when leaving condition is met

In RAN2#127 meeting, proposal was not discussed below [2].

P3 in R2-2406432 (Vivo) Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

Flexible Configuration: gNB can finely manage the timing of UE's L1 measurement reporting, allowing customization according to specific network needs and conditions.

Rapid Adaptation: By performing measurement reporting at the time of departure, it enables quick adaptation to network changes and supports optimal handover decisions, thereby improving connection stability.

Customizable Network Control: The trigger for reporting can be set according to the operational strategy of each network, allowing for more flexible and tailored network management.

Proposal 1: Measurement reports should be sent so that the network can determine when event conditions are no longer met, enabling appropriate handovers.

Need of event-triggered periodic MR

In RAN2#127 meeting, proposals were not discussed below [2].

P4 in R2-2406432 (Vivo) Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration). P5 in R2-2406358 (MediaTek) Proposal 5: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration).

| Item | Proposal 4 (Vivo) | Proposal 5 (MediaTek) |
| --- | --- | --- |
| Reporting Method | Allows multiple L1 measurement reports | Single L1 measurement report |
| Configuration Flexibility | High: Configurable by gNB, enabling flexible response based on specific conditions | Low: Easy management with a single report |
| Implementation Complexity | Relatively high: Requires managing multiple reports | Low: Simple and fixed configuration |
| Signaling Overhead | Potential increase: Concern of overhead increase with frequent reporting | Reduced: Less overhead with a single report |
| Application Scenario | Suitable for rapidly changing network conditions or when detailed analysis is needed | Suitable for stable environments or when reducing load is a priority |
| Pros | Enables detailed data collection and flexible response | Simplicity of implementation and efficient resource use |
| Cons | Requires overhead management and may increase implementation complexity | May lack flexibility |

Based on this comparison, it is appropriate to select which proposal to support according to operational environment and specific needs.

Flexibility: The gNB can be configured so that the UE can perform multiple L1 measurement reports from a single event trigger.

Benefits: It allows for flexible responses when more detailed and frequent data is required due to specific network conditions or demands.

Applications: Effective in situations where network conditions change rapidly, or detailed analysis is needed.

Proposal 2: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report.

Information in MR

In RAN2#127 meeting, proposals were not discussed below [2].

| P1 in R2-2407285 (NTTDCM) Proposal 1: Event-triggered L1 measurement includes below contents; ・Beam information (i.e., SSBRI or CRI) ・Beam quantity (up to RAN1) ・Triggered event information (e.g., ReportConfigID) P2 in R2-2407423 (Samsung) Proposal 2: Event triggered L1 measurement reporting for LTM includes following information: - L1 RSRP of the current beam - L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition) - DL RS resource index (SSB or CSI-RS) |  |  |
| --- | --- | --- |
| Item | Proposal 1 (NTTDCM) | Proposal 2 (Samsung) |
| Beam Information | Includes SSBRI or CRI | L1 RSRP of the current beam |
| Neighbor beam Information | Not included | L1 RSRP of N ≥ 1 neighboring beams ¦ |
| Number of Beams | Dependent on RAN1's decision | No specific information provided |
| Trigger Event Information | Includes ReportConfigID, etc. | Not included |
| DL RS Resource Index | Not included | SSB or CSI-RS |
| Advantages | Allows detailed beam information and understanding of event background | Useful for interference assessment and signal strength analysis |
| Application Scenarios | Scenarios where beam management and event details are important | Scenarios requiring neighbor cell information |

Using this comparison table, it's important to select the most suitable proposal based on the operational environment and objectives. It was agreed at the previous meeting that the report transmission container would be MAC CE not UCI. To reduce overhead, it may not include beam information from neighboring cells. Since it includes information on trigger events, the background of the event occurrence is easier to understand.

Proposal 3: Event-triggered L1 measurement includes below contents;

・Beam information (i.e., SSBRI or CRI)

・Beam quantity (up to RAN1)

・Triggered event information (e.g., ReportConfigID)

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: Measurement reports should be sent so that the network can determine when event conditions are no longer met, enabling appropriate handovers.

Proposal 2: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report.

Proposal 3: Event-triggered L1 measurement includes below contents;

・Beam information (i.e., SSBRI or CRI)

・Beam quantity (up to RAN1)

・Triggered event information (e.g., ReportConfigID)

References

[1] RP-241515 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] RAN2 #127 Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)