3GPP TSG-RAN WG2 Meeting #127bis	R2-2409135

# Hefei, China, 14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1194 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on eutra-CGI-Reporting-NRDC and NR-CGI-Reporting-NRDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#105, RAN2 discussed the CGI reporting when MN and SN are configured with unaligned or aligned DRX pattern. The main problem is NR sub6 and LTE are actually quite similar, in some UE implementation, some RF / Baseband hardware is shared in LTE and NR sub6 for early product launch. Then if DRX offsets are not aligned in MN and SN, such UEs must wait for both LTE and NR being idle (i.e. common idle period in DRX cycles in MN and SN) before measuring CGI. As a result, the exisiting FGIs(17/18/19) is redefined to include the case of UE is configured with EN-DC with the same DRX configurations between MN and SN. And RAN2 introduced extra capabilities for CGI reporting towards 2G/3G/4G neighbour cells when DRX configuration are different between MN and SN or when on-duration configured by MN does not contain on-duration configured by SN if their DRX cycles are same. In RAN2#110, RAN2 also discussed the CGI measurement in NR-DC and agreed to introduce the same limitation. eutra-CGI-Reporting-NRDC Defines whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same.. nr-CGI-Reporting-NRDC Defines whether the UE supports acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same.. It is not clear whether these capabilies include the case that only one node configures the DRX for UE. In our views, these capabilities include this case based on the following reasons. - In our views, these capabilities means UE can try to measure the CGI even if UE does not have the common idle period in DRX cycles in MN and SN. Therefore UE also can try to measure the CGI even if only one node configures the DRX. - If these capabilities do not include this case, MN needs to wait the DRX configuration from SN. But in some cases(e.g. the inter-vendor), SN may not send the DRX configuration to MN. It will have impact on the CGI measurement of MN. - In some cases, SN does not configure DRX. For example,there are lots of data for the SCG bearer(s). If these capabilities do not include this case, it will have impact on the CGI measurement of MN. |
| Summary of change: |  | Change the definitions of eutra-CGI-Reporting-NRDC and nr-CGI-Reporting-NRDC. Impact Analysis Impacted 5G architecture options: NR-DC Impacted functionality: CGI reporting Inter-operability: - If the network is implemented according to the CR and UE is not, UE may detect the reconfiguration failure because UE is unable to comply with (part of) the configuration and trigger the RRC re-establishment. - If UE is implemented according to the CR and the NW is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MN cannot configure CGI measurement if the SN does not send the DRX configuration to the MN or the SN does not configure DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.9	MeasAndMobParameters

| eutra-CGI-Reporting-NRDC Defines whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same, or only one node configures the DRX cycle. | UE | No | No | No |
| --- | --- | --- | --- | --- |

--------------------skip the unchanged text-----------------------------

| nr-CGI-Reporting-NRDC Defines whether the UE supports acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same, or only one node configures the DRX cycle. | UE | Yes | No | No |
| --- | --- | --- | --- | --- |