3GPP TSG RAN WG2 Meeting #127bis	R2-2409140

Hefei, China, 14 - 18 October 2024

Agenda Item:	7.4.3

Source:	Huawei, HiSilicon

Title:	MAC CR rapporteur summary

Document for: Discussion and Decision

# 1	Introduction

This document summarizes all the corrections proposed to TS 38.321 for LTM and indicates the points to make decisions for correction or not.

# 2	Discussion

The following documents were submitted:

| R2-2408262 | Correction on BWP switching during PRACH triggered for LTM candidate cells | MediaTek Inc. |
| --- | --- | --- |
| R2-2408297 | MAC corrections for LTM | Samsung Electronics Co., Ltd |
| R2-2408755 | Clarification to 38.321 on LTM Cell Switch UP Aspect | ZTE Corporation |
| R2-2408784 | Clarification on random access procedure initialization for early UL synchronization | Google |
| R2-2408817 | MAC corrections for feMob | Ericsson |
| R2-2408875 | Coexistence of UE C-DRX and RACH-less LTM | Rakuten Mobile, Inc |
| R2-2409138 | Miscellaneous corrections for LTM | Huawei, HiSilicon |
| R2-2409139 | Correction to LTM MAC CE based CFRA with MSG1 repetition | Huawei, HiSilicon |

## 2.1	Straightforward corrections

R2-2408297 proposes a rewording of a bullet that is proposed to be removed in R2-2409138. This can be handled with R2-2409138.

R2-2408755 proposes:

1)	to add an indication, upon successful completion of RA for LTM cell switch, to upper layers that the RA procedure is complete.

The rapporteur observes that 38.331 clause 5.3.5.3 is expecting an indication that the LTM cell switch execution has been successfully completed, which is currently missing. The rapporteur assumes that this is the intention of R2-2408755. The rapporteur will address this in a revision of the rapporteur CR.

2)	to initiate RA explicitly when RACH-less LTM cell switch is not initiated in 19.35 upon reception of the LTM cell switch MAC CE.

The rapporteur understands that:

-	for the SCG case, the RA is already initiated in TS 38.331 clause 5.3.5.3;

-	for the MCG case, the RA is initiated in TS 38.321 clause 5.4.4 because a SR is pending for the RRCReconfigurationComplete to be transmitted on SRB1.

Therefore, no change is needed.

R2-2408784 proposes:

1)	to add ssb-PerRACH-Occasion in 5.1.1 to the list of parameters for the Random Access procedures, and capture that this is "the number of SSBs mapped to each PRACH occasion for the Random Access procedure initiated by the PDCCH order for an LTM candidate cell". The motivation is that "Currently in the random access procedure initialization, the UE initializes ssb-perRACH-OccasionAndCB-PreamblesPerSSB for 4-step RA type.  However, for the Random Access procedure initiated by the PDCCH order for an LTM candidate cell, ssb-PerRACH-Occasion in EarlyUL-SyncConfig should be used instead of ssb-perRACH-OccasionAndCB-PreamblesPerSSB."

The rapporteur observes that:

-	R2-2409138 has a proposal "select the set of Random Access resources for early UL synchronization " to make it clear that the resources are from EarlyUL-SyncConfig, which includes ssb-PerRACH-Occasion and not ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and the field description of ssb-PerRACH-Occasion already contains the proposed text;

-	5.1.1 also applies to reconfiguration with sync and there is a field that has this name in rach-ConfigDedicated, also not listed in 38.321, so the proposed text may be confusing.

Therefore, the rapporteur thinks the change is R2-2409138 might be better and proposes to discuss this together with this CR.

2)	to change "preamble initial transmission" to " a Random Access Preamble for the first time for the LTM candidate cell" and "the same Random Access Preamble as the last Random Access Preamble transmission". The motivation is that in 38.212, there is no such description matching with current text in 38.321.

The rapporteur sees that 38.212 describes a field called "PRACH association indicator" as "This field indicates initial transmission or retransmission of PRACH", which seems more aligned with the existing than with the proposed description. Of course, some better alignment is always possible.

Therefore, the rapporteur suggests discussing any improvement together with R2-2409138.

R2-2408817 proposes, in the reception of the LTM cell switch command MAC CE:

1)	to add a condition that "Timing Advance Command value (hexa-decimal) is set as FFF" for the UE to apply the TA autonomously acquired by the UE

2)	to add an "else" branch to trigger RA

The motivation is that "in case the TA value is not received or if the TA value in the MAC CE has value FFF it is not clear that the UE should initiate the Random access procedure.".

As explained above, for the SCG case, if MAC does not indicate to skip RA, RA is triggered in 38.311 and for the MCG case, if RACH-less cell switch is not initiated, SR for the RRC complete message will trigger RA, so the motivation is not correct.

In addition, when the UE is configured with UE-based Timing Advance measurement and has acquired the TA, the proposal requires the UE to perform RA if the network sends a value that is not FFF for the TA command. RAN2 already discussed a proposal to allow the network to trigger RA in this case and that proposal was not agreed, so the TP contradicts with the previous RAN2 agreement.

Therefore, the rapporteur suggests not to agree these changes.

Proposal 1: The changes proposed in R2-2408755 and R2-2408817 are not pursued.

Proposal 2: Discuss the MAC rapporteur CR (R2-2409138) and the changes proposed in R2-2408297 and R2-2408784.

## 2.2	Corrections that may need discussion

### 2.2.1	BWP switching during PRACH triggered for LTM candidate cells

R2-2408262 proposes that, upon reception of PDCCH for BWP switching of a Serving Cell during RA to an LTM candidate cell, the UE is not required to performed BWP switching.

According to the coversheet, "concurrent operations for LTM PRACH and legacy BWP switching can result in BWP operation conflicts and pending, which further lead to synchronization issues with the network".

The rapporteur understands that, in Rel-15 specification, upon reception of PDCCH for BWP switching of a Serving Cell during RA to this serving cell, the UE is not required to perform BWP switching. Here, the RA is to another cell, possibly a non-serving cell. However, this is new, so this can be discussed.

Proposal 3: Discuss whether the UE may ignore PDCCH for BWP switching of any serving cell while RA to an LTM candidate cell is ongoing.

### 2.2.2	Msg1 repetition

R2-2409139 raises that MAC CE based CFRA with Msg1 repetition should have the same configuration restriction as RRC based CFRA, that this is captured in a note but the note is missing the case of (e)RedCap UEs.

Therefore, it proposes adding two notes, one for RedCap UEs and one for eRedCap UEs.

Proposal 4: Discuss whether to add notes for Msg1 repetition included in the LTM cell switch command MAC CE for RedCap and eRedCap UEs.

## 2.3	Enhancements

### 2.3.1	Active time after PDCCH order for early RACH

R2-2408875 discusses that DRX could delay sending the LTM cell switch command MAC CE and suggests, as a solution to this problem, the following:

-	RACH-less LTM cell switch is considered to be “ongoing” when the UE receives a PDCCH order from the serving gNB-DU.

-	UE shall return to C-DRX cycle if the RACH-less LTM cell switch command is not received within a predefined time.

The rapporteur notes that:

-	according to existing procedure text, if the current cell is an LTM candidate, the UE will perform an UL transmission using the CG for RACH-less LTM cell switch;

-	upon receiving a DL assignment, the UE considers RACH-less LTM cell switch as complete, so if there is any ongoing DL transmission, the effect of the proposal will be immediately terminated.

Proposal 5: Discuss the proposal to consider RACH-less LTM to be ongoing when the UE receives a PDCCH order for early RACH, and return to C-DRX cycle if the RACH-less LTM cell switch command is not received within a predefined time.

# 3	Conclusion

This contribution makes the following proposals:

Straightforward corrections

Proposal 1: The changes proposed in R2-2408755 and R2-2408817 are not pursued.

Proposal 2: Discuss the MAC rapporteur CR (R2-2409138) and the changes proposed in R2-2408297 and R2-2408784.

Corrections that may need discussion

Proposal 3: Discuss whether the UE may ignore PDCCH for BWP switching of any serving cell while RA to an LTM candidate cell is ongoing.

Proposal 4: Discuss whether to add notes for Msg1 repetition included in the LTM cell switch command MAC CE for RedCap and eRedCap UEs.

Enhancements

Proposal 5: Discuss the proposal to consider RACH-less LTM to be ongoing when the UE receives a PDCCH order for early RACH, and return to C-DRX cycle if the RACH-less LTM cell switch command is not received within a predefined time.

# 4	References

- R2-2408262	Correction on BWP switching during PRACH triggered for LTM candidate cells, MediaTek Inc.

- R2-2408297	MAC corrections for LTM, Samsung Electronics Co., Ltd

- R2-2408755	Clarification to 38.321 on LTM Cell Switch UP Aspect, ZTE Corporation

- R2-2408784	Clarification on random access procedure initialization for early UL synchronization, Google

- R2-2408817 MAC corrections for feMob, Ericsson

- R2-2408875	Coexistence of UE C-DRX and RACH-less LTM, Rakuten Mobile, Inc

- R2-2409138	Miscellaneous corrections for LTM, Huawei, HiSilicon

- R2-2409139	Correction to LTM MAC CE based CFRA with MSG1 repetition, Huawei, HiSilicon