3GPP TSG-RAN WG2 Meeting #127bis	R2-2409142

Hefei, China, 14 – 18 October 2024

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Inter-CU LTM

Document for: Discussion and Decision

# 1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 for the intra-CU scenario. In Rel-19 [1], the scope is extended to inter-CU to support low latency and low interruption cell switch in broader areas. In this contribution, we discuss inter-CU LTM in standalone and NR-DC architectures.

# 2	Discussion

## 2.1	LTM in standalone

### 2.1.1	LTM preparation

In the RAN2 #127 meeting, RAN2 agreed to use a common CSI resource configuration.

RAN2 #127 The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration

In Rel-18 LTM, the UE is configured with a list of LTM-Candidate, where each LTM-Candidate is for one candidate cell and includes its SSB configuration, and a list of LTM-CSI-Resource, configured via ltm-CSI-ResourceConfigToAddModList where each LTM-CSI-Resource can be used to configure a L1 measurement report and includes SSB-Index from different candidates.

In Rel-19, in addition to SSB, RAN1 agreed to support L1 measurements based on CSI-RS:

RAN1 #118 Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS

Therefore, it should be possible for an LTM-CSI-Resource configured via ltm-CSI-ResourceConfigToAddModList to indicate NZP CSI-RS resources associated with different candidates, and it should be possible to configure a list of NZP CSI-RS resources for each candidate.

Proposal 1:	The LTM-CSI-Resource in the common CSI resource configuration (configure using ltm-CSI-ResourceConfigToAddModList-r18) can refer to NZP CSI-RS resources of one or more candidate cell.

In the common CSI resource configuration (ltm-CSI-ResourceConfigToAddModList-r18), only CSI-RS index and SSB index associated with different candidates are provided, instead of the resources themselves. The UE needs to know the exact CSI-RS resource configuration to get the time, frequency and other information. Therefore, the CSI-RS resource configuration should be configured to the UE.

Proposal 2:	The configuration of each candidate cell can include a list of NZP CSI-RS resources.

### 2.1.2	LTM cell switch execution

In the RAN2 #126 meeting, both CG-based RACH-less and DG-based RACH-less procedures were agreed. One missing point is whether RACH-based LTM is supported for inter-CU LTM. In Rel-18, both CFRA and CBRA-based procedures are supported. Therefore, for the case that RACH-less is not feasible, RACH-based LTM should also be supported.

Proposal 3:	Support RACH-based inter-CU LTM cell switch, including CBRA and CFRA.

## 2.2	Inter-CU LTM in NR-DC

### 2.2.1	Inter-CU MCG LTM with SN unchanged/released

For the inter-CU MCG LTM with SN unchanged/release scenario, RAN2 had the following agreements:

RAN2 #127 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

RAN3 agreed that, in inter-CU MCG LTM, the source MN initiates LTM candidate preparation and sends Handover Request to candidate MNs. In the Handover Request message, the source SCG configuration can be included. Then the candidate MN could decide whether to keep or release the SN. If it wants to keep the source SN, the candidate MN can send an SN Addition Request to the source SN.

Proposal 4:	In inter-CU MCG LTM cell switch in the NR-DC scenario, the candidate MN decides whether to reconfigure or release the SCG.

When the SN is not changed, in the inter-CU MCG LTM, both MN key and SN key need to be updated and PDCP re-establishment is required for MN- and SN-terminated bearers. RACH-less LTM cell switch may be performed for the MCG, but RACH is needed for reconfiguration with sync of the SCG. No enhancement to the SCG reconfiguration with sync procedure is needed.

Proposal 5:	In inter-CU MCG LTM with SN unchanged, the UE performs reconfiguration with sync of the SCG (including RACH).

Note that the SCG configuration can be changed in inter-CU MCG LTM with SN unchanged. One additional question is whether the PSCell identity can be changed or not. As we discussed above, RACH to the SCG is anyway needed, the SN key needs to be changed and whether the PSCell identity is changed or not has no specification impact. Therefore, this can be left to network implementation.

Proposal 6:	In inter-CU MCG LTM with SN unchanged, it is up to network implementation whether the PSCell is changed or not.

RAN2 agreed that the legacy Rel-15 RRC reconfiguration with sync procedure is applied. However, for subsequent inter-CU MCG LTM with SN unchanged, the legacy procedure for SN key delivery from the target MN to the SN cannot be re-used directly.

According to TS 37.340 “10.7 Inter-Master Node handover with/without Secondary Node change” section, the SN gets an updated SN key in SN Addition Request message, as shown in the Figure 1.

Figure 1. Part of inter-MN handover with/without MN initiated SN change procedure

Observation 1: In the legacy inter-Master Node handover without Secondary Node change scenario, the SN gets the updated SN key in the SN Addition Request message.

However, for inter-CU MCG LTM with SN unchanged, the SN Addition Request procedure is performed only at the preparation step, and not performed again in the following subsequent MCG LTM procedure. At the preparation phase, the MN keys of different MNs for subsequent inter-CU MCG LTM are not determined yet so it is not possible to use the SN Addition Request to deliver SN keys. Since the MN key update in inter-CU MCG LTM in NR standalone is still under discussion in SA3, RAN2 can only discuss the SN key delivery procedure in the inter-CU MCG LTM with SN unchanged case after the solution for the standalone case is stable.

Observation 2: In inter-CU MCG LTM with SN unchanged scenario, it is not possible to use the SN Addition Request to deliver SN keys for subsequent inter-CU MCG LTM.

Proposal 7:	RAN2 can discuss the SN key delivery procedure for inter-CU MCG LTM with SN unchanged when the solution for MN key update in non-DC scenario is stable.

### 2.2.2	Inter-CU SCG LTM with MN unchanged

For the inter-CU SCG LTM with MN unchanged scenario, RAN2 had the following agreements:

RAN2 #127 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

In SN-initiated inter-CU SCG LTM, the RRC configuration message is transmitted from the MN to the UE, which includes candidate SCG configurations. When SCG LTM is triggered, the UE applies the target SCG configuration and sends an RRCReconfigurationComplete message to the MN. Then, the MN transfers the SN RRCReconfigurationComplete message to the SN. In this case, if the SCG LTM cell switch is RACH-less, the UE may not have uplink data or signalling to transmit to the SCG. If the network configures UL grant skipping, the SCG may not receive any UE uplink transmission and may consider RACH-less cell switch to be failed.  To avoid this problem, the network should ensure that the UE does not skip the UL grant in RACH-less SCG LTM procedure, e.g., not configure enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured and skipUplinkTxDynamic.

Proposal 8: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch.

In Rel-18, intra-CU SCG LTM cell switch does not affect the MN i.e., MCG operation is not affected. Since PDCP data recovery cannot be triggered for MN-terminated bearers and this would be required for MN-terminated bearers with an SCG RLC bearer, all MN-terminated bearers should be MCG bearers (otherwise, some data will be lost).

Observation 3: Rel-18 SCG LTM cell switch does not affect MCG operation at all and has common procedures with MCG LTM, but data can be lost for MN-terminated split and SCG bearers if any (because PDCP data recovery cannot be triggered on PDCP entities in the MN).

For inter-CU SCG LTM, the MN needs to be involved at least in the LTM preparation, as different SNs cannot communicate directly, and also for LTM cell switch notification, if the Xn signalling exchange is necessary at LTM cell switch. PDCP data recovery for MN terminated SCG/split bearers should be supported.

Proposal 9: For inter-CU SCG LTM, support to trigger PDCP data recovery in PDCP entities at the MN for MN terminated SCG/split bearers.

### 2.2.3	Coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM

In RAN2 #126 meeting, RAN2 had the following agreements related to LTM in NR-DC scenario:

RAN2 #126 Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

The Rel-19 WID is updated accordingly, as shown below:

RP-241515, Revised Work Item: NR mobility enhancements Phase 4 - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The co-existence of different MCG and SCG LTM configurations is summarized in the table below according to the WID:

|  | Intra-MN LTM | Inter-MN LTM | Intra-SN LTM | Inter-SN LTM |
| --- | --- | --- | --- | --- |
| Intra-MN LTM | / | / | / | / |
| Inter-MN LTM | Supported in Rel-19 | / | / | / |
| Intra-SN LTM | Supported in Rel-18 | Supported in Rel-19 | / | / |
| Inter-SN LTM | Supported in Rel-19 | Not supported | Supported in Rel-19 | / |

In order to make sure that the inter-CU LTM is not configured at the same time in MN and SN, inter-node interaction is needed. When the MN decides to configure inter-MN LTM candidates, the MN should send a notification to the SN to disable inter-SN LTM. When the SN decides to configure inter-SN LTM candidates, the SN should first send a request to the MN, and the MN can accept or reject the request.

Proposal 10a: When the MN configures inter-CU MCG LTM candidates, the MN sends a notification to the SN to disable inter-CU SCG LTM.

Proposal 10b: When the SN decides to configure inter-CU SCG LTM candidates, the SN sends a request to the MN, which may be accepted or rejected

In Rel-18, intra-CU SCG LTM is configured by the SN without MN involvement (i.e., candidate configurations are included in an SN message sent to the UE via SRB3 or forwarded by the MN via SRB1). If inter-CU MCG LTM is also configured, the target SCG configuration may be changed so that the intra-CU SCG LTM configuration may need to be changed or released. Since the target SCG configuration of inter-CU MCG LTM is prepared by the SN, the SN can include any modification or release of intra-CU SCG LTM configurations.

Proposal 11: In inter-CU MCG LTM with SN kept, the SCG part of the candidate configuration can include the addition, modification or release of intra-CU SCG LTM configurations (without MN involvement).

RAN2 agreed that subsequent CPAC is not simultaneously configured by the MN and by the SN. Subsequent CPAC is similar to SCG LTM, except that it is executed when the execution condition is met while SCG LTM is executed upon receiving an LTM cell switch MAC CE from the SCG.

Proposal 12: Discuss whether to support configuring SCG LTM by the MN and by the SN simultaneously. If this is not supported, when inter-CU SCG LTM is configured, intra-CU SCG LTM should also be configured by the MN (this is not supported in Rel-18).

# 3	Conclusion

This contribution makes the following proposals:

LTM preparation

Proposal 1:	The LTM-CSI-Resource in the common CSI resource configuration (configure using ltm-CSI-ResourceConfigToAddModList-r18) can refer to NZP CSI-RS resources of one or more candidate cell.

Proposal 2:	The configuration of each candidate cell can include a list of NZP CSI-RS resources.

LTM cell switch execution

Proposal 3:	Support RACH-based inter-CU LTM cell switch, including CBRA and CFRA.

Inter-CU MCG LTM with SN unchanged/released

Proposal 4:	In inter-CU MCG LTM cell switch in the NR-DC scenario, the candidate MN decides whether to reconfigure or release the SCG.

Proposal 5:	In inter-CU MCG LTM with SN unchanged, the UE performs reconfiguration with sync of the SCG (including RACH).

Proposal 6:	In inter-CU MCG LTM with SN unchanged, it is up to network implementation whether the PSCell is changed or not.

Proposal 7:	RAN2 can discuss the SN key delivery procedure for inter-CU MCG LTM with SN unchanged when the solution for MN key update in non-DC scenario is stable.

Inter-CU SCG LTM with MN unchanged

Proposal 8: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch.

Proposal 9: For inter-CU SCG LTM, support to trigger PDCP data recovery in PDCP entities at the MN for MN terminated SCG/split bearers.

Coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM

Proposal 10a: When the MN configures inter-CU MCG LTM candidates, the MN sends a notification to the SN to disable inter-CU SCG LTM.

Proposal 10b: When the SN decides to configure inter-CU SCG LTM candidates, the SN sends a request to the MN, which may be accepted or rejected.

Proposal 11: In inter-CU MCG LTM with SN kept, the SCG part of the candidate configuration can include the addition, modification or release of intra-CU SCG LTM configurations (without MN involvement).

Proposal 12: Discuss whether to support configuring SCG LTM by the MN and by the SN simultaneously. If this is not supported, when inter-CU SCG LTM is configured, intra-CU SCG LTM should also be configured by the MN (this is not supported in Rel-18).

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.