3GPP TSG-RAN WG2 Meeting #127bis	R2-2409143

Hefei, China, 14 - 18 October 2024

Agenda Item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Intra-CU conditional LTM

Document for: Discussion and Decision

# 1	Introduction

In RAN #105 meeting, the WID of Rel-19 NR mobility enhancements Phase 4 was updated. According to the updated WID [1], conditional LTM (L1/L2 Triggered Mobility) should be specified for intra-CU case in Rel-19, and we should start with non-DC case discussion.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we discuss the main procedures of intra-CU conditional LTM, including the configuration, RACH-less cell switch execution, and cell switch completion.

# 2	Discussion

## 2.1	Event configuration for conditional LTM

Compared with Rel-18 network-triggered intra-CU LTM, the main difference of conditional LTM is that the UE should be configured with events for conditional LTM cell switch triggering evaluation.

As for the event configuration, there are two possible ways: one is to use condEvent Ax event which is similar to CHO, another is to use Event LTM x which is similar to event-triggered L1 report for LTM.

- condEvent Ax

In CHO, e.g., when condEventA3 is associated with a candidate configuration, the UE considers the cell which the PCI of the SpCell included in the candidate configuration as applicable cell. Then, the UE evaluates whether the condEventA3 is fulfilled for the applicable cell. The evaluation is based on cell level measurement results but if nrofSS-BlocksToAverage and nrofCSI-RS-ResourcesToAverage are not configured, the cell measurement result is the measurement results of the best beam. The derivation of cell measurement results applies L3 filtering but the L3 filtering can be turned off if filterCoefficient is configured with value 0.

Observation 1a: It is possible to configure the L3 derivation of cell measurement results so that the cell measurement result is the result of the best beam (the best SSB or CSI-RS) without any L3 filtering.

Observation 1b: It is possible to configure the existing condEvent Ax with such cell measurement results.

- Event LTM x

In Rel-19 LTM discussions, to support event-triggered L1 report, new events called Event LTM x are introduced. Some related agreements are quoted below:

RAN2 #126 Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. RAN2 #127 Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

According to the current agreements, beam level evaluation for Event LTM x is already agreed. Whether to support cell level evaluation is FFS for conditional LTM case.

Observation 2a: The Rel-19 LTM Event x supports beam level evaluation, and FFS cell level evaluation for conditional LTM cell switch.

In the event-triggered L1 report for network-triggered LTM, in beam level evaluation, if only one or few beams are better than the current serving beam, the event is triggered and the report is transmitted. It is beneficial to send the report to let the network know the current channel condition. The network can decide the LTM triggering by considering the multiple beams measurement results. However, in the case of conditional LTM cell switch triggering, if only one beam of candidate cell is better than the current serving beam, it is not reliable to directly trigger cell switch procedure, because no other beam in the candidate cell is good enough which may cause RLF if the UE switches to this cell and continues to move. In addition, there may be a better beam in the current serving cell, so no need to trigger cell switch. Therefore, it is better that cell level evaluation can be supported for reliable cell switch decision, in addition to the beam level evaluation. The network can configure a suitable one for a particular deployment scenario.

Proposal 1:	Support cell level evaluation for the event of conditional LTM cell switch triggering, in addition to beam level evaluation.

We assume no L3 filtering is applied when evaluating LTM Event x, because in the report procedure, the PHY layer measures the results and passes to the MAC layer to send the results via MAC CE. There should be no RRC involvement to do the L3 filtering. Whether to apply L1 filtering is still FFS.

Observation 2b: In the event-triggered L1 reporting procedure, the evaluation of Rel-19 LTM Event x does not apply L3 filtering.

Based on the above observations, from the beam or cell level evaluation and L3 filtering perspectives, the legacy condEvent Ax can cover the function of Rel-19 LTM Event x. It seems fine to use the condEvent Ax for conditional LTM cell switch triggering. However, companies may argue that the Rel-19 LTM Event x is also fine for conditional LTM if cell level evaluation is added, since it also supports beam level evaluation and cell level evaluation. RAN2 can discuss whether the legacy condEvent Ax is flexible enough to support conditional LTM cell switch triggering.

Proposal 2:	Discuss whether the legacy condEvent Ax is flexible enough to support the conditional LTM cell switch triggering.

In CHO, the execution condition for different candidate configurations can be different, e.g., by using different event type or different thresholds. For conditional LTM, the same flexibility should be supported.

Proposal 3:	The execution condition for conditional LTM is configured per LTM candidate configuration.

## 2.2	RACH-less LTM cell switch

In Rel-18 network-triggered intra-CU LTM, RACH-less cell switch is one of the main features that enables low interruption handover. The motivation of Rel-19 conditional LTM is to increase the robustness and also to enjoy the low interruption so RACH-less cell switch should also be supported for conditional LTM.

Proposal 4:	Support RACH-less cell switch for intra-CU conditional LTM.

In the following sections, we discuss how to achieve RACH-less cell switch procedure.

### 2.2.1	Early TA acquisition

To enable RACH-less cell switch, the UE needs to acquire the target cell TA before cell switch. In Rel-18, both PDCCH-ordered early RACH and UE-based TA measurement are supported. In the UE-based TA measurement procedure, when the network enables the UE-based TA measurement by RRC, the UE measures the TA by the DL timing gap between the source cell and the target cell, and the exact TA measurement time is up to UE implementation. For conditional LTM, the UE is also able to perform UE-based TA measurement before the execution condition is met, so the UE can use the measured TA to send the first UL message to the target cell when the execution condition is met.

Proposal 5:	Support UE-based TA measurement for conditional LTM.

For PDCCH ordered early RACH, the source DU should send the PDCCH order to the UE when the source DU determines that the UE can be switched to a candidate cell soon. In Rel-18 network triggered LTM, it is the source DU that sends the PDCCH order and the LTM cell switch MAC CE, so it is easier to determine when to send the PDCCH order and when to send the LTM cell switch MAC CE. However, in conditional LTM, the source DU does not know when the UE will switch to another cell. If the source DU sends a PDCCH order very early, the TA may be out of date when the UE triggers cell switch later. If the source DU sends a PDCCH order late, the UE may have already attempted to execute cell switch and then failed to receive the PDCCH order. To solve this problem, RAN2 needs to discuss assistance information (e.g., L1 report or new assistance information) from the UE to the source DU, so that the source DU has sufficient information to send a PDCCH order at a proper time.

Proposal 6:	Discuss whether to support PDCCH-ordered early RACH for conditional LTM. If supported, discuss how to obtain the necessary information (e.g., L1 report or some UE-initiated assistance information) to allow the source DU know when to send a PDCCH order.

If PDCCH-ordered early RACH is supported, another issue RAN2 needs to discuss is whether the candidate cell replies a RAR when receiving a preamble for early TA acquisition, as shown in the agreement below:

RAN2 #126 RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

As we discussed above, the source DU should send a PDCCH order at a proper time, this means that there should be some time before the UE triggers cell switch. When the candidate DU receives a preamble, it sends the TA to the CU and then to the source DU, then the source DU sends the TA value to the UE. This does not require the UE to monitor RAR from candidate cells, which reduces the impact to the serving cell data transfer procedure.

Proposal 7:	If PDCCH-ordered early RACH is supported for conditional LTM, the UE does not monitor RAR from the candidate cell. Instead, the UE receives the TA from the current serving cell.

### 2.2.2	UL grant acquisition

To achieve RACH-less cell switch, the UE needs a UL grant to send UL message to the target cell. In Rel-18 network-triggered LTM, both CG-based and DG-based RACH-less cell switch are supported. In the CG-based RACH-less LTM cell switch procedure, the UE is pre-configured with configured grant, and configured grant occasions are associated with downlink beams. The UE selects a CG occasion which is associated with the downlink beam indicated in the LTM cell switch MAC CE. In conditional LTM, the UE can also be configured with CG occasions that are associated with downlink beams. The UE can select a target downlink beam by itself and then use a corresponding CG occasion. This helps the network to receive the first UE uplink transmission (assuming reciprocity) and to know which beam to use for downlink transmissions.

Proposal 8:	Support CG-based RACH-less cell switch for conditional LTM. The UE selects a downlink beam of the target cell by itself and uses a corresponding CG occasion.

In the Rel-18 DG-based RACH-less LTM cell switch procedure, it is the target cell/DU that sends a PDCCH to the UE. This is feasible because the target cell/DU knows when the UE performs the LTM cell switch but for conditional LTM, the network cannot know whether and when the UE will switch to a cell. There could be two methods: A first method is that the UE sends a cell switch notification to the source cell providing the necessary information for the source cell to send the cell switch notification to the target cell. This method can only work if the source cell link is good enough to successfully transmit the indication. The second method is that the UE sends an SR to the target cell and then waits for the DG from the target cell. In this method, RAN2 needs to discuss the benefit compared with CFRA. In LTM cell switch procedure with CFRA, the UE first sends a preamble to the target cell, and receives a RAR which includes an UL grant. If the UE applies the second method above, the UE first sends a SR, then receives an UL grant, which is similar to CFRA procedure since they both have two steps. In addition, RAN2 and RAN1 need to discuss how/whether the target cell can successfully receive the SR if it has not completed beam alignment with the UE.

Proposal 9:	Discuss whether to support DG-based RACH-less cell switch for conditional LTM.

## 2.3	Cell switch completion

In Rel-18 network-triggered LTM, the source cell/DU may stop transmission with the UE after sending the LTM cell switch MAC CE (or receives the HARQ ACK for the LTM cell switch MAC CE). However, in conditional LTM, there is no LTM cell switch MAC CE and the source cell/DU is not aware that the UE is switching cell , especially in the inter-DU scenario.

After the UE successfully switches to a target cell by conditional LTM, the target DU should send an Access Success message to the CU, then the CU should notify the source DU to stop the transmission with the UE. Note that subsequent LTM is supported for conditional LTM, so the source DU does not release the UE context if the source DU still provides candidate cells for the UE for subsequent conditional LTM. The detailed signalling design is up to RAN3.

Proposal 10:	At least for the inter-DU case, the CU notifies the source DU to stop the transmission with the UE after a successful conditional LTM cell switch. The source DU does not release the UE context if the source DU still provides candidate cells for the UE for subsequent conditional LTM. The detailed signalling design is up to RAN3.

# 3	Conclusion

This contribution makes the following proposals:

Event configuration for conditional LTM

Proposal 1:	Support cell level evaluation for the event of conditional LTM cell switch triggering, in addition to beam level evaluation.

Proposal 2:	Discuss whether the legacy condEvent Ax is flexible enough to support the conditional LTM cell switch triggering.

Proposal 3:	The execution condition for conditional LTM is configured per LTM candidate configuration.

RACH-less cell switch

Proposal 4:	Support RACH-less cell switch for intra-CU conditional LTM.

Proposal 5:	Support UE-based TA measurement for conditional LTM.

Proposal 6:	Discuss whether to support PDCCH-ordered early RACH for conditional LTM. If supported, discuss how to obtain the necessary information (e.g., L1 report or some UE-initiated assistance information) to allow the source DU know when to send a PDCCH order.

Proposal 7:	If PDCCH-ordered early RACH is supported for conditional LTM, the UE does not monitor RAR from the candidate cell. Instead, the UE receives the TA from the current serving cell.

Proposal 8:	Support CG-based RACH-less cell switch for conditional LTM. The UE selects a downlink beam of the target cell by itself and uses a corresponding CG occasion.

Proposal 9:	Discuss whether to support DG-based RACH-less cell switch for conditional LTM.

Cell switch completion

Proposal 10:	At least for the inter-DU case, the CU notifies the source DU to stop the transmission with the UE after a successful conditional LTM cell switch. The source DU does not release the UE context if the source DU still provides candidate cells for the UE for subsequent conditional LTM. The detailed signalling design is up to RAN3.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.