3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409145

Hefei, CN, Oct 14-18, 2024

Source:	III

Title:	Discussion on XR scheduling enhancements

Agenda item:	8.7.4

Document for:	Discussion

- Introduction

During the RAN2#127 meeting [1], the following agreements were noted:

- For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

Topics relative to XR-specific capacity issues was raised as mentioned above. In this contribution,” The Conflict of Multiple Triggering Thresholds” and “The issues of multiple reporting thresholds and non-delay critical data in the DSR” are addressed.

- Discussion

In the RAN2#127 meeting [1], the following agreements were noted:

For enhanced DSR:

    - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

    - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

    - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

To support enhancement for scheduling, the following issues need to be further considered:

- The conflict of multiple triggering thresholds,

- The case for a single triggering threshold,

- The issues of multiple reporting thresholds and non-delay critical data in the DSR.

  - The Conflict of Multiple Triggering Thresholds

In the realm of 5G Extended Reality (XR) applications, such as augmented reality (AR) and virtual reality (VR), ensuring optimal performance is critical. This involves managing network delays meticulously to maintain a seamless user experience. One of the tools used to achieve this is the Delay Status Report (DSR). This mechanism allows UE to report network delay conditions to the network, enabling adaptive adjustments to be made to meet the stringent requirements of XR applications. DSR are crucial for monitoring real-time performance metrics, particularly latency, which can significantly impact the quality of XR experiences. By providing timely and accurate delay information, DSR helps to dynamically adjust network parameters to address latency issues and improve overall service quality.

To manage network delays effectively, DSR can be configured with multiple thresholds. Each threshold corresponds to a specific delay level, triggering different network responses based on the severity of the delay. This multi-threshold approach allows for granular control and tailored actions, optimizing the network's ability to handle varying degrees of delay.

However, implementing multiple triggering thresholds introduces significant challenges:

- Inconsistent response strategies: Different thresholds might trigger varying network responses, leading to unpredictable performance and potentially worsening the XR experience instead of improving it.

- Complexity in delay management: Managing multiple thresholds increases the complexity of delay management, making it difficult to implement a coherent strategy that effectively addresses all delay scenarios.

Observation 1: Implementing multiple triggering thresholds presents several significant challenges, including inconsistent response strategies and increased complexity in delay management.

  - The Case for a Single Triggering Threshold

Given these challenges, it is advisable to use a single triggering threshold for DSR. A single threshold simplifies the delay management process, ensuring:

- Consistency: With one threshold, the network has a clear, unified definition of delay-critical conditions, leading to more consistent and predictable performance.

- Simplified Implementation: Managing a single threshold reduces the complexity of delay management systems, making it easier to implement and maintain effective network performance strategies.

- Efficient Resource Allocation: A single threshold enables more efficient allocation of network resources, as there is no need to reconcile conflicting priorities or definitions.

To avoid these issues and ensure optimal performance for 5G XR applications, adopting a single triggering threshold is the recommended approach. This simplifies delay management, promotes consistency, and enhances the overall user experience in the dynamic environment of XR.

Proposal 1: To avoid these issues and ensure optimal performance for 5G XR applications, adopting a single triggering threshold is the recommended approach.

  - The issues of Multiple reporting thresholds and non-delay critical data in the DSR

In Extended Reality (XR) applications, managing DSR effectively is essential for optimal performance. When gNB receives a report of non-delay critical data in a DSR, it relies on the remaining time to determine when this data may transition to delay critical status, without prioritizing immediate resource allocation for that data.

When gNB identifies non-delay critical data, it does not allocate resources to UE right away. Instead, it monitors the remaining time to assess if and when the data will change status. Only when gNB determines that the data has become delay critical does it begin to consider resource allocation for that UE.

Key Issues:

- Delay in Resource Allocation: The time taken for gNB to recognize the transition from non-delay critical to delay critical can lead to significant delays in resource allocation, potentially impacting performance.

- Lack of Feedback on Data Transmission: Since UE does not inform gNB about which specific data has been transmitted, gNB cannot verify if the data in the DSR has been sent. This lack of feedback creates ambiguity in the system.

- Inconsistent Data Requirements: The delay in resource allocation and the absence of feedback can result in a misalignment between the resource allocates of gNB and the actual data transmission status from UE. This inconsistency can lead to incorrect resource configurations, ultimately degrading service quality.

The process of managing non-delay critical data in DSRs highlights significant synchronization challenges between UE and gNB in XR. The inability of gNB to ascertain the transmission status of data can lead to resource misallocation and performance degradation. Addressing these synchronization issues is critical for optimizing DSR functionality and improving overall network efficiency.

Observation 2: If DSRs are to include non-delay critical data, the management of DSR in XR cause critical synchronization challenges between UE and gNB.

Observation 3: For DSRs that include Non-delay critical data in XR, it is required to design a Real-Time feedback mechanism between UE and gNB to enhance resource allocation efficiency and synchronization.

Proposal 2: If DSRs are to include non-delay critical data, the Real-Time feedback mechanism should be discussed.

- Conclusion

The related proposals and observations from above discussions are below:

Observation 1: Implementing multiple triggering thresholds presents several significant challenges, including inconsistent response strategies and increased complexity in delay management.

Proposal 1: To avoid these issues and ensure optimal performance for 5G XR applications, adopting a single triggering threshold is the recommended approach.

Observation 2: If DSRs are to include non-delay critical data, the management of DSR in XR cause critical synchronization challenges between UE and gNB.

Observation 3: For DSRs that include Non-delay critical data in XR, it is required to design a Real-Time feedback mechanism between UE and gNB to enhance resource allocation efficiency and synchronization.

Proposal 2: If DSRs are to include non-delay critical data, the Real-Time feedback mechanism should be discussed.

References

[1] 	Report of 3GPP TSG RAN WG2 #127