3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409154

Hefei, People’s Republic of China, 14 – 18 Oct 2024

Agenda item:	8.1.5

Source:	Samsung

Title:	Model transfer/delivery: key issues and way forward for RAN2

Document for:	Discussion & Decision

# Introduction

In TR 38.843, model transfer and model delivery are defined as follows:

AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

As per above, model transfer refers to model delivery which is not transparent to 3GPP system. Sometimes these terms are used interchangeably e.g. in the definition of Solution 4a (TR 38.843, section 7.2.1.4), where ‘transfer’ should not apply but only ‘delivery’. In this tdoc we will use these terms interchangeably, even though we note they are not. As the focus of the discussion in the present paper is whether delivery of a model in a non-transparent way is needed, our focus is therefore anyway on model transfer.

During the R18 Study Item on AI/ML for air interface, RAN2 studied and captured 8 potential model transfer solutions (as listed in section 7.2.1.4 of the above TR) covering all possible scenarios for the model transfer/delivery, and analysed current status and gaps, and potential RAN specification impact.

In this document we use these model transfer solutions as a starting point. We link those to collaboration levels x/y/z as captured in ibid, Section 4.3. We then highlight recent RAN1 agreements related to collaboration levels, and explain how they impact RAN2 discussion on model transfer solutions. We noted that RAN1 and RAN2 discussed the same topic (model transfer) but from different angles, with RAN1 focusing on collaboration levels and RAN2 focusing on any signalling that may or may not be needed for model transfer/delivery. RAN1 made some progress during work item phase, while RAN2 has not progressed this topic as yet. We therefore provide a set of related proposals for RAN2’s consideration, largely based on RAN1 agreements and how we believe they impact our (RAN2) work.

# Link between identified model transfer solutions and collaboration levels

During the Study Item, RAN2 identified the following solutions for model transfer/delivery (TR 38.843, section 7.2.1.4):

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

-	Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

During the study item, RAN1 also identified potential collaboration levels between UE and Network for model delivery/transfer and summarized them as shown in the following table (ibid, Section 4.3):

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

The TR does not appear to capture the link between solutions for model transfer (focus of RAN2), and the collaboration levels (focus of RAN1), even though such link clearly exist. In some cases a solution is incompatible with the given collaboration level by definition. For instance, Solution 1a for model transfer cannot be developed with a collaboration level y, as collaboration level y does not offer a possibility for the network to be aware of the model and transfer it using RRC. In some other cases, a single solution could be developed with multiple levels of collaboration; as an example, Solution 4b could function at both collaboration levels z4 and z5.

Observation 1: Solutions for model transfer (chiefly discussed by RAN2) and collaboration levels between the UE and the network (chiefly discussed by RAN1 so far), are not independent.

# RAN1 agreements impacting collaboration levels and their impact on RAN2’s work

During the study phase of the R19 WI, RAN1 reached certain conclusions as captured in RP-241911. We highlight here those which we argue impact RAN2’s work:

From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons... - From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons... - From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

Based on RAN1 conclusion immediately above, we put it forward that RAN2 should also focus on y, z1 and z4 because z2, z3 and z5 are deprioritized in RAN1:

Proposal 1: RAN2 to focus on y, z1 and z4 collaboration levels when studying the required signalling to support model transfer/delivery.

Proposal 2: RAN2 to deprioritize model transfer/delivery solutions (TR 38.843, section 7.2.1.4) which are identified as incongruous with y, z1 or z4 collaboration levels.

# Further details on collaboration levels

## Case y

In case of collaboration level y, the model is delivered over-the-top as model storage location is outside of the 3GPP network:

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |

Similar to the data collection discussion, RAN2 can assume that there is no spec impact to support Case y:

Proposal 3: RAN2 to confirm that any solutions for model transfer/delivery at collaboration level y have no specification impact.

## Case z1

In case of collaboration level z1, the model is trained in UE-side/neutral site but it is stored in 3GPP network. Therefore, at least from the UE-side model point of view, model transfer/delivery would be required from UE to 3GPP network and also from 3GPP network to UE if the trained model is used at UE side.

Although the model is provided by the NW, this option supports model format in a proprietary format. Therefore, it is not clear how the UE and NW could manage model transfer/delivery. There is a view that there should be cross-vendor collaboration to ensure that model is correctly transferred from NW to UE. In our understanding, it is due to these reasons that there was not much discussion in RAN1 on the characteristics of underlying signalling for Case z1 so far. We believe that it is not possible to design details of signalling for z1 without further RAN1 progress:

Proposal 4: RAN2 to wait for further RAN1 input on the operation of case z1.

## Case z4

In case of collaboration level z4, model storage is located at 3GPP network and model is trained in NW side. However, the actual model inference is performed at the UE using the model transfer.

In RAN1 #118, it was discussed whether it is necessary to study z4 for one-sided model. There seemed to be divergent view about the feasibility and necessity of Case z4 for one-sided model. Due the diverging views, RAN1 concluded to study case z4 as a generic case i.e. applicable to both one sided (UE side) and two-sided model. However, it is noted that the main use case for z4 would be the two-sided model. In RAN1, it has not been clearly discussed why and how case z4 could be operated in one-side model scenario. We believe that RAN1 needs to answer this question.

Observation 2: RAN1 has been discussing that collaboration level z4 could be applicable to both one-sided model and UE-side model of the two-sided model, but there is no conclusion yet about feasibility and usefulness of applying z4 to one-sided model.

Proposal 5: RAN2 to wait for RAN1 progress on the feasibility of applying collaboration level z4 to one-sided model.

In addition, for z4 collaboration level, model transfer is supported with open format of a known model structure at UE. This implies that 3GPP should define signalling framework for a specific model structure supported in both the UE and NW.

RAN1 identified two alternatives (Alt A and Alt B) for further study and main difference seems to be which entity (between UE and NW) provides known model structure(s):

In Alt. A, UE reports the supported known model structure(s) to the network (Step A-1), while in Alt B, NW provides the candidate known model structure(s).  It seems common in both alternatives that NW provides the UE with the parameters for supported known model structure(s).

The key consideration to identify the requirements for the signalling would be the characteristic of known model structure.

In RAN1, the following agreement was made for the “known model structure(s)”.

Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

Based on this RAN1 agreement, it is understood that information related to the known model structure is exchanged instead of raw model data. However, RAN2 would need more details to understand characteristics of the required information in order to design any signalling to support it.

Proposal 6: RAN2 to wait for RAN1 progress on the detailed characteristics of “known model structure” related parameters for collaboration level z4.

# Conclusion

On the topic of the link between collaboration levels and model transfer/delivery options, recent RAN1 agreements on collaboration levels and how they impact RAN2 work on model transfer/delivery, in this tdoc we proposed the following:

Observation 1: Solutions for model transfer (chiefly discussed by RAN2) and collaboration levels between the UE and the network (chiefly discussed by RAN1 so far), are not independent.

Proposal 1: RAN2 to focus on y, z1 and z4 collaboration levels when studying the required signalling to support model transfer/delivery.

Proposal 2: RAN2 to deprioritize model transfer/delivery solutions (TR 38.843, section 7.2.1.4) which are identified as incongruous with y, z1 or z4 collaboration levels.

Specifically related to collaboration levels y and z1, we made the following proposals:

Proposal 3: RAN2 to confirm that any solutions for model transfer/delivery at collaboration level y have no specification impact.

Proposal 4: RAN2 to wait for further RAN1 input on the operation of case z1.

And finally, on the issue of collaboration level z4, we made the following proposals:

Observation 2: RAN1 has been discussing that collaboration level z4 could be applicable to both one-sided model and UE-side model of the two-sided model, but there is no conclusion yet about feasibility and usefulness of applying z4 to one-sided model.

Proposal 5: RAN2 to wait for RAN1 progress on the feasibility of applying collaboration level z4 to one-sided model.

Proposal 6: RAN2 to wait for RAN1 progress on the detailed characteristics of “known model structure” related parameters for collaboration level z4.