3GPP TSG-RAN WG2 Meeting #127bis	R2-2409155

Hefei, China, Oct. 14 – 18, 2024

Agenda item:	8.7.4.1

Source:	Samsung

Title:	LCP enhancements for Rel-19 XR

Document for:	Discussion & Decision

# Introduction

In the last RAN2#127 meeting, RAN2 made the following agreements for scheduling enhancements:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

Also, according to the revised WID [1], as highlighted below, RAN2 should specify the additional logical channel priority handling using delay/deadline information of packets.

Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In this paper, we discuss the essential aspects of the operations related with the additional LCH priority.

# Discussion

## Using Delay-Critical Data delay/deadline info in LCP Enhancements

In Rel-18, the concept of delay-critical data was defined by comparing remaining PDCP discardTimer value with the remainingTimeThreshold configured by NW, for DSR triggering and reporting. In Rel-18, the remainingTimeThreshold is configured for a Logical Channel Group, or LCG, and so the assumption is that all the LCHs which belong to the same LCG have the same remainingTimeThreshold. This same threshold could now be reused when determining whether a LCH carries delay-sensitive data, for the purposes of determining whether to apply the additional (Rel-19 introduced) LCH priority of a LCH. The LCP for XR in Rel-19 should (as per the WID quoted above) use the delay/deadline information of packets (of which remainingTimeThreshold is one).

There could be an option that another measure of information delay/deadline carried by a LCH including a threshold different from the one configured for Rel-18 DSR could be introduced for defining another concept of delay-critical data dedicated for Rel-19 LCP enhancements, with the rationale that Rel-18 DSR triggering/reporting time is a different mechanism from that of the LCP, such that the remainingTimeThresholds should be tailored for respective requirements of the two mechanisms.

But we believe the benefit from introducing different respective thresholds for LCP and for Rel-18 DSR is marginal, since NW would anyway allocate UL resource for the delay-critical data as soon as possible based on the DSR, such that the time gap between DSR and LCP would be minimized by NW. So, we think it a proper baseline to adopt is to consider reusing the remainingTimeThreshold and the way we define the delay-critical data in Rel-18 DSR, for Rel-19 LCP enhancements, in order to avoid unnecessary signalling overhead/ambiguity as well as extra UE complexities in taking into account two types of delay-critical data.

Proposal 1. RAN2 to reuse the definition of delay-critical data defined for Rel-18 DSR in Rel-19 LCP enhancements, when determining whether to apply the enhanced Rel-19 XR LCP procedure (including any additional LCP priority) to a given LCH.

## Additional LCH Priority in LCP

One issue would be whether we should apply the additional LCH priority for the entire data of the LCH (LCH-level prioritization) including both delay-critical and non-delay-critical data, or only for the delay-critical data of the LCH (SDU-level prioritization), when allocating UL resource for the LCH during LCP. This issue was identified in RAN2#126-bis via the FFS immediately below:

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

- - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

This issue closely relates to the trade-off between complexity and capacity. That is, to enable SDU-level prioritization, UE should check whether each buffered SDU of an LCH is delay-critical or not during LCP when allocating UL resource, which brings in non-trivial additional complexity compared to the legacy LCP, while, with LCH-level prioritization, UE only needs to determine the appropriate LCH priority based on the existence of the delay-critical data, and reuse the legacy LCP procedure, when allocating UL resource. On the other hand, SDU-level prioritization can enable higher XR capacity, since the UL resource is fully exploited by the delay-critical data across different LCHs, while LCH-level prioritization cannot completely resolve the issue of delay-critical data being delayed by non-delay-critical data.

The UE already needs to check PDCP discardTimers among all SDUs that are buffered for a LCG (and how the value of individual discardTimers compares with the remainingTimeThreshold) but have not been transmitted in any MAC PDU. Therefore this check already needs to happen. However, if we were to introduce SDU-level prioritization as a result of this check, the complexity of the LCP procedure itself would be increased, potentially significantly. It could also impact the scheduler in the network, which currently allocates resources based on buffer reports of LCG granularity. Since LCP is one of the most complicated mechanisms in UE implementation, which also faces tight delay budget (i.e., the time gap between UL grant reception and PUSCH transmission), we think LCH-level prioritization is a reasonable compromise to be considered for Rel-19 XR. Additionally, we believe introducing SDU-level prioritization into LCP would violate the existing RAN2 agreement to no longer consider the enhancement of the LCP restriction. And finally, the following was also agreed in the same meeting (RAN2#126-bis), which already implies the LCH-level prioritization:

LCP prioritization within a logical channel will not be considered in RAN2 discussions

We therefore propose the following:

Proposal 2. The additional LCH priority is applied to the entire data of a LCH with both delay-critical and non-delay-critical data.

Another issue would be whether the additional LCH priority is applied during the entire resource allocation procedure of LCP, i.e., both 1st and 2nd Rounds, or part of it, where 1st and 2nd Rounds are highlighted below in yellow and green, respectively.

TS 38.321 The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

Regarding this issue, the point worth discussing is whether to allow the UE check the availability of delay-critical data per LCH when starting 1st Round of the LCP, or both 1st and 2nd Rounds, respectively, e.g., how to handle the case when all the delay-critical data is exhausted by 1st Round, and no residual delay-critical data remains when starting 2nd Round.

From the UE complexity point of view, it is more “light-weight” to prioritize an LCH during the whole resource allocation procedure in the same manner, as long as it has delay-critical data when starting the LCP.

From the fairness and system capacity points of view, it may be more beneficial to prioritize an LCH (in 1st or 2nd Round) only when delay-critical data is available for transmission, i.e., if the delay-critical data of an LCH is exhausted in 1st Round, the LCH would not be prioritized in 2nd Round.

Accordingly, we can list up two possible options:

- Option 1: additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

- Option 2: additional LCH priority is applied for an LCH in 1st Round of resource allocation procedure in LCP, if the LCH has delay-critical data available for transmission when starting the 1st Round, and the additional LCH priority is applied for an LCH in 2nd Round of resource allocation procedure in LCP, if the LCH has residual delay-critical data available for transmission when starting 2nd Round.

So, a discussion is expected on whether the UE should additionally check the availability of the delay-critical data when starting 2nd Round of the resource allocation procedure in LCP.

Proposal 3. RAN2 to discuss on the preferred option between:

- Option 1: additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

- Option 2: additional LCH priority is applied for an LCH in 1st Round of resource allocation procedure in LCP, if the LCH has delay-critical data available for transmission when starting the 1st Round, and the additional LCH priority is applied for an LCH in 2nd Round of resource allocation procedure in LCP, if the LCH has residual delay-critical data available for transmission when starting 2nd Round.

## Additional LCH Priority in Other Procedures

In general, LCH priority is used in 1) resource allocation in LCP, 2) BSR triggering, 3) truncated BSR LCG selection, and 4) intra-UE prioritization.

For BSR triggering and truncated BSR LCG selection, it is not essential to consider the additional LCH priority of the LCH with delay-critical data, considering that DSR can handle the delay-critical data.

According to 38.300, “in case a configured uplink grant transmissions or a dynamically allocated uplink transmission overlaps in time with a scheduling request transmission, the UE prioritizes the transmission based on the comparison between the priority of the logical channel which triggered the scheduling request and the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDU associated with the overlapping resource”.

Therefore, in this legacy intra-UE prioritization, when more than one UL grant(s) and/or SR(s) overlap in time domain, a priority-based comparison among the overlapped UL grant(s) and/or SR(s) is conducted to determine which UL grant or SR should be transmitted. The priority-based comparison is based on legacy (default) LCH priority, i.e., the priority of UL grant is defined as the highest (legacy) priority of LCHs multiplexed in the corresponding MAC PDU, and the priority of SR is defined as the (legacy) priority of the LCH triggering the SR.

In the last meeting, during the discussion on Rel-19 LCP enhancement, an FFS was added on whether/how the additional LCH priority (originally intended to be used for LCP enhancement) will affect the intra-UE prioritization. The expected operation regarding this issue, would be, in the priority-based comparison elaborated above, when defining the priority of UL grant, the additional LCH priority can be used (instead of legacy priority) for LCH(s) with delay-critical data (similar to the case of LCP enhancement). Similarly, the priority of SR can be defined as the (legacy) priority of the LCH if without delay-critical data or the additional priority of the LCH if with delay-critical data, for the LCH triggering the SR.

For intra-UE prioritization, we can expect similar benefit to that of LCP enhancements (i.e., preventing delay-critical data from being delayed by non-delay-critical data), via the prioritization of the delay-critical data, by making use of the additional LCH priority in determining the priority of an uplink grant, and/or that of LCH triggering an SR.

Observation 1: Prioritization of uplink grant/SR associated with delay-critical data in intra-UE prioritization can be used to achieve a goal consistent with that of LCH enhancements, i.e., to prevent delay-critical data from being delayed by non-delay-critical data.

Proposal 4. RAN2 to consider prioritizing the uplink grant/SR associated with delay-critical data in intra-UE prioritization using the additional LCP priority.

## Prioritization of Data over MAC CE

For delay-critical data, in-time delivery is important. For this, dedicated configured grant or resource separated by LCP have been introduced over multiple releases of NR. In Rel-18, multiple CG occasions, DSR reporting and discard mechanism were introduced to support in-time delivery with higher resource efficiency. However, most of the MAC CEs, e.g., BSR/DSR/PHR, etc., still have higher absolute priorities over any of XR data, and those MAC CEs can use any type of uplink grants. This could make unexpected interruption during LCP and MAC PDU generation, resulting in delay-critical data being delayed due to such MAC CEs. Therefore, we see value in data prioritization over some less important/urgent MAC CEs.

Proposal 5. RAN2 to consider prioritizing LCH with delay-critical data over comparatively less important/urgent MAC CEs.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to reuse the definition of delay-critical data defined for Rel-18 DSR in Rel-19 LCP enhancements, when determining whether to apply the enhanced Rel-19 XR LCP procedure (including any additional LCP priority) to a given LCH.

Proposal 2. The additional LCH priority is applied to the entire data of a LCH with both delay-critical and non-delay-critical data.

Proposal 3. RAN2 to discuss on the preferred option between:

- Option 1: additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

- Option 2: additional LCH priority is applied for an LCH in 1st Round of resource allocation procedure in LCP, if the LCH has delay-critical data available for transmission when starting the 1st Round, and the additional LCH priority is applied for an LCH in 2nd Round of resource allocation procedure in LCP, if the LCH has residual delay-critical data available for transmission when starting 2nd Round.

Proposal 4. RAN2 to consider prioritizing the uplink grant/SR associated with delay-critical data in intra-UE prioritization using the additional LCP priority.

Proposal 5. RAN2 to consider prioritizing LCH with delay-critical data over comparatively less important/urgent MAC CEs.

# References

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3