3GPP TSG-RAN WG2 Meeting #127bis  	                   R2-2409156

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.3

Source: 	CEWiT

Title:	On-demand SIB1 for IDLE/INACTIVE UEs

Document for:	Discussion

- Introduction

Following objectives were agreed for on-demand SIB1 for NES work item in RANP#105[1]

2. Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

This contribution discusses some aspects for on demand SIB1 including WUS configuration and UE behavior to acquire on-demand SIB1(OD-SIB1) etc.

- Discussion

2.1 Access restrictions for legacy UEs

In the RAN2#127[2] meeting, the access restrictions for legacy UEs and for UEs not supporting on-demand SIB1 were discussed for the NES cell with OD-SIB1. Several options were considered, as given below and a downselection is needed among these options:

- Option 1: Legacy UEs bar the OD-SIB1 cell based on the cellBarred bit being set to "barred" in the MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on the absence of a SIB1 indication via ssb-SubcarrierOffset in the MIB.

- Option 3: The network adds cells supporting OD-SIB1 to the list of excluded cells.

In a way, option 1 can be used to bar the legacy UEs from accessing the NES cell since it ensures the barring and is a simple solution with less specification impacts. Here, the legacy UEs will be barred from accessing the cell when it identifies the cellBarred bit as enabled. Whereas, an NES UE supporting OD-SIB1 can proceed with the OD-SIB1 acquisition based on UL WUS configuration by ignoring this bit. Alternatively, option 2 can also be used for the barring of legacy UEs, it is also relevant with a proposal under discussion in RAN1 as given below, where the legacy UEs can assume the NES cell as barred if they receive the indication using ssb-SubcarrierOffset (k_SSB)

### FL Proposal 7-1 RAN1 to further study UE identification of NES cell with on-demand SIB1 based on both of the following options: - Option 1: By WUS configuration - Cell(s) that the UL WUS configuration(s) applies to are NES cell(s) - Option 2: By PBCH payload of NES cell - FFS: How to use the (reserved) k_SSB values to indicate a NES cell with on-demand SIB1 - E.g. k_SSB for FR1 and k_SSB =14 for FR2 FFS: Whether to introduce a new parameter to distinguish Case 1/2 (on-demand SIB1 from NES cell) and Case 3 (on-demand SIB1 from Cell A)

Proposal 1: Prioritize following options to prevent legacy UEs from accessing cells that support OD-SIB1.

Option 1: Legacy UEs bar the OD-SIB1 cell based on the cellBarred bit being set to "barred" in the MIB.

Option 2: Legacy UEs bar the OD-SIB1 cell based on the absence of a SIB1 indication via ssb-SubcarrierOffset in the MIB.

2.2 Channel for UL WUS configuration for on-demand SIB1

In the SI phase for OD-SIB1, it was agreed that either a new SIB or an existing SIB could be used to configure OD SIB1-related settings. The UL WUS configuration for NES cells can be included in SIB1 or another existing SIBx of Cell A. This will avoid the efforts in designing a new SIBx. However, Cell A may need to include multiple UL WUS configurations for different NES cells, which needs further enhancements in case of using exiting SIBx for WUS provision.

Proposal 2: Discuss provision of UL WUS configuration for NES cells by existing SIBx of Cell A.

2.3 WUS configuration from any cell including NES cell

In the RAN2#127[2] meeting, it was agreed that once a Rel-19 NES UE camps on an NES cell, it expects to receive UL WUS configuration updates from the NES cell as given below.

Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

In a scenario where UE finds NES cell before Cell A, if the NES cell is not allowed to provide the UL WUS configuration, the UE will not be able to camp on the NES cell, as it cannot request on-demand SIB1 without the UL WUS configuration and has to wait to find the Cell A. This may cause the change or degradation of the link quality for the NES cell by the time the UE finds the Cell A, thereby increasing delay in the initial access procedure. Hence the WUS configuration should be provided by any cell instead of providing by only Cell A.

Proposal 3: RAN2 to discuss the provision of WUS configuration by any cell including NES cell.

- Conclusion

In this contribution, we discussed the techniques for on demand SIB1 and the following proposals are made,

Proposal 1: Prioritize following options to prevent legacy UEs from accessing cells that support OD-SIB1.

Option 1: Legacy UEs bar the OD-SIB1 cell based on the cellBarred bit being set to "barred" in the MIB.

Option 2: Legacy UEs bar the OD-SIB1 cell based on the absence of a SIB1 indication via ssb-SubcarrierOffset in the MIB.

Proposal 2: Discuss provision of UL WUS configuration for NES cells by existing SIBx of Cell A.

Proposal 3: RAN2 to discuss the provision of WUS configuration by any cell including NES cell.

- Reference

- Chairman Notes, RANP#105, September, 2024.

- Chairman Notes, RAN2#127, August, 2024.