3GPP TSG-RAN WG2 #127bis	R2-2409162

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection for beam management and positioning

Document for:	Discussion

# 1	Introduction

The topic of NW-side data collection was discussed during the Rel.18 SI, and different technical enhancements to support this feature were included in the TR 38.843. In this paper, we first provide an overview of the Rel.18 discussion, and then we draw some proposals for the Rel.19 work item.

# 2	Discussion on NW-side data collection for beam management

Related to network side data collection, the following agreements were reached during the last RAN2#127 meeting. In this contribution we will mainly focus on the FFS from last meeting:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported. FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

In the following, we further develop the expected technical impacts related to this topic, taking into account the above agreements and the online discussion during the last meeting.

## 2.1	Configuration aspects

In the last RAN2 meeting, it was agreed that when the network wants to perform NW-side data collection, it will transmit to the UE a measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. For the case of beam management, the NW may configure the UE with a specific set of CSI-RS/SSB resources specifically configured for training purposes. On such resources, the UE will then start performing layer-1 measurements and log the measurement results and any possible useful information associated to it. 
Going into more details on the necessary configuration, we know that from the point of view of the gNB, this set of CSI-RS/SSB resources used for training purposes may correspond to the set A and set B, and it may include the NW side additional conditions; and we also know that the UE should report at least the L1-RSRP. However, unlike the case of UE-side data collection, the UE does not need to know that a certain resource set is a Set A or B, because that will be left to the NW implementation, and the UE just has to perform the same type of measurements in those sets of resources.
Another aspect to be discussed is how to report the L1 measurements related to NW-side data collection purposes in case the UE does not support data logging. In this case in our view, there is no motivation to support L3 reporting. The L3 reporting framework is very different from the L1 reporting framework in UCI, and if data logging cannot be supported by the UE, it does not make sense from a technical point of view to replicate the UCI framework over L3, also considering the high specification/design complexity.

However, all the above assumptions, and the details of the content of the radio resource configuration, and the details of the content of the radio measurement report should be confirmed by RAN1.

RAN2 to assume the following for the case of beam management:

  - The radio resource configuration for the purpose of NW-side data collection consists of a set of CSI-RS/SSB resources in which the UE shall perform radio measurements and optionally logging.

  - The reporting of L1 measurement results includes, at least, L1-RSRP.

  - If data logging is not supported by the UE, the L1 measurements results associated to the NW-side data collection are reported via UCI.

Ask RAN1 confirmation of the above assumptions, and any further details on the content of the radio resource configuration and on the content of the report including the L1 measurement results.

As captured in the above list of agreements from RAN2#127, it was left as FFS whether multiple configurations for NW-side data collection can be provided to the UE. 
On this regard, we first note that unlike the inference case, the data collection for NW-side model training does not have any requirement on latency. Hence, whenever the network wants to configure the UE to perform radio measurements for the purpose of data collection in a different set of resources, the network can simply reconfigure the UE. Even if this reconfiguration causes a delay gap in the radio measurement, this is not a problem for the case of NW-side data collection.

No need to introduce multiple configurations for NW-side data collection.

Hence, for the case of NW-side data collection, the NW can simply de-configure the UE when data collection is no longer needed, and reconfigure it when needed, without the need for any activation/deactivation mechanism.

No need to introduce dynamic activation/deactivation mechanism for NW-side data collection.

## 2.2	When the UE should perform/stop the radio measurements for NW-side data collection

When the logging should start is something that RAN2 should discuss. In legacy logged MDT, the following approaches are used:

- In logged MDT, the logging may start after receiving the logged MDT configuration from the NW, and as soon as the UE enters RRC_IDLE or RRC_INACTIVE. Then the logging, once started, may be a periodic logging for a configurable logging duration; or

- The logging may be a periodic logging and start upon fulfilling certain events while in RRC_IDLE or RRC_INACTIVE (e.g. UE going out of coverage or camping cell quality below a threshold).

Both above approaches seem to have merits also for the use case of NW-side data collection. For example, if the network needs to collect data for training a model to improve the system performances in regions with poor coverage, it might be useful to configure the UE to start the data collection when the UE ends up in poor coverage regions, or in proximity of cell borders (event-based measurements); whereas in another case, the network can simply configure the UE to perform continuous logging for a certain logging duration.

RAN2 to consider the following approaches for the starting of the NW-side data collection procedure:

  - The UE starts the data collection upon receiving a L3 configuration for NW-side data collection (as in logged MDT).

  - The UE starts the data collection upon fulfilling certain events (as in logged MDT), e.g. based on L1 measurement threshold.

Once the UE starts the data collection, the logging interval depends on the specific CSI measurement configuration in time domain, e.g. periodic/aperiodic measurement.

Once the data collection is started as per Proposal 5, the logging interval is given by the specific CSI measurement configuration (e.g. periodic/aperiodic measurements).

As mentioned above, the data collection can continue for certain time configured by the network (as in logged MDT), or it can continue until certain events are fulfilled. For example, the data collection can continue until the UE is in poor radio coverage. Further, we need to consider potential UE-side issues that may cause the stopping of the data collection. In particular, related to this latter issue, it agreed in RAN2#127, that if the UE becomes memory limited, the data collection could be stopped.

Besides the case of memory limitation issues (already agreed in RAN2#127), RAN2 to consider the following additional mechanisms for the stopping of the NW-side data collection procedure:

  - The UE continues data collection for a certain configurable logging duration (as in logged MDT)

  - The UE continues data collection until certain events are fulfilled (as in logged MDT), e.g. based on L1 measurement threshold.

## 2.3	How the UE should report the collected data

In RAN2#126 meeting, it was first agreed to support the periodical reporting of the logged/collected data. Then, in RAN2#127 meeting, it was agreed to support the “on-demand” reporting of the logged data, and it was left as FFS whether to also support event-based reporting. In this section, we explore whether the “on demand” and the “periodic” reporting can easily coexist, and whether it is worth specifying both approaches.

Related to the on-demand reporting, we note that this is already supported in legacy for the reporting of logged measurement data (logged MDT), as well as for the reporting of all the SON reports (RLF-Report, RA-Report, Successful Handover Report, etc). In all such cases, the UEInformationRequest/UEInformationResponse framework with SRB2 is used, i.e the gNB explicitly requests the UE to transmit one of these reports in the UEInformationRequest, and the UE then replies with the concerned report in the UEInformationResponse. The same framework based on SRB2 can be used for the on-demand report of the AIML collected data.

The UEInformationRequest/UEInformationResponse framework, based on SRB2 (as for the logged MDT), is used for the on-demand report of the NW-side collected data from the UE to the gNB.

Related to the periodic reporting, we believe that its benefit is questionable. The AIML-related collected data are not subject to any latency requirement, so their periodic transmission is not justified by any real-time type of decisions at the network, e.g. scheduling or mobility-related decision. The gNB is also always in control of when the data collection starts (at least in the case in which the UE starts the data collection upon receiving a configuration from the gNB) and what data the UE is collecting, so the gNB can always fetch the data at any point in time by leveraging on the mentioned UEInformationRequest/UEInformationResponse framework. On the other hand, for the case in which the UE starts the data collection upon fulfilling certain events, the UE can simply notify the network when data are available and the network can fetch them.
We also note that specifying both the on-demand fetching (via UEInformationRequest/Response framework) and the periodic reporting might require quite extensive specification work and UE impact. For the latter, since periodic reporting is used in the RRM framework, one could think to reuse the RRM framework, but then the UE would need to support two very different reporting mechanisms (one based on UEInformationRequest/UEInformationResponse, the other on RRM) for the reporting of the same type of data. Additionally, the RRM framework only support SRB1, and SRB1 cannot be certainly used for the transmission of AIML data. 
Unless this is justified, RAN2 should strive to specify just a single reporting framework.

The need for a periodic reporting is not well justified, given the lack of any latency requirement for the AIML-related collected data reporting.

For the sake of UE and specification implementation simplicity, RAN2 should strive to specify just a single reporting framework for the AIML collected data.

Periodic reporting of AIML-related collected data is not supported. Only on-demand framework (based on UEInformationRequest/UEInformationResponse) is supported.

Related to event-based reporting, it was discussed in RAN2#127 that if the UE stops the data collection due to UE memory becoming full, this is definitely an information that the network should know as soon as possible. In this way, the network can start retrieving the data in order to free up the UE memory as soon as possible, thereby avoiding that the UE needs to skip the logging of some radio measurements.

When this happens, the UE can just inform the network (e.g. via UAI) that data collection has stopped or that the remaining storage capacity has dropped below a certain level, and the gNB can simply retrieve the collected data by leveraging on the UEInformationRequest/UEInformationResponse framework. One could argue that the UE could instead transmit the full report, but as said above, this would require the need to support in the spec. and UE implementation a second reporting mechanism (besides the UEInformationRequest/UEInformationResponse framework). Additionally, since the UE/gNB may be engaged in other operations of higher priority (e.g. transmitting/serving higher priority traffics), and the AIML-related collected data are not subject to latency requirements, it is not necessary to promptly receive at the NW such reports.

When the UE stops the data collection (e.g. due to memory limitations) or when the storage capacity drops below a certain limit, it informs the gNB about the availability of collected data. The gNB may than fetch “on-demand” the available data (via UEInformationRequest-UEInformationResponse).

The data availability indication can be transmitted in the UEAssistanceInformation or in the RRC complete messages (as in logged MDT).

## 2.4	How to handle the case of collected data exceeding single RRC message size

In RAN2#127, it was agreed that no standardized RRC segmentation procedure is needed, as for the logged MDT measurements. The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded.
Then, with this agreement in place, RAN2 just need to discuss how to transmit the collected AIML data that could not fit into a single RRC message. Again, we can get some inspiration from the logged MDT and foresee the following options:

- In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response); or

- Once the gNB requests the UE to transmit the AIML collected data, the UE continues the transmission of a certain amount of samples (as indicated by the gNB in the UEInformationRequest) in the following RRC UEInformationResponse messages. In such case, it would be useful for the network to get to know how many data are stored/left for transmission so that the gNB can allocate the necessary resources and decide when to fetch those data and how many data (e.g. taking into account cell load).

- Once the gNB requests the UE to transmit the AIML collected data, the UE continues the transmission of all the remaining samples in following RRC UEInformationResponse messages without any further request from the gNB. Also in such case, it would be useful for the network to get to know how many data are stored/left for transmission so that the gNB can allocate the necessary resources and decide when/whether to fetch those data taking into account cell load.

In case all the collected data do not fit into a single RRC message, RAN2 to discuss the following options:

  - In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response) (similar to logged MDT).

  - The gNB indicates (i.e. in the UEInformationRequest) the amount of collected data the UE should transmit in the following RRC UEInformationResponse messages, or the amount of RRC UEInformationResponse messages the UE should transmit, without any further request from the gNB.

  - The gNB indicates (i.e. in the UEInformationRequest) that all the remaining collected data should be transmitted by the UE in the following RRC UEInformationResponse messages without any further request from the gNB.

As said above, in order to facilitate the gNB decisions of whether to fetch or postpone the fetching of the data, it would be beneficial if the UE could indicate as part of the availability indication, how many date the UE has collected or how many RRC messages are expected to be transmitted.

The UE indicates in the availability indication how many data or RRC samples are available for transmission.

## 2.5	UE memory capabilities

During the last RAN2 meeting, we have agreed that the UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. Before discussing the minimum value of the memory size, we propose waiting for more RAN1 inputs related to the data that the UE could log and report to the gNB, so that RAN2 can do a more thorough analysis of such minimum requirement. Additionally, in our view if the UE supports more memory than this minimum memory size, that should be indicated by the UE.

RAN2 to wait RAN1 inputs on the type of data to be logged by the UE, before discussing the minimum memory size for storing collected data.

Define in a UE capability some different minimum memory values that a UE can support for the logging of collected data.

# 3	Discussion on NW-side data collection for positioning

Network-side data collection for positioning occurs in the following network-based positioning methods:

- Case 3a: NG-RAN node assisted/LMF-based positioning with gNB side model (direct positioning)

- Case 3b: NG-RAN node assisted/LMF-based positioning with LMF-side model (assisted positioning)

Case 3a and Case 3b both are based on UL measurements performed at the gNB and share similar flows as the legacy UL-TDOA positioning method. As such the impact is primarily in RAN3 domain NRPPa.

The main impact in terms of RAN2 is the RRC configuration for SRS which can follow the legacy mechanism. However, one additional aspect to consider is that for AI/ML model both measurements and label is needed. For the training, the necessary information is the gNB measurements (RTOA) and the label/ground truth is UE location or true RTOA. The gNB would have to configure the reporting of UE location coinciding the SRS transmission periodicity. The gNB for the case 3a can also obtain the UE location using RRC; that is there would be no requirement of having an LPP session. This is already possible in legacy as below for MDT.

#### –	CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

CommonLocationInfo field descriptions

gnss-TOD-msec Parameter type gnss-TOD-msec defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

UE sending ground truth UE location coordinate to gNB via RRC is more efficient compared to sending it first to LMF which further forwarding to gNB, especially when UE is requested to send updated location coordinate when moving.

Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact. The main impact on RAN2 can be to align the gNB measurements and UE reporting of ground truth during training.

UE sending ground truth UE location coordinate directly to gNB via RRC is an efficient way to support gNB-side data collection.

A window to report UE ground truth coinciding with SRS transmission periodicity is configured.

UE sends ground truth UE location coordinate to gNB via RRC for gNB-side data   collection.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The need for a periodic reporting is not well justified, given the lack of any latency requirement for the AIML-related collected data reporting.

Observation 2	For the sake of UE and specification implementation simplicity, RAN2 should strive to specify just a single reporting framework for the AIML collected data.

Observation 3	Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact. The main impact on RAN2 can be to align the gNB measurements and UE reporting of ground truth during training.

Observation 4	UE sending ground truth UE location coordinate directly to gNB via RRC is an efficient way to support gNB-side data collection.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to assume the following for the case of beam management:

a.	The radio resource configuration for the purpose of NW-side data collection consists of a set of CSI-RS/SSB resources in which the UE shall perform radio measurements and optionally logging.

b.	The reporting of L1 measurement results includes, at least, L1-RSRP.

c.	If data logging is not supported by the UE, the L1 measurements results associated to the NW-side data collection are reported via UCI.

Proposal 2	Ask RAN1 confirmation of the above assumptions, and any further details on the content of the radio resource configuration and on the content of the report including the L1 measurement results.

Proposal 3	No need to introduce multiple configurations for NW-side data collection.

Proposal 4	No need to introduce dynamic activation/deactivation mechanism for NW-side data collection.

Proposal 5	RAN2 to consider the following approaches for the starting of the NW-side data collection procedure:

a.	The UE starts the data collection upon receiving a L3 configuration for NW-side data collection (as in logged MDT).

b.	The UE starts the data collection upon fulfilling certain events (as in logged MDT), e.g. based on L1 measurement threshold.

Proposal 6	Once the data collection is started as per Proposal 5, the logging interval is given by the specific CSI measurement configuration (e.g. periodic/aperiodic measurements).

Proposal 7	Besides the case of memory limitation issues (already agreed in RAN2#127), RAN2 to consider the following additional mechanisms for the stopping of the NW-side data collection procedure:

a.	The UE continues data collection for a certain configurable logging duration (as in logged MDT)

b.	The UE continues data collection until certain events are fulfilled (as in logged MDT), e.g. based on L1 measurement threshold.

Proposal 8	The UEInformationRequest/UEInformationResponse framework, based on SRB2 (as for the logged MDT), is used for the on-demand report of the NW-side collected data from the UE to the gNB.

Proposal 9	Periodic reporting of AIML-related collected data is not supported. Only on-demand framework (based on UEInformationRequest/UEInformationResponse) is supported.

Proposal 10	When the UE stops the data collection (e.g. due to memory limitations) or when the storage capacity drops below a certain limit, it informs the gNB about the availability of collected data. The gNB may than fetch “on-demand” the available data (via UEInformationRequest-UEInformationResponse).

Proposal 11	The data availability indication can be transmitted in the UEAssistanceInformation or in the RRC complete messages (as in logged MDT).

Proposal 12	In case all the collected data do not fit into a single RRC message, RAN2 to discuss the following options:

a.	In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response) (similar to logged MDT).

b.	The gNB indicates (i.e. in the UEInformationRequest) the amount of collected data the UE should transmit in the following RRC UEInformationResponse messages, or the amount of RRC UEInformationResponse messages the UE should transmit, without any further request from the gNB.

c.	The gNB indicates (i.e. in the UEInformationRequest) that all the remaining collected data should be transmitted by the UE in the following RRC UEInformationResponse messages without any further request from the gNB.

Proposal 13	The UE indicates in the availability indication how many data or RRC samples are available for transmission.

Proposal 14	RAN2 to wait RAN1 inputs on the type of data to be logged by the UE, before discussing the minimum memory size for storing collected data.

Proposal 15	Define in a UE capability some different minimum memory values that a UE can support for the logging of collected data.

Proposal 16	A window to report UE ground truth coinciding with SRS transmission periodicity is configured.

Proposal 17	UE sends ground truth UE location coordinate to gNB via RRC for gNB-side data   collection.