3GPP TSG RAN WG2 Meeting #127bis	R2-2409170

Hefei, China, October 14 – 18, 2024

Source:	Inmarsat, Viasat

Title:	Operator views on EDT enhancements and CRDSA

Agenda Item:	8.9.3

Document for:	Discussion and Decision

# Introduction

In RAN2#127, after a very lengthy discussion, the following agreements were reached in respect to the specification of a new EDT-like procedure to improve the capacity of UL transmissions for IoT NTN, in particular NB-IoT.

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

.

This paper aims at summarizing some key considerations from an operator’s perspective that we think must taken into account.

# Discussion

## Evaluation Assumptions

During RAN2#127, extensive discussion took place on the advantages of Diversity Slotted Aloha (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) over conventional Slotted Aloha with 2-step RA or 4-step RA.

A number of simulations were contributed by various companies. The majority of which clearly outlining the significant capacity gains achievable with DSA and CRDSA over conventional SA.

One contribution, R2-2407555 [4], however, presented a comparison between SA with 4-step RA and CRDSA, arguing that 4-step RA performs better in high system load conditions. The BLER/PLR assumption in this contribution was 10% (10^-1).

The same contribution also provides a very good explanation of how 4-step RA works, by relying, in case of collisions, on a back-off mechanism with incremental power, such that users are delayed from accessing the network and are forced to try again later.

The first thing that needs to be noted is that, whilst 10^-1 can appears to be a typical BLER/PLR assumption for terrestrial networks, it is not for satellite networks, where BLER/PLR targets of 10^-2 (1%) or 10^-3 (0.1%) are much more typical.  The typical target being 10^-3, with 10^-2 being a minimum target.

Observation 1: In satellite communications, an equivalent Transport Block Error Rate (BLER) of 10^-3 (0.1%) is considered the standard target, with 10^-2 (1%) being the minimum. For the purposes of this discussion BLER and Packet Loss Rate (PLR) are equivalent.

This is primarily due to the long latency, which reduces the utility of feedback-based mechanisms, as well as the tight link budget.

More specifically, even in current NB-IoT NTN live deployments in GEO supporting IoT and smartphones, handheld and wearables, the BLER/PLR target is typically not higher than 10^-2 (1%).

Observation 2: Current NB-IoT NTN GEO implementations in the field typically use a 10^-2 BLER (1%) target.

Proposal 1: Consider a Packet Loss Rate target of 1% or less, ideally 0.1% (10^-3 BLER).

The second aspect that should be taken into account is that “load” is a relative metric, not an absolute one.  It is rather more useful to consider absolute system capacity in terms of actually delivered information, as a metric for comparison.  If the absolute channel capacity in terms of delivered information increases for a given metric of time, it doesn’t matter if a system operates less efficiently at high load (near congestion), because the congestion threshold will be pushed further away, therefore the high load scenario is less likely to occur. Therefore, schemes that deliver useful information faster, with a higher probability, and with a lower overhead from re-transmissions, should be preferred.

Observation 3: “Load” is a relative metric, different from the actual useful information channel capacity.

Observation 4: As also pointed out in R2-2407555, at high MAC load, all schemes end up in a very high packet loss anyways due to a high collision rate, and other mechanisms have to be used, such as Access Class Barring (ACB) and other access control techniques.

Proposal 2: Consider absolute capacity in terms of delivered information bits/s as the comparison metric, instead of system load.

The time to deliver a message or to complete a transaction is also one of the most important key metrics affecting user experience and service quality, thus it cannot be ignored.  The whole transaction time should be taken into account, or we should look at channel capacity within a fixed unit of time to compare apples to apples.

Proposal 3: A fixed reference time interval shall be taken into account when comparing capacity of DSA/CRDSA with SA/4-step RA and other mechanisms, e.g one RTT or the total time of a complete 4-step EDT transaction (Msg1, Msg2 and Msg4) in ideal condition and 0% PLR.

## CRDSA and SIC

It has repeatedly been stated that CRDSA requires the implementation of Successive Interference Cancellation (SIC), in order to cancel out the interference contributions from other replicas after the first successful decoding.

Whilst this is correct, it should be noted that the interference cancellation scheme is something that is typically left to implementation.

In addition, there are a number of aspects and factors that could be leveraged to aid interference cancellation, that could make replica indexing unnecessary.

Firstly, as previously indicated to RAN1 by [5] and [6], in satellite deployments there are a number of factors that can result in a power imbalance between received transmissions from different UE at the SAN, such as different link conditions, different power class, different antenna, etc.

However, there would be a strong correlation/similarity between the received replica signals for replicas coming from the same UE.

Observation 5: In satellite deployments, there are many factors that can result in power imbalances between received transmissions, but a relatively close correlation/similarity between replicas from the same UE.

It is fairly well-known that differences in received signal power can be used to aid interference cancellation at the receiver.  Therefore, power imbalances between replicas from different UE compared to similarities in replicas from the same UE would actually help the SIC process and would remove the need for replica indexing.

Observation 6: Power imbalances between received transmissions can be used by the base station to aid interference cancellation.

Proposal 4: SIC and replica indexing mechanisms at the Network could be left to implementation, thus RAN1 impact may not be required.

In addition, as noted by some companies, the replica indexing could be potentially fulfilled by Orthogonal Cover Codes (OCC), by ensuring a known correlation between OCC sequences for each replica or for replicas belonging to the same UE.

Thus, from our perspective, the introduction of an additional explicit replica indexing mechanism is not a mandatory requirement and impact to RAN1 could be avoided, by leaving such aspects to implementation.

# Conclusions

Observation 1: In satellite communications, an equivalent Transport Block Error Rate (BLER) of 10^-3 (0.1%) is considered the standard target, with 10^-2 (1%) being the minimum. For the purposes of this discussion BLER and Packet Loss Rate (PLR) are equivalent.

Observation 2: Current NB-IoT NTN GEO implementations in the field typically use a 10^-2 BLER (1%) target.

Proposal 1: Consider a Packet Loss Rate target of 1% or less, ideally 0.1% (10^-3 BLER).

Observation 3: “Load” is a relative metric, different from the actual useful information channel capacity.

Observation 4: As also pointed out in R2-2407555, at high MAC load, all schemes end up in a very high packet loss anyways due to a high collision rate, and other mechanisms have to be used, such as Access Class Barring (ACB) and other access control techniques.

Proposal 2: Consider absolute capacity in terms of delivered information bits/s as the comparison metric, instead of system load.

Proposal 3: A fixed reference time interval shall be taken into account when comparing capacity of DSA/CRDSA with SA/4-step RA and other mechanisms, e.g one RTT or the total time of a complete 4-step EDT transaction (Msg1, Msg2 and Msg4) in ideal condition and 0% PLR.

Observation 5: In satellite deployments, there are many factors that can result in power imbalances between received transmissions, but a relatively close correlation/similarity between replicas from the same UE.

Observation 6: Power imbalances between received transmissions can be used by the base station to aid interference cancellation.

Proposal 4: SIC and replica indexing mechanisms at the Network could be left to implementation, thus RAN1 impact may not be required.

# Reference

- RP-242397 - Revised WID IoT_NTN_Ph3	Mediatek

- R2-2407573 - Report from Break-out session on NR-NTN and IoT-NTN

- R2-2407502 - Discussion on DSA and CRDSA Performance   ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot

- R2-2407555 - UL capacity enhancements objectives for IoT NTN   Ericsson

- R1-2407138 - Views on uplink capacity enhancement for IoT NTN

- R1-2407139 - Views on uplink capacity enhancement for NR NTN