3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409172

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.2.2

Source:		Futurewei

Title:		Discussions on AS ID for Ambient IoT devices

Document for:		Discussion and Decision

# Introduction

AS ID has been studied in RAN1 for the purpose of R2D reception (e.g., to identify the intended device to receive the R2D transmission) and for the purpose of D2R scheduling (e.g., to identify the intended device to make the D2R transmission). For D2R scheduling, there are also proposals of using the device’s ID to map to the time-frequency resource scheduled for the device to send its response [1].

During [POST127][033] e-mail discussion on AIoT random access in RAN2 [2], the following two candidate options for obtaining the AS scheduling ID, after the reader addresses the contention by Msg2 in CBRA case, have been identified, provided that the AS scheduling ID is to be supported:

- Option 1: the random ID in Msg1 can be reused as the AS scheduling ID;

- Option 2: reader assigns the AS scheduling ID, which is unique among the devices under service.

In this contribution, we provide more detailed views on AS ID for (AIoT) devices.

# Discussions

In the latest endorsed TP for TR 38.769 [3], the overall procedures between the device and the reader are illustrated as in Figure 1 below (same as Figure 6.2.1-1 in [3]).

Figure 1 Overall AS procedures between A-IoT device and reader

## For the “inventory-only” use case

According to [3]:

For the detailed use case of “inventory-only”, it is supported by the procedure with step A and step B as baseline.

In step A, based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. RAN2 has agreed to study the following cases:

-	The A-IoT paging message containing an identifier of a single A-IoT device

-	The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

-	The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond

-	The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [SA2 TR 23.700-13].

Since the need for the last case (the fourth bullet above) is pending on SA2 study, the “multiple identifiers” in the fourth bullet should be IDs from the NAS layer or above. The case in the third bullet above requires no ID in the A-IoT paging message. It may be debatable whether the ID of the single device in the case of the first bullet above and the group ID in the case of the second bullet above are IDs from the NAS layer or above, or they can also be AS-assigned ID. However, we prefer that, at least for the “inventory-only” use case, the ID of the single device in the case of the first bullet above and the group ID in the case of the second bullet above do not use AS-assigned ID.

Proposal 1. For the “inventory-only” use case, AS ID is not used in the A-IoT paging message, as baseline.

In step B, the device begins its CBRA by sending Msg1 with a random ID generated by the device in it and the reader sends back Msg2 echoing the random ID of the device if the device wins the contention. Therefore, the random ID echoed in Msg2 can already identify the intended device for receiving Msg2 and for subsequently transmitting Msg3. The Device ID (e.g., a CN-assigned temporary ID or a more permanent ID such as EPC) that the device includes in its Msg3 is sufficient to identify the device in the NAS layer and above. If Msg4 is needed for acknowledging the reception of Msg3, the random ID of the device can still be used for identifying the intended device for receiving Msg4. Therefore, for the “inventory-only” use case, there is no need for using AS scheduling ID in step B.

After being successfully inventoried for the current inventory service session, the device can stop using its current random ID and can later generate a new one to use when a need for responding to another inventory service session arises. This can be applicable to both the case due to multiple separate inventory service requests and the case due to a single inventory service request of periodical inventorying. And for the “inventory-only” use case, because there are no further data communications with the device after Msg3/Msg4, there is no need to assign an AS ID to the device.

Proposal 2: For the “inventory-only” use case, AS scheduling ID is not used in step B, as baseline.

If the device needs to further perform a NAS procedure, such as authentication and authorization, security key management, etc., after it is successfully inventoried, e.g., for the first time at a network, the case can be modeled as an “inventory-and-command” use case if the NAS procedure can be triggered soon after the device is successfully inventoried (so that the time gap between step B and step C can be considered small); otherwise, if the NAS procedure is initiated with a significant time gap after the device is successfully inventoried, the AS procedure for the NAS procedure can be modelled as a “command-only” use case after the prior “inventory-only” use case. Both cases are discussed subsequently.

Observation 1: As far as the AS scheduling ID is concerned, if the device needs to further perform a NAS procedure immediately after it is successfully inventoried, the AS procedure can be modeled in the same way as an “inventory-and-command” use case; otherwise, if the NAS procedure is initiated with a significant time gap after the device is successfully inventoried, then the AS procedure for the NAS procedure can be modelled in the same was as a “command-only” use case after the prior AS procedure for the “inventory-only” service transaction.

## For the “inventory-and-command” use case

According to [3]:

For the detailed use case of “inventory and command”, it is supported by the procedure with step A, step B, step C1 and step C2, as baseline.

For this use case, we continue to prefer that the AS ID is not used in step A (the A-IoT paging message) or step B (Msg1, Msg2, and Msg3). However, we understand that there are proposals of the reader including AS ID assigned to the device in Msg2, which can be a topic for further discussions.

Proposal 3: For the “inventory-and-command” use case, AS scheduling ID is not used in step A nor step B, as baseline.

Although RAN2 has not discussed nor made any agreement on how long a device can hold onto its random ID after sending its Msg3, there is a general feeling that the longer the device keeps using a particular random ID, the higher the probability that the same random ID may be selected by another device and hence the higher probability of ID collision. We think the random ID of the device (as sent in Msg1 and echoed in Msg2) can be used as the AS scheduling ID of the device in step C until step C is finished, assuming no significant time gap between step B and step C. However, we see no need for any additional signaling for assigning the random ID to the device as its AS scheduling ID, because Msg2 itself already validates the random ID for representing the device for a short time period where there is no chance for another device to be validated for using the same random ID, e.g., from scheduling for Msg3 by the current Msg2 in step B until step C is finished.

When step C is finished, we think the device should stop using its random ID, and later when a need for responding to another “inventory-and-command” service request arises, the device should generate a new random ID to use.

If step C is somehow interrupted, e.g., due to a transmission failure, the device may try to re-access, for which the device may generate a new random ID for sending another Msg1 to begin with. Once the new random ID is echoed in Msg2, the new random ID is validated for representing the device subsequently in step C during the re-try.

Proposal 4: For the “inventory-and-command” use case, the random ID of the device, as echoed in the latest Msg2, is used as its scheduling ID in step C until step C is finished. There is no need for additional signaling for such ID assignment.

## For the “command-only” use case

According to [3]:

- For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with step A, step B, step C1 and step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [SA2 TR 23.700-13] and [SA3 TR 33.713]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g. the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

If a service transaction of “command-only” is supported by using the first approach (i.e., using steps A/B/C1/C2), we also assume no significant time gap between step B and step C, which may otherwise allow another device to gain a use of the same random ID of the device before the device is able to finish the AS procedures for the current “command-only” service transaction. In that case, the previous Proposals 3 and 4 are also applicable.

Proposal 5. P3 and P4 above are also applicable to the “command-only” use case, if a service transaction of “command-only” is supported by using steps A/B/C1/C2 and there is no significant time gap between step B and step C.

On the other hand, if a service transaction of “command-only” is supported by using the second approach (i.e., using step A’ and step C2 only), it may be debatable whether it is beneficial to assign an AS scheduling ID to the device, e.g., during a prior inventory-and-command transaction with the reader if knowing that the command is a periodical command or that the command can be followed up by additional commands, so that the AS scheduling ID (instead of a longer Device ID) can be used for identifying the device as the intended device in step A’ and for making a D2R transmission in step C2, later during a “command-only” service transaction.

If RAN2 decides to support AS scheduling ID for this case, then we would prefer either that the AS scheduling ID is not the random ID of the device, or if the AS scheduling ID is the random ID of the device, that the random ID of the device is not used by the device for random selection of access resource during CBRA. For the latter case, the device should generate another random number for the purpose of random selection of access resource during CBRA, like in RFID. The reason for not using the random ID of the device for random selection of access resource during CBRA, such as being proposal in [1], is that once the random ID of the device is fixed for a long duration (e.g., longer than the duration of a single round of steps A-to-C, which would be the case if keeping a random ID of a device as its AS scheduling ID for the follow-up commands) and is used for random selection of access resource, then the resource selection becomes more deterministic rather than random for the duration, e.g., a device with a certain random ID always has advantage over another device with another random ID for the duration. Such bias may encourage cheating behaviors in random ID generation. Using an analogy of airline tickets, we always use our true names to identify ourselves when purchasing airline tickets. Meanwhile, our seat assignments may be different (and may appear random if we are unable to select seats) for each flight. However, if the airline’s policy of seat assignment for all its flights is based on passenger’s name, then some passengers may cheat on their names to get a favorable seat assignment. Therefore, the purpose of identification and the purpose of random selection of access resource (which is similar to seat assignment) should be met using different inputs, not both based on the random ID.

During [POST127][033] e-mail discussion on AIoT random access, more companies are in favor of using the random ID of the device as its AS scheduling ID. Although we are not in favor of it, we recognized that there may be signaling overhead saving for AS ID assignment if the random ID of the device is directly converted into its AS scheduling ID. Therefore, we propose the following:

Proposal 6. If RAN2 decides to support AS scheduling ID for “command-only” case and the AS scheduling ID is the random ID of the device, then the random ID of the device is not to be used by the device for random selection of access resource during CBRA. The device should generate another random number for the purpose of random selection of access resource during CBRA.

# Conclusions

We propose the following:

Proposal 1. For the “inventory-only” use case, AS ID is not used in the A-IoT paging message, as baseline.

Proposal 2: For the “inventory-only” use case, AS scheduling ID is not used in step B, as baseline.

Proposal 3: For the “inventory-and-command” use case, AS scheduling ID is not used in step A nor step B, as baseline.

Proposal 4: For the “inventory-and-command” use case, the random ID of the device, as echoed in the latest Msg2, is used as its scheduling ID in step C until step C is finished. There is no need for additional signaling for such ID assignment.

Proposal 5. P3 and P4 above are also applicable to the “command-only” use case, if a service transaction of “command-only” is supported by using steps A/B/C1/C2 and there is no significant time gap between step B and step C.

Proposal 6. If RAN2 decides to support AS scheduling ID for “command-only” case and the AS scheduling ID is the random ID of the device, then the random ID of the device is not to be used by the device for random selection of access resource during CBRA. The device should generate another random number for the purpose of random selection of access resource during CBRA.

# References

R2-2406710, Paging Related Aspects for Ambient IOT, InterDigital.

R2-2407985, Report of [POST127][033][AIoT] Random Access, Huawei (Rapporteur).

R2-2406271, TP for TR 38.769 Update, Huawei (Rapporteur).