3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409173

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.2.4

Source:		Futurewei

Title:		Further discussions on Ambient IoT random access

Document for:		Discussion and Decision

# Introduction

During [POST127][022] e-mail discussion on TP for TR 38.769 Update [1], comments were raised questioning the use of explicit R2D signals, which were originally represented by arrows, to mark the starting point of access occasion(s), citing a pending decision in RAN1 on timing-based determination of the starting point of access occasion(s). As a result, all arrows representing an R2D message, including the A-IoT paging message, are removed from the final figure describing the general framework, as Figure 1 below (same as Figure 6.3.4-1 in [1]).

Figure 1. General framework of slotted-ALOHA for A-IoT random access procedure

In this contribution, we provide our views on this and some other issues related to random access for Ambient IoT.

# Discussions

## On the definition of access occasion and access occasion set

To elaborate our views, we first need to clarify some definitions. First, we agree on the following definition of an access occasion:

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device).

Due to the discussion of potentially using one R2D message (such as A-IoT paging message or other triggering message) to schedule multiple access occasions in a TDM and/or FDM manner, we also agree on a need for providing a definition of access occasion set as the following:

Access occasion set: A set of access occasions scheduled by the reader via a same R2D message and for different devices to perform access, respectively.

Figure 2 below, which is cut-and-pasted from Figure 1, would represent one access occasion set, with each little box in it representing a unique access occasion and the “…” representing additional columns of access occasions in time (while the vertical direction representing frequency) until a time when the remaining (yet to be scheduled) devices are expected to have their clocks drifted too far to be scheduled together in the same access occasion set. That is where the current access occasion set stops and another R2D message (or signal) is needed to schedule (or at least to mark the beginning of) the next access occasion set. Therefore, the access occasion set depicted in Figure 2 corresponds to X number of D2R transmissions contiguously in time and Y number of D2R transmissions in frequency, possibly being interrupted only by guard time and guard frequency in-between but without any R2D transmission interrupting them in-between. This description is aligned with RAN1’s pending study on the TDM/FDM aspect. We suspect that companies that had objected to the arrows originally in Figure 1 may have overlooked or had different understanding of the “…” in Figure 2, and as a result, they had concluded that multiple instances of the resource unit depicted in Figure 2 are needed to represent a X by Y number of access occasions.

Figure 2. Illustration of one access occasion set

With the above explanation of our understanding on Figure 1 and the definition of an access occasion in it, and our understanding on the definition of an access occasion set and what Figure 2 represents for, we believe R2D message is needed between consecutive access occasion sets, e.g., to schedule access resources for the new access occasion set triggered by the R2D message, to mark the beginning of the new access occasion set, and to possibly provide assistance to the remaining (unscheduled) devices to correct their clocks after missing a reliable reference clock throughout the duration that corresponds to the previous access occasion set.

Proposal 1. RAN2 confirms the following definitions and includes them in the next revision of TR 38.769:

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device).

Access occasion set: A set of access occasions scheduled by the reader via a same R2D message and for different devices to perform access, respectively.

If P2 is agreed, then each access occasion set is led by an R2D message (or at least an R2D signal) by definition.

Proposal 2. Include arrows in Figure 6.3.4-1 to represent an R2D message/signal triggering each access occasion set.

In addition, we believe that a device that decides to send its response within a particular access occasion set should make no more than one D2R transmissions within that access occasion set. Allowing a device to make more than one D2R transmissions within a same access occasion set, whether it is making different D2R transmissions or blindly (for there is no chance of getting R2D feedback in the middle of an access occasion set) repeating the same D2R transmission would be unfair and unsound. Hence, any attempts of re-access made by a device should be made at least in a different access occasion set.

Proposal 3. A-IoT devices are prohibited from making more than one D2R transmissions within a same access occasion set.

Proposal 4. Any attempts of re-access made by a device should be made at least in a different access occasion set.

## On what is in the R2D message that triggers an access occasion set

Due to the clock drifting issues at the A-IoT devices, there is a limit as to how far an access occasion set can span in time. Therefore, when the number of devices that satisfy the criteria to respond to an A-IoT paging message is too large, these devices need to be partitioned into multiple groups so that each group of devices can be served sequentially by a different access occasion set. Such partitioning does not have to be fixed and can be changed dynamically, e.g., a device failed in an earlier access occasion set may automatically qualify for a later access occasion set, provided that the failure has been indicated to the device or detected by the device reliably. Therefore, the R2D message that triggers an access occasion set may need to include information indicating which devices are to participate in using an access occasion with the current access occasion set.

Proposal 5. The R2D message that triggers an access occasion set may include information indicating criteria that determines which devices are to participate in using an access occasion within the current access occasion set. Devices satisfying the criteria indicated in the prior A-IoT paging message but not satisfying the criteria indicated in the R2D message cannot access the access occasion set triggered by the R2D message.

In RAN2#127, the following was agreed [2]:

Figure 1 above includes one round of A-IoT paging and another round of subsequent A-IoT paging. It is unclear whether the subsequent A-IoT paging is due to a different A-IoT service request or is a duplicated or a delta version of an A-IoT paging message associated with the same A-IoT service request. Some clarifications may be needed.

Proposal 6. RAN2 to discuss whether the subsequent A-IoT paging, as depicted in Figure 6.3.4-1 in [1], is due to a different A-IoT service request or is a duplicated or a delta version of an A-IoT paging message associated with the same A-IoT service request, and based on a result, to revise Figure 6.3.4-1 if needed.

## On how Msg2 is sent when multiple Msg1s are successfully received within a same access occasion set

While studying TDM/FDM for Ambient IoT has been very appealing in both RAN1 and RAN2 from latency reduction point of view, some aspect that may have not been discussion before is how Msg2 should be sent when multiple Msg1s are successfully received within a same access occasion set. There can be two basic approaches: one is that the reader sends multiple separately encoded R2D transmissions, each carrying a different Msg2. The other is that the reader sends a single R2D transmission, which carries a MAC payload consisting of multiple Msg2s.

The first approach may have an issue with the timing aspect. For example, when a device successfully decodes the Msg2 with its own random ID in it, the device may stop receiving the rest of the Msg2s. Then, the device may not know when the end of the last R2D transmission is to start counting down TR2D_max for failure detection.

On the other hand, the second approach may have an issue with the increased processing load for the device (and hence more energy being consumed and possibly added complexity to the devices), because the device needs to receive and process the entire MAC payload to check the CRC bits, even though there is only one random ID that the device needs to match with.

Proposal 7. RAN2 to discuss how Msg2(s) is/are sent when multiple Msg1s are successfully received within a same access occasion set, with the following two options to begin with:

- Option 1: the reader sends multiple separately encoded R2D transmissions, each carrying a different Msg2.

- Option 2: the reader sends a single R2D transmission, which carries a MAC payload consisting of multiple Msg2s.

## On 2-step CBRA

For “inventory-only” and “inventory-and-command” use cases, Table 1 below shows the TB sizes of each D2R transmission for Msg1 and Msg3, and the sum of both, if applicable for 3-step CBRA and 2-step CBRA. If we assume that the energy consumed by the device for receiving the R2D transmission of Msg2 is negligible comparing to the energy consumed for making a D2R transmission. Then, the difference between the total TBS transmitted is only one preamble plus 6 CRC bits between 3-step CBRA and 2-step CBRA, which is only a small fraction considering that the Msg3 payload can include the 96-bit EPC. However, if 2-step CBRA is introduced and used, we would have to consider the consequence of an increased probability of collision and a need for the reader to indicate the RA type to the devices.

Table 1.

|  | TBS of D2R Tx for Msg1 | TBS of D2R Tx for Msg3 | Total TBS of D2R Tx |
| --- | --- | --- | --- |
| 3-step CBRA | Preamble + 22 bits (16 bits RNID + 6-bit CRC) | Preamble + Msg3_size + 16 bits (CRC) | 2x preamble + Msg3_size + 38 bits |
| 2-step CBRA | Preamble + Msg3_size + 32 bits (16 bits RNID + 16-bit CRC) | N/A | Preamble + Msg3_size + 32 bits |

Therefore, we propose that 2-step CBRA be deprioritized.

Proposal 8. 2-step CBRA is deprioritized.

# Conclusions

We propose the following:

Proposal 1. RAN2 confirms the following definitions and includes them in the next revision of TR 38.769:

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device).

Access occasion set: A set of access occasions scheduled by the reader via a same R2D message and for different devices to perform access, respectively.

Proposal 2. Include arrows in Figure 6.3.4-1 to represent an R2D message/signal triggering each access occasion set.

Proposal 3. A-IoT devices are prohibited from making more than one D2R transmissions within a same access occasion set.

Proposal 4. Any attempts of re-access made by a device should be made at least in a different access occasion set.

Proposal 5. The R2D message that triggers an access occasion set may include information indicating criteria that determines which devices are to participate in using an access occasion within the current access occasion set. Devices satisfying the criteria indicated in the prior A-IoT paging message but not satisfying the criteria indicated in the R2D message cannot access the access occasion set triggered by the R2D message.

Proposal 6. RAN2 to discuss whether the subsequent A-IoT paging, as depicted in Figure 6.3.4-1 in [1], is due to a different A-IoT service request or is a duplicated or a delta version of an A-IoT paging message associated with the same A-IoT service request, and based on a result, to revise Figure 6.3.4-1 if needed.

Proposal 7. RAN2 to discuss how Msg2(s) is/are sent when multiple Msg1s are successfully received within a same access occasion set, with the following two options to begin with:

- Option 1: the reader sends multiple separately encoded R2D transmissions, each carrying a different Msg2.

- Option 2: the reader sends a single R2D transmission, which carries a MAC payload consisting of multiple Msg2s.

Proposal 8. 2-step CBRA is deprioritized.

# References

R2-2407984, TP for TR 38.769 update, Huawei (Rapporteur).

Draft_RAN2_127_Meeting_Report_v2.