3GPP TSG-RAN WG2 Meeting # 127bis	 R2-2409177

# Hefei, China,  October 14th – 18th , 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0527 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to posSIB segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LMF may perform two different types of segmentation as below: - Pseudo segmentaion: Each segment can be independently decodable. - Octet Segmentation: UE needs to assmeble each segment before decoding. gNB needs to map each SIB segment into an SI message. In order to be able to efficeintly map each SIB segment into the SI message, it is essential that each SIB segments are of same size rather than variable size. Since positionign SIBs are segmented outside gNB, gNB does not have the control of the segments size. The split performed by LMF may not be of equal size. Further, it may so happen that with pseudo segmentation one segment is of 1000bits whereas another is half the size 500 bits; for an example one of the segments has 4 RTK stations and 2 SV-ID correction info whereas another segment is of 500bits as only 2 RTK stations and one SV-ID correction info were included. This variable size segmentation would lead to some inefficieny in SI-scheduling as the gNB has to reserve resources and mapping of the SIB to SI considering the size and in variable size scenario it has to consider then the maximum L1 size. That is, if the SIBs are segmented in variable size, it would waste the resource as it blocks mapping of the some other SIB type into the same SI message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that each segments are consistent in terms of size. Impact Analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: SI-Scheduling Resources Inter-operability: - If NW implements as per the CR and UE does not - UE should be prepared to decode posSIB of variable size - If UE implements as per the CR and NW does not - There may be resource wastage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of inefficenent radio resouce utilization. Inefficent SIB to SI mapping and SI-Scheduling |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

## 7.1	General

Broadcast of positioning assistance data is supported via Positioning System Information Blocks (posSIBs) as specified in TS 36.331 [12] or TS 38.331 [35]. The posSIBs are carried in RRC System Information (SI) messages (TS 36.331 [12] or TS 38.331 [35]).

For LTE RRC System Information (SI), a single SystemInformationBlockPos IE is defined in TS 36.331 [12] which is carried in IE PosSystemInformation-r15-IEs specified in TS 36.331 [12]. The mapping of positioning SIB type (posSibType) to assistance data carried in SystemInformationBlockPos is specified in clause 7.2.

For NR RRC System Information (SI), a single SIBpos IE is defined in TS 38.331 [35] which is carried in IE PosSystemInformation-r16-IEs specified in TS 38.331 [35]. The mapping of positioning SIB type (posSibType) to assistance data carried in SIBpos is specified in clause 7.2.

posSIBs may be segmented because of physical layer size limitations as described in clause 5.2.1 of TS 38.331 [35]. Two different types of segmentation scheme may be applied where one of them is pseudo-seg where each segment is independently decodable, and the other is octet-string-seg where all the segments need to be concatenated before decoding. The LMF ensures the size of posSIB segments while using any of the segmentation types are in equal or almost equal size.

End of Changes