3GPP TSG-RAN WG2 #127bis	R2-2409179

Hefei, China, 14 - 18 October 2024

Agenda Item:	8.8.5

Source:	Ericsson

Title:	Regenerative payload

Document for:	Discussion, Decision

# 1	Introduction

The work item [1] has the objective:

- Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

And RAN2#126 the following agreements were made:

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

And in RAN2#127

Agreements:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

In this contribution, we discuss regenerative payload and possible stage 2 updates.

# 2	Discussion

RAN3 have endorsed a baseline CR [2] for stage 2.

Summary of RAN3 changes to stage 2:

In 3.2 Definitions we have only changes to “NTN payload” definition.

RAN3 adds a regenerative figure in 16.14.1 and clarifies the existing figure is for transparent payload.

RAN3 did not touch 16.14.2.1, it may be beneficial if RAN2 clarifies that the figure is for transparent architecture.

The timing and synchronization in 38.300 section 16.14.2.1 is out of RAN3’s scope.

RAN3 made modifications to feeder link switch over in 16.14.4 and cell identities in 16.14.5, and they indicate that they will update 16.14.6 but have not concluded on the updates there yet.

Then for Appendix B.4, RAN3 clarified that the figure is an example of transparent payload.

RAN3 has endorsed a baseline CR for 38.300 that includes almost all changes needed.

Clarify that the 38.300 figure in 16.14.2.1 is for transparent payload.

Consider the text proposal in section 4.

There are many possible configurations of common TA and Kmac in Rel-17, some of these can be used with regenerative payload. There is no need to capture potential network implementation in the specification.

Specific configurations of common TA and Kmac in regenerative architecture are not captured in Stage 2.

# 4	Stage 2 Text proposal

Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN Gateway with transparent payload)

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	The timing and synchronization in 38.300 section 16.14.2.1 is out of RAN3’s scope.

Observation 2	RAN3 has endorsed a baseline CR for 38.300 that includes almost all changes needed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify that the 38.300 figure in 16.14.2.1 is for transparent payload.

Proposal 2	Consider the text proposal in section 4.

Proposal 3	Specific configurations of common TA and Kmac in regenerative architecture are not captured in Stage 2.

# 6	References

RP-241789, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#105, Sep. 2024.

R3-243949, “Support for Regenerative Payload in NR NTN”, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, RAN3#125, Aug. 2024.