3GPP TSG-RAN WG2 Meeting #127bis	R2-2409184

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

In previous meetings, RAN2 reached several agreements on the shape/format of the intended service area as well as some of the related UE behaviours when located inside and outside the intended service area. However, details on signalling to indicate the intended service area attending power efficiency and overhead are still open. This contribution presents our views on the end-to-end solution for intended service area for MBS broadcast and PWS in NTN cells.

# 2	Support for MBS broadcast

### 2.1	Problems with SIB as a solution

The WID objective clearly indicates to specify SIB signalling to indicate the intended service area of a broadcast service. However, in RAN2#127, several companies raised concerns about the suitability of SIB to carry this information and suggested that the MBSBroadcastConfiguration provided via MCCH could be a more efficient alternative. During the meeting, energy saving and preventing any possible missing service notifications were highlighted as they key decision criteria.

In light of the stalemate in discussions, this section aims to illuminate the drawbacks of using SIB for this purpose. Specifically, we consider that any changes to either the intended service area or the MBS flows associated with the intended service area (e.g., addition of a new MBS broadcast service) will require a system information (SI) update, which leads to the following problems:

- Slower modification period. SI updates are bound to a certain modification period, which is common for all SIBs. This is usually less flexible and dynamic than MCCH modification period and, from the network perspective, it may not be possible nor desirable to configure SIB’s modification period specifically for the use of MBS.

- Impact on non-MBS UEs. All UEs in the cell are notified about SI updates either through Short Messages or through the change of valueTag in SIB1, regardless of their interest in an MBS service. This underscores the relevance of adhering to legacy design which includes most of MBS related information in MCCH rather than SIB.

- Increased energy consumption. Legacy MBS already includes optimizations to minimize the impact of updates whenever they are not relevant for the UE. For example, MCCH includes a 2-bitmap to transmit update notifications (one for “session start” and another for “change”). This enables the UE to save some energy by only acquiring the MCCH message on PDSCH whenever needed. These mechanisms are, unfortunately, not present in the SI update procedure.

- Inefficient signalling: certain changes would lead to notify the UE twice, causing unnecessary overhead. For example, provided the intended service area must be associated with an MBS session (RAN2#126 agreement), the starting of a new MBS service would require: a SIB update to reflect its mapping to the intended service area, and an MCCH update to convey the MBS related details.

- Providing the intended service area in SIB has several drawbacks: slow modification period, impacts on non-MBS UEs, increased energy consumption, and inefficient signalling.

Given the decision criteria of energy consumption and effective notification of updates, we believe SIB is a suboptimal option to transmit the MBS intended service area in NTN.

The intended service area of an MBS broadcast service is not provided in system information. FFS on other broadcast services (e.g., ETWS).

In RAN2#127, the other alternative discussed was to include the intended service area in the MBSBroadcastConfiguration provided via MCCH. While this solution can help avoid some drawbacks of the SIB approach (e.g. inefficient signalling, slower modification period, and impact to non-MBS UEs), there were a few companies that expressed concerns about energy consumption. In their view, this solution would imply that a UE needs to constantly monitor MCCH and argued for mechanisms for MCCH acquisition and updates relaxation, i.e., a UE can avoid decoding MCCH if the UE is not located in the intended service area. We understand these concerns and introduce in the sections a new approach, which was already informally discussed among companies in RAN2#125bis and can fulfil energy consumption and efficient notification criteria with minimal specification impact.

### 2.2	Intended Service Area through MBS Service Announcement

#### 2.2.1	Background

The Service Announcement provides essential information that enables users to discover available multicast and broadcast services. This includes identifying what services are available (identified with their TMGI), the type of content being offered, and how to access these services. For instance, it may include the MBS Service Area, in the format of Cell ID list, TAI list, geographical area information or civic address information, or the MBS FSAI and optional frequency information.

- The Service Announcement contains the MBS Service Area.

- MBS Service Area can be in the form of Cell ID list, TAI list, geographical area information or civic address information.

According to TS 23.247, which defines architectural aspects of MBS, to receive the data of broadcast communication service, the UE is either:

- Preconfigured with the needed configuration for the UE to receive MBS service (in this case the UE does not need interact with NW).

- Provisioned with the configuration of a Broadcast MBS session on the application level via the Service Announcement.

The Service Announcement may be provided to a UE by AF (Application Function) or MBS function (MBSF) or may be retrieved by the UE from those entities. The Service Announcement may be delivered to the UE via application specific means or as defined in TS 26.502, using one of the following methods:

  - Pre-configured at the UE side;

  - Via an MBS Session; or

  - Via a regular PDU Session.

Figure 1 illustrates different phases of broadcast data provisioning and Figure 2 depicts a more detailed signalling flow for MBS broadcast session establishment. It can be seen from Figure 2 that the Service announcement is delivered to the UE before the MCCH. In our understanding, the UE becomes first acquainted with the available MBS services (e.g., content, identifiers, start and stop times) through the Service Announcement. Once it has this information, based on user input, the UE may become interested in an MBS service and trigger RAN procedures such as MCCH acquisition or MRB establishment.

Figure 1: [Figure 4.2.2-1, TS 23.247 V18.6.0] Phases of Broadcast data provisioning

Figure 2: Broadcast session establishment flow (adapted from Figure 7.3.1-1 TS 23.247 V18.6.0)

In conclusion, the Service Announcement plays a pivotal role in the MBS architecture. We understand that a UE interested in MBS must first acquire Service Announcement to become aware of the existing MBS services and their respective characteristics (e.g., start time). This may happen even before attempting to decode the relevant SIBs and the MBS configuration provided in MCCH. In fact, the current RRC specification allows the UE to skip MCCH monitoring based on USD/Service Announcement:

[NOTE in TS 38.331 V18.3.0 - Section 5.9.2.2]

“It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.”

On a related note, Stage-2 specification allows the UE to decide when to start monitoring the MCCH based on the information signalled in the Service announcement:

[NOTE in TS 38.300 V18.2.0 - Section 16.10.6.2]

It is up to UE implementation to use the start and stop times in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

- Service Announcement is essential for MBS. A UE interested in an MBS service must have acquired the Service Announcement to become interested in an MBS service and before attempting to decode MCCH.

- UE interested in MBS services can use start and stop times and/or the cell/tracking area list in the Service Announcement to avoid acquiring the MCCH when the MBS session is not ongoing and/or the UE is outside the MBS service area of the MBS broadcast service.

In addition, it’s worth noting that in the case of the second delivery option, the Service Announcement being provided to the UE via an MBS session, the network can set up a special and always running MBS session for this purpose. Therefore, a UE in RRC_IDLE/INACTIVE can use this special MBS session and avoid transitioning to RRC_CONNECTED to obtain Service Announcement updates.

#### 2.2.2	Way forward

The preceding section showed the advantages of the Service Announcement in terms of signalling efficiency, up-to-date notifications, and energy consumption. Indeed, this legacy mechanism can easily be extended to provide the Intended Service Area in an NTN cell. This approach is beneficial for both the UE and network. On the one hand, it allows the UE to decode MCCH whenever it is strictly needed (e.g., the UE is located inside the intended service area of a certain MBS service) helping to reduce battery consumption. Even for the rare case of changes to the intended service area, the UE can continue with its legacy behaviour and re-acquire the Service Announcement every now and then, up to application implementation. On the other hand, the network can spare unnecessarily updates and overhead in system information broadcast that may also affect non-MBS UEs. Provided that this solution reuses existing functionality, we believe it can ensure timely completion of the Work Item and speed up the deployment in real networks.

- Extending Service Announcement to include the Intended Service Area for NTN cells is a simple solution that has minimal specification impact and fulfils the energy consumption and signalling efficiency criteria.

MBS Service Announcement is used to indicate the intended service area for MBS broadcast in NTN cells.

#### 2.2.3	Specification impact

To facilitate the discussion about this new scheme, we have identified the following specification impact:

  - SA2 specifications need to be updated to state that the MBS service area in the form of geographical area information is to be delivered to the UE in the Service Announcement. An LS should be sent to SA2 to inform about RAN2’s conclusion.

  - The note for MCCH monitoring relaxation in RRC specification has to be extended to include the case when the UE is not located within the intended service area.

### 2.3	Service continuity

#### 2.3.1 Service continuity for 5G MBS broadcast

Principles of service continuity for MBS broadcast are specified in TS 38.300 V18.2.0, Section 16.10.6.5. For UEs in RRC_IDLE or RRC_INACTIVE is ensured through a combination of the following mechanisms:

- MCCH indicates a list of neighbour cells providing the same MBS broadcast services as provided in the serving cell.

- Information about which frequency is providing which MBS broadcast services via point-to-multipoint (PTM) transmission is obtainable through User Service Description (USD) or through a combination of USD and SIB21. Specifically, SIB21 contains MBS Frequency Selection Area ID (FSAI) which identifies a preconfigured area within, and in proximity to, which the cell(s) announces the FSAI and the associating frequency.

The first mechanism allows a UE to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using point-to-multipoint (PTM) transmission. The second mechanism helps the UE avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies. The UE can figure out the TMGI-Frequency mapping from USD or from the combination of USD and SIB21 and is allowed to make the frequency highest priority in cell reselection.

Additionally, a UE in RRC_CONNECTED can send MBS Interest Indication (MII) to the gNB to indicate the UE’s interest in certain MBS broadcast services.

#### 2.3.2	Service continuity for NTN MBS broadcast

In our view, the same mechanisms for service continuity in 5G MBS broadcast can be applied to NTN. That means, the list of neighbour cells providing the same MBS broadcast services in MCCH, the FSAI in SIB21, and the TMGI-frequency mapping or TMG-SAI mapping in the USD (which is delivered to the UE via Service announcement) can be reused.

Legacy mechanisms for service continuity in 5G MBS Broadcast should be reused for MBS broadcast with NTN.

One aspect where NTN differs from terrestrial networks (TN) is that the time duration that an NGSO satellite serves a quasi-earth-fixed cell is only a couple of minutes. Since each new serving satellite typically implies a new cell, to facilitate service continuity, an MBS UE in RRC_IDLE or RRC_INACTIVE needs to acquire the list of neighbour cells that providing the same MBS services (carried in MCCH) very often. This can be a challenge since there is a risk the UE does not obtain the MCCH in time. Therefore, we suggest RAN2 to discuss this issue and find potential solutions.

RAN2 focuses on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement.

### 2.4	Public warning systems in NTN

The scope of the work item to include a service area smaller than a cell in NTN is not limited to MBS but also encompasses other broadcast services such as Public Warning Systems (PWS). The two most prominent PWS are ETWS and CMAS. A flexible geo-fencing mechanism is already supported in CMAS, which can be repurposed for this NTN enhancement without necessitating further specification changes. During RAN2#127, the feasibility of applying this solution to ETWS was discussed. RAN2 concluded to send an LS to SA1 to inquire about the requirements and clarify whether such capability can be extended to ETWS.

Although the LS response from SA1 is not expected before RAN2#129, two recent developments suggest that further progress can be made within RAN2. Firstly, a new study item [3] was approved in SA#103 to further develop service requirements for emergency and mission-critical services over NTN. Secondly, a new work item revision [4] was approved in RAN#105 that includes the support of PWS for NB-IoT NTN and the potential extension of the intended service area concept. We believe these developments indicate a consensus that a geo-fencing solution should be adopted in ETWS to address the specific needs of NTN. RAN2 can begin confirming this understanding, while the detailed solution can await the reception of the LS response.

Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

# 3	Text Proposals

## 3.1	TP to TS 38.300 v18.3.0

#### 16.10.6.2	Configuration

MBS broadcast can be received by UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. A UE can receive the MBS configuration for broadcast session (e.g., parameters needed for MTCH reception) via MCCH in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The parameters needed for the reception of MCCH are provided via System Information.

The following principles govern the MCCH structure:

-	MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session includes MBS session ID, associated G-RNTI scheduling information and information about neighbouring cells providing certain service on MTCH(s). MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

-	MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; a notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification;

NOTE:	It is up to UE implementation to use the start and stop times and the intended service area in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

-	When the UE receives an MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.

## 3.2	TP to TS 38.331 v18.3.0

#### 5.9.2.2	Initiation

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

NOTE 1:	It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

NOTE 2:	It is up to UE implementation to use the cell/tracking area list or the intended service area in the USD along with the UE’s GNSS position to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Providing the intended service area in SIB has several drawbacks: slow modification period, impacts on non-MBS UEs, increased energy consumption, and inefficient signalling.

Observation 2	The Service Announcement contains the MBS Service Area.

Observation 3	MBS Service Area can be in the form of Cell ID list, TAI list, geographical area information or civic address information.

Observation 4	Service Announcement is essential for MBS. A UE interested in an MBS service must have acquired the Service Announcement to become interested in an MBS service and before attempting to decode MCCH.

Observation 5	UE interested in MBS services can use start and stop times and/or the cell/tracking area list in the Service Announcement to avoid acquiring the MCCH when the MBS session is not ongoing and/or the UE is outside the MBS service area of the MBS broadcast service.

Observation 6	Extending Service Announcement to include the Intended Service Area for NTN cells is a simple solution that has minimal specification impact and fulfils the energy consumption and signalling efficiency criteria.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The intended service area of an MBS broadcast service is not provided in system information. FFS on other broadcast services (e.g., ETWS).

Proposal 2	MBS Service Announcement is used to indicate the intended service area for MBS broadcast in NTN cells.

Proposal 3	Legacy mechanisms for service continuity in 5G MBS Broadcast should be reused for MBS broadcast with NTN.

Proposal 4	RAN2 focuses on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement.

Proposal 5	Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

# 4	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.

R2-2404841, Support for broadcast services in NR NTN, Ericsson, RAN2#126, Fukuoko, Japan, May 2024.

SP-240495, New SID: Study on satellite access - Phase 4, SA#103, Mar. 2024

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.