3GPP RAN WG2 Meeting #127bis 	R2-2409189

# Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.9.2

Source:	InterDigital, Inc.

Title:	Support of Store and Forward.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#127 the following was agreed:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we look at some of the open issues from the previous meeting.

# Discussion

## 2.1 S+F Operation mode

According to the agreements from last meeting:

UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

It is not clear what the intention of a static indication of general support would be, however we assume there may be more than one usage:

- to enable a UE to apply some prioritisation to cell reselection. For example, prioritisation of a cell which is not using S+F in order to allow for longer service time, if possible.

- Use by the application layer to determine whether to start a service.

In the previous meeting, there was also a discussion related to informing the UE with a t-Service like parameter to indicate the remaining time of the current mode. In our opinion, if either of the purposes above a relevant, then a time based parameter would be more suitable and would negate the need for a static indication (because presence of a time based parameter would also imply a general support). The time based parameter is more suitable, because with that the UE can determine whether there is any need to perform a cell reselection prioritization or to defer or start a service, based on the amount of time remaining in the current mode.

Without the time based parameter, then the UE would have no knowledge of how long is left in the current mode, and this might result in more frequent failures, depending on the service. It should be noted that this does not mean that S+F does not work at all, however this is an optimisation that could be used to improve the overall user experience. Reading the WID literally, RAN2 should not introduce such an optimisation in order to minimize UE impact. However, as this type of optimisation can improve user experience, it should not be precluded on the basis of this alone but the technical merits should be considered and sufficiently discussed.

Proposal 1: RAN2 to consider the use-case for, and then whether to introduce a parameter indicating the remaining time of the current mode of S+F.

If the parameter is agreed to be introduced, then only 1 timer is needed and it can be used in correspondence with the indication whether its serving satellite is currently operating in S&F to determine what mode this applies to.

Proposal 2: If a parameter indicating the remaining time of the current mode of S+F is agreed, only 1 time based parameter is needed and it can be used in correspondence with the indication whether its serving satellite is currently operating in S&F to determine what mode this applies to.

In addition, as mentioned above, if we have the time based parameter it can be used to implicitly determine network support for S+F.

Proposal 3: If a parameter indicating the remaining time of the current mode of S+F is agreed, there is no need for another static parameter indicating general NW support.

If the use of the parameter is simply to enable the UE to determine if and when to start a service, then only serving cell information is needed. If the parameter is also used to perform some cell reselection prioritisation then the information would obviously also be beneficial for neighbouring cells.

Proposal 4: If a parameter indicating the remaining time of the current mode of S+F is agreed, and the use-case is related to cell reselection, then this information would be beneficial to broadcast for neighbour cells in addition to the current cell.

## 2.1 Barring

In addition, the barring mechanism was discussed with the following outcome:

RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In principle, there could be a linkage defined between overriding the legacy cellBarred or cellBarred-NTN, such as the time based parameter discussed above, or whether the cell is currently operating in S+F mode.

However, this may not be the optimal way.

When a cell is barred, it is excluded from cell selection and reselection for up to 300s, according to 36.304 [3]

- the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

So, for example, the UE may determine from a legacy barring bit that the cell should be barred, but also notice that the cell is not currently operating in S+F mode. In this case the UE could bar the cell even though it is about to enter S+F mode sooner than 300s, limiting the UE chance to select and use this cell.

For this reason it seems safer (And simpler!) to introduce a new barring bit as we have done in the past for many features. This way, the barring functionality is not made dependent on any other behaviour and the impact is isolated.

Proposal 5: A new barring bit is introduced to enable UEs supporting S+F to select a cell which has barred legacy UEs.

# Conclusion

In this contribution we discuss support of PWS and make the following proposals for discussion:

Proposal 1: RAN2 to consider the use-case for, and then whether to introduce a parameter indicating the remaining time of the current mode of S+F.

Proposal 2: If a parameter indicating the remaining time of the current mode of S+F is agreed, only 1 time based parameter is needed and it can be used in correspondence with the indication whether its serving satellite is currently operating in S&F to determine what mode this applies to.

Proposal 3: If a parameter indicating the remaining time of the current mode of S+F is agreed, there is no need for another static parameter indicating general NW support.

Proposal 4: If a parameter indicating the remaining time of the current mode of S+F is agreed, and the use-case is related to cell reselection, then this information would be beneficial to broadcast for neighbour cells in addition to the current cell.

Proposal 5: A new barring bit is introduced to enable UEs supporting S+F to select a cell which has barred legacy UEs.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”

- 36304-i20, “User Equipment (UE) procedures in idle mode”