3GPP RAN WG2 Meeting #127bis 	R2-2409190

# Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.9.3

Source:	InterDigital, Inc.

Title:	EDT/PUR enhancements.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#126, the following was agreed:

Agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements

In RAN2#127, the following was agreed:

Agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In this contribution, we look at what could potentially be enhanced.

# Overview of existing functionality

## 2.1	MO-EDT

It was agreed that both CP and UP solutions, NB-IoT and eMTC, are in the scope of the study, but only EPS.

The following show MO-EDT procedures for the CP and UP EPS optimizations.

The MO-EDT procedure for Control Plane CIoT EPS optimisation is illustrated in Figure 7.3b- of 36.300 [2]

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

The MO-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3b-2 of 36.300 [2]

Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisation

## 2.2	Transmission using PUR

The procedure for transmission using PUR for the Control Plane CIoT EPS optimisation and for the Control Plane CIoT 5GS optimisation is illustrated in Figure 7.3d-2 of 36.300 [2].

Figure 7.3d-2: Transmission using PUR for the Control Plane CIoT EPS/5GS Optimisations

The procedure for PUR configuration request and PUR configuration is common to the Control Plane CIoT EPS/5GS optimisations and the User Plane CIoT EPS/5GS optimisations and is illustrated in Figure 7.3d-1 of 36.300 [2].

Figure 7.3d-1: PUR Configuration Request and PUR Configuration

# Msg3 transmission without msg1/RAR

As can be seen, if PUR is used, it is already possible to transmit Msg3 without Msg1/RAR as this is the whole point of PUR.

Observation 1: It is already possible with the existing PUR feature to transmit Msg3 without Msg1/RAR.

It should be noted that a PUR resource, although from the UE point of view is a dedicated resource, can be configured by the network to more than one UE. This was discussed during the standardisation of the PUR feature and it is left to eNB implementation whether to configure PUR resources in a shared manner, and how to deal with any contention issue. For example, in case more than one UE uses a PUR resource which has configured, and at least one of those UEs fail, then the application layer can attempt to retry on a future PUR occasion, using RACH based EDT, or using the CIoT optimisation without EDT.

Observation 2: It is already possible with the existing PUR feature to configure PUR resources to more than one UE (shared/contention-based resources).

As can be seen above, the current PUR feature, although it enables Msg3 transmission without Msg1/RAR, and can be configured as contention based, this relies on entering RRC_CONNECTED to be configured with PUR resources which are applicable only in the current cell and when the TA validity conditions are met.

In addition, there is no standardised method to detect a contention, and no standardised method to recover from contention or transmission failure. While introduction of a new transmission scheme may seem attractive on paper, the system and standardisation impact from such a new scheme could be significant.

Proposal 1: To support enhanced contention-based PUR, the following schemes may be considered.

- Introduction of new transmission scheme at layer 1 (CRDSA)

- Introduction of L2/3 based diversity slotted ALOHA (DSA)

- Introduction of contention resolution procedure at L2/3 (e.g. similar to 2 step RA)

Clearly, it is not possible to conclude on (1) without input and work in RAN1, which seems unlikely given that no update was made to the WID for this objective and no additional Tus were allocated for it in RAN1.

RAN2 may need to assume therefore that a L2/3 based procedure is necessary. A L2/3 based procedure might use a form of DSA, the impact of this could be limited to RAN2 specifications and avoid the need to add new TUs to the already heavily loaded work in RAN1.

In addition, a contention resolution procedure would be needed (similar to 2-step RA) whether DSA (or CRDSA) is used or not, since it is still possible that some transmissions will fail because the schemes do not completely avoid contention, and it is possible that CRDSA is not mandatory/supported/configured in all cases.

Proposal 2: A L2/3 based contention resolution procedure is necessary, based on DSA and/or contention resolution procedure (i.e. CRDSA is not feasible to complete in this release).

In addition, there is also no method for the network to configure the resources to be used for these transmissions (E.g. PUR resources) without dedicated signalling. For shared resources, system information configuration is the natural choice.

Proposal 3: Shared PUR resources are configured in system information.

For the case of shared (contention based) PUR resources configured by system information, we will also need to discuss whether a UE needs to be authorised/configured by dedicated signalling before being allowed to use shared PUR resources, or whether this is done only using broadcast signalling (e.g. always allowed based on capability, or allowed based on a broadcast configuration like barring).

Proposal 4: For the case of transmission using shared PUR resources, consider the following options for allowing the UE to transmit

- Always allowed based on UE capability and NW enabling the feature

- Allowing the network to select a subset of UEs (e.g. like access barring)

- Dedicated signalling for authorisation, either at AS or NAS.

Finally, it needs to be discussed how the UE should select a resource, when multiple resources are configured by the eNB. This may somewhat depend on whether a new transmission scheme is introduced by RAN1. However, in case multiple configurations/resources are provided by system information, the UE may need to use a selection mechanism, such as selecting a set of resources based on coverage level, UE-ID, etc.

Proposal 5: RAN2 needs to consider how to select a shared resource (or multiple shared resources in case of DSA), when multiple resources are configured by the eNB.

In addition, it has already been captured that for DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetitions. Currently, the network can configure PUSCH resources for each CE level and configure a certain number of repetitions for those resources, as well as the largest allowed TBS and a RSRP selection threshold.

While the purpose of repetitions is to improve coverage, the purpose of DSA is to improve contention failure. It needs to be carefully considered how these 2 features will work together because using a large number of repetitions to ensure coverage is already demanding on the resource usage, and adding duplication on top may result in an inefficient use of resources.

In Release 13, the maximum TBS for NPUSCH transmission is 1000 bits and the maximum delay k0 that can be set is 64 ms. The maximum number of RUs (NRU ) and repetitions (NRep) are 10 and 128. Therefore, depending on the duration of the RUs which can be up to 32ms depending on subcarrier spacing and number, the maximum duration for a NPUSCH transmission is 10 × 128 × 32 ms = 40.96s. Transmitting just one duplicate would use 256 repetitions in total increasing that transmission to 81.92s.

Proposal 6: FFS if and how DSA (if agreed) and repetitions interact.

While shared PUR configuration may be beneficial, it may also be beneficial in some cases to use a dedicated resource configuration. For example, to provide some UEs with a more reliable transmission mechanism or to avoid contention altogether for some cases. However, the signalling overhead from having to perform PUR configuration in every cell is high, therefore it should also be studied whether dedicated resource configuration can be optimised for NTN, such that a PUR configuration can be made in one cell and be used in another cell.

Proposal 7: Study the support of dedicated signalling for configuring PUR resources across cells.

Proposal 8: FFS whether and how shared and dedicated PUR resources can co-exist.

# Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

As can be seen, only the MO-EDT procedure for CP solution uses “RRCEarlyDataComplete”. User data is attached to Msg3 and Msg4 during the RACH procedure. The procedure is terminated by Msg4.

The MO-EDT procedure for UP solution transfers data in RRC_CONNECTED, after completing random access (5 steps) and the procedure is terminated using RRCConnectionRelease.

The WI objective clearly states that the intention is to reduce overhead of Msg4/RRCEarlyDataComplete, so it is clear that this part of the objective is only applicable to the CP solution.

Proposal 9: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

As can be seen, if PUR is used, it is already possible to terminate EDT without using RRCEarlyDataComplete. As described in 36.300 [2] :

7a	If the (ng-)eNB is aware that there is no pending downlink data or signalling, the (ng-)eNB can send a Layer 1 ACK optionally containing a Time Advance Adjustment to the UE to update the TA and terminate the procedure.

7b	If the (ng-)eNB is aware that there is no further data or signalling, the (ng-)eNB can send a Time Advance Command to update the TA and terminate the procedure.

Observation 3: It is already possible with the existing PUR feature to terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

The main question as to whether early termination of EDT works in practise is “how does eNB know that there is no pending downlink data from the application layer?“

Proposal 10: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

As can be seen, the UE can already transmit an indication (L1Ack) in PURConfigurationRequest, which informs the eNB a preference for using RRCEarlyDataComplete. Note that, although the figure states “L1 ACK” this is named “rrc-ACK” in the RRC specification and it indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR. This is an indirect way to specify that the UE can send some information about whether it expects downlink data in response to the uplink. It is then up to the eNB implementation what (if anything) to do with that knowledge. The main issue with terminating EDT early is when there is some downlink application layer data to send in response. If the procedure is terminated early then the network must page the UE in order to transmit this, which is even more inefficient than waiting and terminating using RRC.

Observation 4 : It is already possible with the existing PUR feature for UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Given that we have established above that UE can already indicate a preference for RRC response, and eNB can already terminate EDT without RRC response, the question is how this can possible be further improved or whether the existing feature is in fact sufficient.

There are 3 potential deficiencies with the existing EDT termination feature with PUR in the context of NTN.

- The current PUR feature only applies if the UE remains in the same cell – i.e. PURConfigurationRequest, and RRCConnectionRelease containing PURConfiguration is needed in the cell on which transmission using PUR is used. For NTN, the cell can change frequently such that the overhead in performing this is excessive.

- The UE can only send an indication “rrc-ACK” in the PUR configuration request, which would apply to all subsequent PUR occurrences configured with PURConfiguration. If we enable dedicated PUR resources which are applicable in multiple cells, or if we enable shared (contention based) PUR in a cell, then there is currently no way for the UE to indicate “rrc-ACK”. There is also currently no network signalling to transfer this information from one eNB to another.

- As a minimum, a L1 ACK needs to be sent in the downlink. There is currently no way to terminate EDT without downlink signalling, or without sending downlink signalling to all UEs always.

Proposal 11: Confirm that the existing PUR feature may not be optimal in an NTN deployment as there is currently no mechanism for the eNB to determine whether UE expects a downlink application layer response unless the UE moves to RRC_CONNECTED in every cell to perform dedicated PUR configuration.

To address these deficiencies, some possible enhancement can be considered.

- Enabling PUR configuration information across cells. This may imply that the information sent by the UE (i.e. the rrc-ACK indication) needs to be transferred between eNBs using network signalling)

- Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter – which is an indication whether the UE expects a downlink application layer response. This would enable the UE to indicate to the cell on which the transmission using PUR is initiated, and indicate per transmission. (This could work whether dedicated or shared PUR resources are used).

- Enabling EDT termination without any downlink ACK

- Using a common ACK for multiple transmissions or UEs (E.g. using a group common signalling)

Proposal 12: RAN2 to consider whether the following enhancements are beneficial:

- Introducing network signalling so that PUR configuration request information to be transferred across cells.

- Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter (whether UE expects a downlink application layer response)

- Enabling EDT termination without any downlink ACK

- Using a “common” ACK for multiple transmissions or UEs

# Conclusion

In this contribution we discuss uplink capacity enhancements and make the following proposals for discussion:

Msg3 transmission without msg1/RAR

Observation 1: It is already possible with the existing PUR feature to transmit Msg3 without Msg1/RAR.

Observation 2: It is already possible with the existing PUR feature to configure PUR resources to more than one UE (shared/contention-based resources).

Proposal 1: To support enhanced contention-based PUR, the following schemes may be considered.

- Introduction of new transmission scheme at layer 1 (CRDSA)

- Introduction of L2/3 based diversity slotted ALOHA (DSA)

- Introduction of contention resolution procedure at L2/3 (e.g. similar to 2 step RA)

Proposal 2: A L2/3 based contention resolution procedure is necessary, based on DSA and/or contention resolution procedure (i.e. CRDSA is not feasible to complete in this release).

Proposal 3: Shared PUR resources are configured in system information.

Proposal 4: For the case of transmission using shared PUR resources, consider the following options for allowing the UE to transmit

- Always allowed based on UE capability and NW enabling the feature

- Allowing the network to select a subset of UEs (e.g. like access barring)

- Dedicated signalling for authorisation, either at AS or NAS.

Proposal 5: RAN2 needs to consider how to select a shared resource (or multiple shared resources in case of DSA), when multiple resources are configured by the eNB.

Proposal 6: FFS if and how DSA (if agreed) and repetitions interact.

Proposal 7: Study the support of dedicated signalling for configuring PUR resources across cells.

Proposal 8: FFS whether and how shared and dedicated PUR resources can co-exist.

Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Proposal 9: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

Observation 3: It is already possible with the existing PUR feature to terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 10: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Observation 4 : It is already possible with the existing PUR feature for UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Proposal 11: Confirm that the existing PUR feature may not be optimal in an NTN deployment as there is currently no mechanism for the eNB to determine whether UE expects a downlink application layer response unless the UE moves to RRC_CONNECTED in every cell to perform dedicated PUR configuration.

Proposal 12: RAN2 to consider whether the following enhancements are beneficial:

- Introducing network signalling so that PUR configuration request information to be transferred across cells.

- Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter (whether UE expects a downlink application layer response)

- Enabling EDT termination without any downlink ACK

- Using a “common” ACK for multiple transmissions or UEs

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”