3GPP RAN WG2 Meeting #127bis 	R2-2409192

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.6.3

Source:	InterDigital, Inc.

Title:	Event triggered L1 measurement reporting for LTM.

Document for:	Discussion

# Introduction

The Rel-19 WI on mobility enhancements contains the following measurement related objectives.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

In RAN2#126, the following progress was made:

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. - FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. - FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. - Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. - It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. - FFS on the need of measurement reporting once leaving condition is met.

In RAN2#127, the following progress was made:

Agreements on L1 MR event evaluation 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering.

# Discussion

Although it was agreed that MAC layer performs the evaluation and reporting, which implies that the RRC text on event triggering needs to be copy/pasted to the MAC specification, one other aspect to consider is where the most suitable place would be to define the mobility events.

As the events agreed for LTM are very much based on the existing RRC Ax events, and rather than copy/paste everything from RRC, it seems most convenient to re-use what we can of the existing RRC framework and enhance this to support MAC layer triggering and reporting based on L1 beam measurements, and the events already defined in RRC.

Proposal 1: LTM measurement event definitions to be defined in RRC.

Proposal 2: Re-use existing RRC measurement events as much as possible.

It needs to be discussed whether the existing events can simply be extended to allow beam-based evaluation, and MAC (or L1) based reporting, or whether to introduce separate event definitions. If the functionality is more or less the same, with some small changes then it’s better to extend the existing events. The main enhancements needed to RRC measurement reporting events themselves are to include new (shorter) time-to-trigger values and to define beam quality trigger.

Proposal 3: RAN2 to discuss whether to extend the existing measurement events (Ax), or to introduce separate ones.

MAC CE has been agreed as the reporting container for LTM event reporting. As an example, the MAC CE report could be encoded in a similar way as below.

Figure 1: Potential MAC CE report encoding.

In the example, the MAC CE contains the following information.

- Event ID – assuming multiple events can be configured, the report need to include as a minimum the event ID.

- Beam indication(s) – the report may need to include a bitmap indicating which of the configured candidate beams meet the event criteria (Assuming that one event might consider multiple candidate beams – then the report needs to indicate which beams are above a threshold or other criteria). The beam bitmap might also be used to indicate which beams are included in the list of reported power values (e.g. in the illustration above, candidate beam 1 in octet 7 could correspond to the lowest bit which is set to “1”). Another way would be to explicitly indicate beam indexes. The most efficient way in terms of signalling overhead depends on how many beams may need to be indicated, and what the information reported will be. At a high level it should be clear we need a way to identify the reported beams, and the details can be discussed later.

- Power value(s) – the power value of the best N beams, or an explicitly configured N beams, may need to be included. It’s likely that this power value would take the form of a table index, similar to e.g. power headroom report, in order to minimise signalling overhead.

Figure 2: Possible power value mapping.

Proposal 4: At least the following information needs to be contained in an event-triggered L1 measurement report for LTM:

- Event ID.

- Beam indication(s) (identifying the beams reported/meeting event criteria).

- Power value(s).

As mentioned above, the most efficient way to define the MAC CE depends on how many beams can be reported. For example, if only the beam triggering the event needs to be reported then it can be done with explicit beam ID and power value. However, if multiple beams can be reported (e.g. best N) then bitmap approach might be better.

Proposal 5: RAN2 to discuss the maximum number of measured beams to be included in an LTM report.

With the existing mechanism, and assuming MAC CE for LTM reporting has higher logical channel priority for transmission, most of the time there will be sufficient uplink grant to transmit the report straight away. However, in case insufficient grant is available to transmit all of the information, a truncated or reduced report content could be considered. For example, if only enough grant is available to transmit the event ID, in some cases the event ID may be sufficient (for some events, at least). If only enough grant is available to transmit event ID and beam bitmap then the power values may be omitted.

Proposal 6: A truncated MAC CE report should be defined, and transmitted in case there is not enough grant to transmit a full report.

In some cases there may not be sufficient grant to transmit a report at all. In this case, the legacy procedure would be to transmit a SR And BSR, which would delay the LTM report transmission. Some consideration should be given to what the procedure should be in this case – for example, special SR value to indicate an LTM report.

For UCI-based event-triggered beam management reporting, RAN1 agreed to support two modes. In the first mode (Mode A), the UE transmits a first PUCCH to request a resource for a second UL channel (at least PUSCH) to carry the beam report, and subsequently provides the report in the resource indicated by DCI. In the second mode (Mode B), the UE transmits a first PUCCH to notify that it will transmit in a second pre-configured UL channel. From the perspective of reliability, Mode A seems attractive provided that a retransmission mechanism for the first PUCCH is supported in case the UE does not get the second resource.

In case event-triggered beam management reporting is configured, an early indication for L1 event triggered LTM report could be sent as part of the UCI resources used for beam management reports – in other words, re-use the beam management mechanism at least in some specific cases to ensure latency. E.g. in case there is not enough grant to transmit MAC CE, or for specific events, an indication can be sent using the first PUCCH resource in beam management reporting mode A to notify the gNB that an LTM report is available for transmission.

Proposal 7: Support early indication of LTM report using UCI, at least for the case of insufficient/no grant for MAC CE transmission.

One additional consideration would be what happens when MIMO and LTM events are running in parallel. For example, if a MIMO BM event is triggered because, for example, (agreed Event-7b): Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality. If LTM events are based only on best beam, regardless of TCI state, then there may not be any inter-dependency. However, if (e.g.) beam management event 7b causes the serving cell beam to be changed, then we may need to consider how this impacts LTM measurement evaluation. For example, if an LTM event with TTT currently running based on a serving cell beam which is changed, does the evaluation restart or continue? Given that the WI indicates that there will be a check in RAN#105, this can be revisited in a later meeting.

Proposal 8: FFS whether there is any interaction between evaluation of MIMO BM events and LTM events (can revisit in a later meeting)

# Conclusion

In this paper we make some initial suggestions on event-triggered L1 reporting for LTM as follows:

Proposal 1: LTM measurement event definitions to be defined in RRC.

Proposal 2: Re-use existing RRC measurement events as much as possible.

Proposal 3: RAN2 to discuss whether to extend the existing measurement events (Ax), or to introduce separate ones.

Proposal 4: At least the following information needs to be contained in an event-triggered L1 measurement report for LTM:

- Event ID.

- Beam indication(s) (identifying the beams reported/meeting event criteria).

- Power value(s).

Proposal 5: RAN2 to discuss the maximum number of measured beams to be included in an LTM report.

Proposal 6: A truncated MAC CE report should be defined, and transmitted in case there is not enough grant to transmit a full report.

Proposal 7: Support early indication of LTM report using UCI, at least for the case of insufficient/no grant for MAC CE transmission.

Proposal 8: FFS whether there is any interaction between evaluation of MIMO BM events and LTM events (can revisit in a later meeting)

# References

- RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”