3GPP RAN WG2 Meeting #127bis 	R2-2409193

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.6.4

Source:	InterDigital, Inc.

Title:	Conditional LTM.

Document for:	Discussion

# Introduction

The Rel-19 WI on mobility enhancements contains the following measurement related objectives.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In the previous meeting, the following progress was made:

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. - FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. - FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. - Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. - It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. - FFS on the need of measurement reporting once leaving condition is met.

In RAN2#127, the following progress was made:

Agreements on L1 MR event evaluation 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering.

# Discussion

For event triggered reporting, it has been agreed to consider the following scenarios:

  - Select the candidate beam/cell to trigger early synchronization.

  - Select the target beam/cell and trigger LTM cell switch procedure.

For the conditional case, similar scenarios should also be considered. The conditional LTM trigger could be a trigger to execute LTM cell switch itself (i.e. similar to CHO, the RRC reconfiguration is executed, or it could be a trigger for performing early synchronization (e.g. activate TCI on target, trigger a RA for TA acquisition).

Proposal 1: Conditional LTM should be designed for the following LTM purposes:

  - Trigger early synchronization on the candidate beam/cell meeting trigger condition.

  - Trigger LTM cell switch procedure on the candidate beam/cell meeting trigger condition.

For the case of triggering LTM, the most straightforward way is to perform the LTM cell switch according to the candidate configuration stored in the LTM candidate list. This is almost the same as CHO, but the conditional reconfiguration refers to the LTM candidate configuration.

Proposal 2: Conditional LTM cell switch means that the UE executes LTM switch using the LTM candidate configuration corresponding to the trigger condition which has been met (similar to CHO, but apply the LTM candidate configuration as if MAC CE indication had been received)

In terms of what trigger(s) to use, in R19 we are introducing beam based events triggered based on L1 beam measurements. While these may be suitable for some scenario (e.g. trigger early sync) they may not be good enough for triggering cell switch without carefully considering parameters such as TTT and hysteresis. For triggering of cell switch, a more suitable approach might be cell based conditions, in which case it may be better to re-use the L3 events. In other words, it is worth considering allowing a conditional LTM to be associated with either a beam based event (e.g. e.g. LTMx) or a cell based events (e.g. event Ax), or perhaps both.

Proposal 3: A conditional LTM may be configured to be associated with a beam-based condition (event LTMx), a cell based condition (event Ax), or both.

As discussed above, conditional LTM may be used for different purposes. Not only could this be used to trigger cell switch/reconfiguration, it could be used to trigger early synchronization. Essentially, it should be possible to configure a condition (e.g. beam-based, cell-based threshold) and an action (cell switch, UL sync, DL sync).

Proposal 4: A conditional LTM may be configured to be associated with a cell switch (reconfiguration), a DL early sync (TCI state activation), or an UL early sync (RA for TA acquisition).

One of the main advantages of LTM in R18 is the reduction of interruption due to support of RACH-less. If the UE has a valid TA for a particular target meeting the condition, then support of RACH-less conditional LTM could be supported. Some discussion would be needed on the precise details of how the UE receives and maintains TA validity, but this is entirely possible.

Options include

- Perform PDCCH order triggered RA then receive the TA in a MAC CE indicating cell switch (similar to R18 LTM). Execute the cell switch only after a condition is met (not immediately) and execute RACH-less if the TA is still valid (E.g. before a validity timer expires)

- Receive the TA in a RAR from the target after the RA is conditionally triggered, execute cell switch based on another condition.

- Perform PDCCH order triggered RA then receive the TA in a RAR from the target. Trigger the cell switch when the condition is met.

Proposal 5: Support RACH-less conditional LTM. Details of TA validity and acquisition FFS.

# Conclusion

In this paper we make some initial suggestions on conditional LTM as follows:

Proposal 1: Conditional LTM should be designed for the following LTM purposes:

  - Trigger early synchronization on the candidate beam/cell meeting trigger condition.

  - Trigger LTM cell switch procedure on the candidate beam/cell meeting trigger condition.

Proposal 2: Conditional LTM cell switch means that the UE executes LTM switch using the LTM candidate configuration corresponding to the trigger condition which has been met (similar to CHO, but apply the LTM candidate configuration as if MAC CE indication had been received)

Proposal 3: A conditional LTM may be configured to be associated with a beam-based condition (event LTMx), a cell based condition (event Ax), or both.

Proposal 4: A conditional LTM may be configured to be associated with a cell switch (reconfiguration), a DL early sync (TCI state activation), or an UL early sync (RA for TA acquisition).

Proposal 5: Support RACH-less conditional LTM. Details of TA validity and acquisition FFS.

# References

- RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”