3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409200

Hefei, China, 14 - 28 October 2024	Revision of R2-2408511

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Initial analysis on RAN2 impact for Rel-19 NR MIMO Ph5

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, according to the WID as shown below [1] and recent RAN1 inputs [5], objectives relevant to UE-initiated/event-driven beam management (UEIBM) and asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present detailed analysis and provide our considerations on how RAN2 start work on the UEIBM and asymmetric DL sTRP/UL mTRP, based on RAN1 agreements so far.

2	UE Initiated/Event-driven Beam Management

In order to reduce the periodic CSI reporting overhead and latency, event-based CSI reporting is agreed by RAN plenary for both Rel-19 MIMO WI and Rel-19 LTM WI i.e., the UE reports CSI only when the event is triggered. This includes specification of the event triggering procedure and of the reporting procedure after the event is triggered. Those points are generally similar between two WIs. However, after some detailed discussion in RAN1 and RAN2 meetings, there were diverging agreements on reporting procedure. More specifically, RAN1 agreed for MIMO to use UCI to carry the CSI report while RAN2 agreed for LTM to use MAC CE. Based on the latest RAN plenary agreement, each WI can proceed its solution separately.

Although the beam reporting in MIMO and LTM can use different signalling, other aspects e.g., event triggering can be quite similar, which could be specified in the MAC layer. In order to reduce the maintenance work of the specification, it is better to capture this aspect of MIMO and LTM together. In RAN2 LTM, it is also raised by several companies that the relevant LTM discussion in RAN2 can be to align with the MIMO progress in RAN1.

Observation 1: even though the signalling used for reporting is different, event-triggered reporting for MIMO and for LTM can be unified under the same UE procedure in the MAC specification.

In the following, we provide our considerations on the RRC configuration, trigger event for event-driven beam management, beam reporting and transmission. Then, we consider how to model this in the specifications.

2.1	RRC Configuration

The parameters described above are part of the RAN1 Liaison Statement to RAN2 (R1-2407285), focusing on RRC impacts for Rel-19 MIMO. Here is the usage of the above parameters.

- The parameter reportTransmissionMode-r19 indicates the mode applied for beam reporting.

- The parameter eventThreshold-r19 defines the RRC-configured threshold value for Event-2.

- The parameters eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 represent the time window length and the number M used in event evaluation procedure.

- The parameter enabledCurrentBeamReport-r19 determines whether the current beam should be reported while the parameter nrofReportedRS-UEIBR-r19 shows the number of RS to be reported.

- The parameter newBeamResourceSetEvent2-r19 is used to configured the resource set of new beams for event-2.

Currently, L1 report configuration is in the CSI-ReportConfig IE. In this IE, the reportConfigType field is a choice between periodic, semi-persistent, and aperiodic reporting. For Rel-19 MIMO, event-triggered reporting is an additional option with a corresponding configuration. Each report configuration is associated with a single event type. Parameters such as the threshold, time window, and the value M should also be configured for the event-trigger reporting.

Proposal 1: For UEIBM, create a new choice value of the reportConfigType field in the CSI-ReportConfig IE including reportTransmissionMode-r19, eventThreshold-r19, eventInstanceCount-r19, eventDetectionTimeWindowLength-r19, enabledCurrentBeamReport-r19, nrofReportedRS-UEIBR-r19, newBeamResourceSetEvent2-r19.

2.2	Event triggering

The following agreement was made for event triggering in RAN1:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

According to the RAN1 agreements, an event-2 instance occurs when the L1-RSRP of the new beam is a threshold value greater than the current beam. This threshold is configured by the network. Then, there are two options:

- option1: upon occurrence of an event-2 instance, the UE initiates the reporting procedure.

- option2 (subject to a UE capability): when M event-2 instances for the same beam occur within a time window, the UE initiates the reporting procedure.

For LTM, RAN2 agreed to support the TTT mechanism. That mechanism provides the same functionality like the time window and number of instances agreed for MIMO in RAN1, so the mechanism agreed by RAN1 could be reused for LTM. Doing so could simplify the RAN2 specifications.

Proposal 2: RAN2 will strive for a common design for LTM event evaluation and MIMO event triggering procedures at least for Event-2 to reduce maintenance work for RAN2 TS.

Based on the RAN1 agreements, RRC should configure a threshold, time window length and M for controlling UE on triggering event determination (Event-2). It is likely that this event evaluation procedure can also be applied to other MIMO events, such as event-1 and event-7. RAN2 can wait for further progress from RAN1 on this.

Currently, BF and RLF are declared upon receiving a certain number of instances of an event from lower layers, which is similar to RAN1 agreement for option-2 above. However, these procedures have a difference:

- Option-1: In the BF detection procedure, the timer is (re)started upon the reception of an instance, and the counter is incremented. Upon timer expiry, the UE resets the counter to 0. When the counter reaches the configured number of event instances, the UE consider beam failure to be detected.

- Option-2: In the RLF procedure, the timer is started upon reception of N310 consecutive ”out-of-sync” instances and continues running until receiving N311 consecutive “in-sync” instances. At the point of timer expiration, RLF occurs.

Either of the two options could be used to trigger beam reporting, this can be kept FFS.

Proposal 2a: For UEIBM, the UE maintains a timer and counter. The timer is started upon reception of the event instance and the counter is incremented at the reception of each event instance. FFS whether the timer is restarted at every even instance (like for BF detection) or not (like for RLF detection).

A further aspect is whether to use common counter for all the beams or a separate for each beam needs further discussion. Here we show two different understandings:

- Option 1: only event instances for the same beam as the first event instance are counted.

Start the timer

√means the beam satisfies the event condition

Event instance

Event instance

Beam 1√

Beam 1√

Beam 1√ √

Beam 1√

→ Counter: 4

Start the timer

Option 2: any event instances are counted; the beam may be different between first event instance and subsequent event instance.

Event instance

Event instance

→ Counter: 4

Counter: 4

Beam 3√

Beam 1√

Beam 2√

Beam 1√

In option 1, the time window is defined per beam. In option 2, the time window is defined per event type, any beam which satisfies the event conditions can be considered as an event instance. Compared with option 1, the benefit of option 2 is that the network is able to receive the beam report in time. For this part, MIMO and LTM should also be captured together to simplify the specification.

Proposal 2b: For UEIBM, discuss how to maintain the counter for trigger event determination.

- Option-1: only event instances for the same beam as the first event instance are counted.

- Option-2: any event instances are counted; the beam may be different between first event instance and subsequent event instance.

2.3	Beam reporting and UL transmission

For MIMO, the UE sends the L1 report to the network using UCI. Two modes (Mode-A and Mode-B) have been agreed by RAN1.

During RAN1 discussions, an alternative option of transmitting the beam report via a MAC CE (Option-1) was considered but ultimately excluded. This decision was guided by a WID which explicitly assumes the reuse of the legacy CSI-RS measurement and reporting framework, where CSI-RS reports are transmitted via UCI. Given that UCI serves as a suitable container for these reports, the reuse of UCI was considered a reasonable approach.

For the UCI-based beam reporting, three options were listed during the discussions:

•	Option 2 (mode A): beam report is transmitted in a UCI in dynamically allocated UL resource.

•	Option 3 (mode B): beam report is transmitted in a UCI in pre-configured UL resources.

•	Option 4: beam report is transmitted in a UCI in pre-configured UL resources not specifically dedicated for UEI beam report.

Initially, option 3 (mode B) was accepted due to several advantages. This mode significantly reduces the complexity for the network as the network does not need to monitor whether a UEI beam report is carried in the pre-configured resource at all times. Additionally, from the network perspective, mode B offers better resource efficiency, as the pre-configured resources can be shared among UEs.

Then, option 2 (mode A) was also discussed. One key advantage of mode A is that the network can schedule flexibility to avoid collision issue on the second channels. It has very high resource utilization. However, the dynamic resource allocation of step 2 in mode A could introduce some latency. Therefore, mode A was also agreed by RAN1 as a fundamental feature for the UE.

Regarding option 4, the uplink resource to carry the UEIBM reporting is pre-configured, the absence of pre-indication requires the network to monitor all pre-configured uplink resources for potential UE beam reporting. This could result in unused resources or lead to collisions if multiple UEs share the same pre-configured resources. Therefore, from the perspective of network complexity and resource utilization, option 4 is not advantageous.

In conclusion, there are two modes used for the beam reporting procedure: mode A (option 2) and mode B (option 3).

2.3.1	Mode A reporting procedure

Mode A reporting has the following procedure:

Figure.1 Mode A for UE-initiated/event-driven beam reporting

On mode A configuration, RAN1 has the following agreement:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following. - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID Above applies at least for the single CC case.

Based on the above agreement, two options are considered for mode A configuration: dedicated SR or new UCI type. The first PUCCH in mode A provides a functionality very similar to SR, as it requests resources for PUSCH and is followed by the network response (DCI) that indicates the allocated PUSCH resources for beam reporting. Reusing the dedicated SR configuration and procedure would simplify the MAC procedure design.

Proposal 3a: For mode A, re-use the existing SR RRC configuration and MAC procedure for first PUCCH channel.

2.3.2	Mode B reporting procedure

Mode B reporting has the following procedure:

Figure.2 Mode-B for UE-initiated/event-driven beam reporting

On mode B configuration, RAN1 has the following agreement:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

SR is currently used to request a DG for new transmission. However, in mode B, it is used to indicate a CG occasion that is used for beam reporting, so the purpose and the procedure are different On the other hand, the MAC specification could be simpler if SR is used for mode B. The decision can be left to RAN1. Regardless of whether the SR or a new UCI type is selected, the procedures can still be described in the MAC specification.

Proposal 3b: For mode B, it is up to RAN1 to decide for the first PUCCH channel configuration.

2.4	Specification aspects

RAN1 is discussing in which specification to capture the UEIBM. For the BFR procedure, two components are used for beam failure detection, BFI_COUNTER and beamFailureDetectionTimer. Once beam failure is detected, the UE can perform BFR via random access or MAC CE with beam failure and candidate beam information. These procedures are captured in MAC.

UEIBM contains the event evaluation and reporting procedure, the event evaluation is also based on a counter and a timer. The whole procedure is quite similar to beam failure detection and BFR so it makes sense to also capture it in MAC.

Proposal 4: Capture the event evaluation and reporting (using timer and counter) for UEIBM in the MAC specification.

3	Asymmetric DL sTRP/UL mTRP

In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. In pre-Rel-19 mTRP scenarios, each TRP has downlink and may have uplink, as shown in figure 3 below. In Rel-19, a new scenario, asymmetric DL sTRP/UL mTRP is considered, in which one TRP has downlink and uplink while the other TRP only has uplink, as shown in Fig.4.

Figure 3: pre-Rel-19 multi-TRP scenario

Figure.4 Asymmetric DL sTRP/UL mTRP scenario

3.1	Pathloss

In pre-Rel-19 mTRP scenarios, each TRP can transmit DL RS and the UE can measure the path loss directly based on each DL RS from the TRP. In Rel-19, a new scenario is considered, in which one TRP does not transmit any DL RS. For the TRP with DL RS transmission, the UE calculates the path loss based on the DL RS. However, for the TRP without DL RS transmission, a new mechanism is needed e.g., the path loss is calculated using the path loss to the TRP with DL transmission and an offset. RAN1 agreed that different path loss offset values can be configured for different TCI states of a TRP (as the TRP deployment is not visible to the UE):

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the above agreement, RAN2 should introduce in RRC a path loss offset for each joint and UL TCI state and a new MAC CE with eLCID for updating the path loss offset for TCI states after RRC configuration. However, this implies that the UE configuration, provided by RRC would be updated by a MAC CE, which is normally not allowed.

Proposal 5: RAN2 assumes that RRC configures an PL offset for each TCI state configuration. A new MAC CE with eLCID is introduced to update PL offset for TCI states associated. Discuss how to realize the functionality agreed by RAN1 without modifying RRC configuration with a MAC CE.

However, it is still unclear what format details this new MAC CE looks like. To finalize MAC CE format, it would be necessary to send RAN1 LS to ask some questions for details, for instance:

- whether this new MAC CE applies to CA and DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

Proposal 6: RAN2 send RAN1 LS to ask for MAC CE format details:

- Whether this new MAC CE applies to CA and/or DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

3.2	PHR

3.2.1	Triggering

Based on the TS 38.321, the following PHR triggering is defined as:

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

In the legacy mTRP scenario, different TRPs can transmit different DL RSs and the UE can measure the PLs separately based on each DL RS from the mTRP. In asymmetric DL sTRP/UL mTRP scenarios, the PL for one UL TRP is derived based on the DL TRP of the TRP and the PL for the other UL TRP is derived based on the DL TRP and PL offset as mentioned in above. Though the PL calculation method of each mTRP has been modified, the PHR is still triggered according to the change of PL. Since the triggering mechanism has not been changed, such PL determination may not impact the PHR triggering.

Proposal 7: RAN2 to discuss the PHR triggering impact on pathloss regarding asymmetric DL sTRP/UL mTRP scenarios.

3.2.1	Format

Based on the TS 38.213, PH is calculated by physical layer and indicated the calculated PH to MAC layer. Then MAC layer includes it in the PHR MAC CE in respect to UL resource after PHR triggering is met. In Rel-19, RAN1 made the following agreement on PH calculation for asymmetric DL sTRP/UL mTRP scenarios.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Based on the above agreement, RAN2 can assume physical layer will indicate calculated PH for asymmetric DL sTRP/UL mTRP scenarios to MAC layer, and as in legacy, the MAC layer includes it in the existing MTRP PHR MAC CE, i.e. no new MTRP PHR MAC CE is introduced.

Proposal 8: RAN2 assumes no new MTRP PHR MAC CE is introduced, i.e. the existing MTRP PHR MAC CE can be reused.

# 4	Conclusion

In this paper, we have addressed the objective regarding initial analysis on RAN2 impact for MIMO. There are the following observations and proposals:

UE Initiated Beam Management

Observation 1: even though the signalling used for reporting is different, event-triggered reporting for MIMO and for LTM can be unified under the same UE procedure in the MAC specification.

Proposal 1: For UEIBM, create a new choice value of the reportConfigType field CSI-ReportConfig IE including reportTransmissionMode-r19, eventThreshold-r19, eventInstanceCount-r19, eventDetectionTimeWindowLength-r19, enabledCurrentBeamReport-r19, nrofReportedRS-UEIBR-r19, newBeamResourceSetEvent2-r19.

Proposal 2: RAN2 will strive for a common design for LTM event evaluation and MIMO event triggering procedures at least for Event-2 to reduce maintenance work for RAN2 TS.

Proposal 2a: For UEIBM, the UE maintains a timer and counter. This timer is started upon the event instance and the counter is incremented at the reception of each event instance. FFS whether the timer is restarted at every even instance (like for BF detection) or not (like for RLF detection).

Proposal 2b: For UEIBM, discuss how to maintain the counter for trigger event determination.

- Option-1: only event instances for the same beam as the first event instance are counted.

- Option-2: any event instances are counted; the beam may be different between first event instance and subsequent event instance.

Proposal 3a: For mode A, re-use the existing SR RRC configuration and MAC procedure for first PUCCH channel.

Proposal 3b: For mode B, it is up to RAN1 to decide for the first PUCCH channel configuration.

Proposal 4: Capture the event evaluation and reporting procedure (using timer and counter) of UEIBM  in the MAC specification.

Asymmetric DL sTRP/UL mTRP

Proposal 5: RAN2 assumes that RRC configures an PL offset for each TCI state configuration. A new MAC CE with eLCID is introduced to update PL offset for TCI states associated. Discuss how to realize the functionality agreed by RAN1 without modifying RRC configuration with a MAC CE.

Proposal 6: RAN2 send RAN1 LS to ask for MAC CE format details:

- Whether this new MAC CE applies to CA and/or DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

Proposal 7: RAN2 to discuss the PHR triggering impact on pathloss regarding asymmetric DL sTRP/UL mTRP scenarios.

Proposal 8: RAN2 assumes no new MTRP PHR MAC CE is introduced, i.e. the existing MTRP PHR MAC CE can be reused.

# 5	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #108 chairman notes;

- TS 3GPP 38.331 V18.2.0

- TS 3GPP 38.321 V18.3.0

- R1-2407285 LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5