3GPP TSG-RAN WG2 Meeting #127bis					           R2-2409203

Hefei, China, October 14th – 18th, 2024					Revision of R2-2408736

Agenda Item:		8.3.2

Source:			Huawei, HiSilicon

Title:			Simulation results and discussion on RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#126 meeting, the agreement on the priority of each use cases of RRM measurement prediction were reached as follows:

Agreements Case number; Prioritization; Evaluation scenario combination; target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell.

According to the agreement, case 2/3/4 are considered as high priority. In this paper, we will provide some simulation results for RRM measurement prediction, which will cover cases 2/3/4. In addition, we will also provide the detailed simulation assumptions and model description. The simulation assumptions that we used are provided in Annex A.

# 2   Simulation results for RSRP prediction accuracy

## 2.1 General

To facilitate the discussion of the prediction results, a unified report template within excel file were agreed. The simulation results in that form are attached to this contribution in separate files for easy merging with results from other companies. In this paper, we list our RRM measurement prediction results in the tabular report format which was agreed earlier and is captured in Annex A.1 of [1]. The simulations were performed using different filtering methods to observe the effect of L1/L3 filtering on the prediction accuracy.

## 2.2 Case 2 (FR1 intra-frequency temporal domain case B)

Table 3: Simulation assumptions, settings and model information for Case 2

| Report parameters |  |  |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50%, 66% |
|  | Observation window (Note3) | See below |
|  | Prediction window (Note3) | See below |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 60000 |
|  | Testing | 1000 |
| AIML model input/output | Model input (Note 5) | RRM sub-use case 1 |
|  | Model output(Note 6) | RRM sub-use case 1 |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.03 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | See below for results for different speeds and OW/PW values. |

We simulated MRRT of 50% and 66%, according to the following patterns:

| Measurement | Prediction |
| --- | --- |

|  |  |  |  | MRRT=50% |
| --- | --- | --- | --- | --- |

|  |  |  |  |  |  | MRRT=66% |
| --- | --- | --- | --- | --- | --- | --- |

The results are presented in tables below.

Table 4: Simulation results for Case 2 (in red, results with RMSE<3dBm)

| MRRT: 50%, without L1 filtering, RRM sub-use case 1 |  |  |  | MRRT: 50%, with L1 filtering (non-sliding window), RRM sub-use case 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |  | OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |
| 120 | 40 | 0.15/0.25/1.11/1.46 |  | 600 | 200 | 0.21/0.73/2.49/2.80 |
|  | 360 | 0.54/1.83/4.47/5.48 |  |  | 600 | 0.57/1.84/3.00/4.36 |
| 200 | 40 | 0.18/0.25/1.07/1.50 |  | 1000 | 200 | 0.18/1.27/2.10/2.98 |
|  | 360 | 0.55/1.72/4.39/5.54 |  |  | 600 | 0.56/1.52/3.33/4.58 |
| 600 | 200 | 0.34/0.89/3.29/4.56 |  |  |  |  |
|  | 600 | 0.83/2.00/3.79/5.91 |  |  |  |  |
| 1000 | 200 | 0.33/1.41/2.79/5.07 |  |  |  |  |
|  | 600 | 0.75/1.59/3.68/6.24 |  |  |  |  |
| MRRT: 66%, without L1 filtering, RRM sub-use case 1 |  |  |  | MRRT: 66%, with L1 filtering (non-sliding window), RRM sub-use case 1 |  |  |
| OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |  | OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |
| 160 | 80 | 0.24/0.58/2.11/2.43 |  | 800 | 400 | 0.56/2.11/2.54/4.68 |
|  | 440 | 0.81/2.47/5.48/7.09 |  |  | 1000 | 1.11/2.14/4.47/5.89 |
| 280 | 80 | 0.22/0.68/2.02/2.43 |  | 1400 | 400 | 0.57/2.05/4.00/5.88 |
|  | 440 | 0.77/2.61/5.20/7.12 |  |  | 1000 | 1.09/2.22/4.52/6.00 |
| 800 | 400 | 0.76/2.40/4.90/8.21 |  |  |  |  |
|  | 1000 | 1.33/2.64/4.91/8.30 |  |  |  |  |
| 1400 | 400 | 0.73/2.72/5.02/8.12 |  |  |  |  |
|  | 1000 | 1.28/2.65/4.76/8.96 |  |  |  |  |

In the results above, we also highlighted the predictions where RMSE was below a 3dBm threshold. This threshold is used only to facilitate an analysis of the prediction accuracy and it does not mean that the prediction accuracy is necessarily good or bad. In order to better understand what kind of prediction accuracy may be acceptable for the system, it is necessary to consider other KPIs, such as system level KPIs, as discussed in section 3.

In general, based on the results it can be observed that the prediction accuracy is highly dependent on UE speed, prediction window length and whether L1 filtering is used or not, as well as combination of these factors. In case L1 filtering is applied, it is possible to obtain RMSE<3dBm even for very high speed scenarios (120 km/h) when PW is no higher than 200 ms.

The following observations can be made based on the above results:

Observation 1 (for case 2): With the MRRT increasing, the prediction accuracy is declining.

Observation 2 (for case 2): With the UE speed increasing, the prediction accuracy is declining.

Observation 3 (for case 2): When L1 filtering is applied, the prediction accuracy increases.

Observation 4 (for case 2): With the increase of PW length, the accuracy is declining.

Observation 5 (for case 2): Increasing an OW beyond some point results in decreased prediction accuracy for higher UE speeds (>=60 km/h), while for low speed UEs the results for different OW values are comparable (for the simulated OW/PW set).

## 2.3 Case 3 (FR1 to FR1 inter-frequency (frequency domain))

For Case 3, simulation assumptions and results can be found in Table X and Y below.

Table 5: Simulation assumptions, settings and model information for Case 3

| Report parameters |  |  |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | See below |
|  | Prediction window (Note3) | N/A |
|  | Any other parameters (Note 4) | High to low frequency prediction was performed |
| Data Size (Number of Samples) | Training/validity | 30000 |
|  | Testing | 500 |
| AIML model input/output | Model input (Note 5) | RRM sub-case 1 and 2 |
|  | Model output(Note 6) | RRM sub-case 1 and 2 |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.03 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | See below |

Table 6: Simulation results for Case 3

| Without L1 filtering, RRM sub use case 1 Measured frequency: 4 GHz, predicted frequency: 2 GHz |  |  | With L1 filtering, RRM sub use case 1 Measured frequency: 4 GHz, predicted frequency: 2 GHz |  |
| --- | --- | --- | --- | --- |
| OW [ms] | UE speed: 30/60/90/120 km/h |  | OW [ms] | UE speed: 30/60/90/120 km/h |
| 40 | 0.19/0.19/0.53/0.57 dBm |  | 200 | 0.23/0.23/0.70/0.76 dBm |
| 80 | 0.20/0.21/0.46/0.52 dBm |  | 400 | 0.30/0.32/0.70/0.78 dBm |
| 120 | 0.25/0.25/0.48/0.54 dBm |  | 600 | 0.33/0.38/0.73/0.81 dBm |
| 160 | 0.26/0.26/0.54/0.55 dBm |  | 800 | 0.48/0.50/0.80/0.89 dBm |
| 200 | 0.27/0.27/0.58/0.60 dBm |  | 1000 | 0.67/0.68/0.81/0.91 dBm |

| With L1 filtering, RRM sub use case 2 Measured frequency: 4 GHz, predicted frequency: 2 GHz |  |
| --- | --- |
| OW [ms] | UE speed: 30/60/90/120 km/h |
| 200 | 0.46/0.55/1.04/1.58 dBm |
| 400 | 1.01/1.53/2.28/2.83 dBm |
| 600 | 1.61/1.82/2.60/3.30 dBm |
| 800 | 1.75/1.90/2.61//3.51 dBm |
| 1000 | 1.80/1.92/2.53/3.40 dBm |

Based on the results it can be seen that the accuracy of inter-frequency measurement prediction is very good for all the simulated UE speeds and observation window lengths. The effect of L1 filtering for this scenario is not always positive and, in some cases, applying L1 filtering results in worse performance as compared to using non-filtered result. The following observations can be made:

Observation 6 (for Case 3): 1 With the OW equal to 200ms, the prediction accuracy with L1 filter applied is higher than in the case without L1 filtering for low UE speeds (30 and 60 km/h), while for higher UE speed (90 and 120 km/h), the prediction using non-filtered measurements performs better. This may be justified in the following way:

- For low UE speeds, since both input and output is L1 filtered RSRP, the previous measurement results will still be well correlated with the RSRP of the predicted cell.

- For high UE speeds, the measured results closer to the sampling timestamp are more correlated with the RSRP of the predicted cell than the samples more distant in time.

Observation 7 (for Case 3): With the increase of OW, the prediction accuracy declines for prediction using both filtered and non-filtered results. This may be justified by the fact that the current RSRP of the inter-frequency cell is best correlated with the latest measurements on the serving frequency cell.

Observation 8 (for Case 3): For RRM sub use case 1, the prediction accuracy is very good for all simulated UE speeds (RMSE<1 dBm), even though it declines with the increase of the UE speed.

Observation 9 (for Case 3): Results for RRM sub use case 1 are significantly better than the results using RRM sub-use case 2:

- Error range for sub-use case 1: 0.23~0.91 dBm

- Error range for sub-use case 2: 0.46~3.40 dBm

## 2.4 Case 4 (FR2 intra-frequency temporal domain case A)

Table 7: Simulation assumptions, settings and model information for Case 4

| Report parameters |  |  |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | See below |
|  | Prediction window (Note3) | See below |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 60000 |
|  | Testing | 1000 |
| AIML model input/output | Model input (Note 5) | RRM sub-case 1 and 2 |
|  | Model output(Note 6) | RRM sub-case 1 and 2 |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.04 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | See below |

Table 8: Simulation results for Case 4

| Without L1 filtering, RRM sub use case 1 PW=80ms |  |  |  | Without L1 filtering, RRM sub use case 1 PW=320ms |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OW [ms] | UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  |  | UE speed: 30/60/90/120 km/h |
| 80 | 0.42/0.58/0.80/0.99 dBm |  |  | 320 |  |  |  | 1.69/1.99/2.87/3.58 dBm |
| 160 | 0.41/0.47/0.81/0.89 dBm |  |  | 640 |  |  |  | 1.75/2.05/2.72/3.57 dBm |
| 240 | 0.44/0.45/0.84/0.93 dBm |  |  | 960 |  |  |  | 2.00/1.99/2.84/3.85 dBm |
| Without L1 filtering, RRM sub use case 1 PW=160ms |  |  |  | Without L1 filtering, RRM sub use case 1 PW=400ms |  |  |  |  |
| OW [ms] | UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  |  | UE speed: 30/60/90/120 km/h |
| 160 | 0.81/0.92/1.56/1.88 dBm |  |  | 400 |  |  |  | 2.11/2.28/3.11/4.41 dBm |
| 320 | 0.85/1.07/1.43/2.06 dBm |  |  | 800 |  |  |  | 2.36/2.40/3.22/4.37 dBm |
| 480 | 0.93/1.14/1.45/1.91 dBm |  |  | 1200 |  |  |  | 2.16/2.54/3.35/4.51 dBm |
| Without L1 filtering, RRM sub use case 1 PW=240ms |  |  |  | Without L1 filtering, RRM sub use case 1 PW=480ms |  |  |  |  |
| OW [ms] | UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  |  | UE speed: 30/60/90/120 km/h |
| 240 | 1.29/1.42/2.11/2.92 dBm |  |  | 480 |  |  |  | 2.35/2.85/3.79/5.19 dBm |
| 480 | 1.34/1.51/2.19/2.99 dBm |  |  | 960 |  |  |  | 2.65/2.93/3.93/4.88 dBm |
| 720 | 1.34/1.66/2.20/2.90 dBm |  |  | 1440 |  |  |  | 2.17/2.95/3.95/4.91 dBm |

| With L1 filtering, RRM sub use case 1 PW=400ms |  |  |  | With L1 filtering, RRM sub use case 2 PW=400ms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OW [ms] | speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  | speed: 30/60/90/120 km/h |
| 400 | 1.93/2.22/3.16/3.81 dBm |  |  | 400 |  |  | 1.91/2.08/2.98/3.72 dBm |
| 800 | 1.82/1.85/2.59/3.42 dBm |  |  | 800 |  |  | 1.31/1.72/2.35/3.01 dBm |
| 1200 | 2.18/2.20/2.58/3.57 dBm |  |  | 1200 |  |  | 1.40/1.79/2.39/2.96 dBm |
| With L1 filtering, RRM sub use case 1 PW=800ms |  |  |  | With L1 filtering, RRM sub use case 2 PW=800ms |  |  |  |
| OW [ms] | speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  | speed: 30/60/90/120 km/h |
| 800 | 3.62/3.63/4.83/6.68 dBm |  |  | 800 |  |  | 2.83/3.18/4.48/5.90 dBm |
| 1600 | 3.19/3.74/4.94/6.92 dBm |  |  | 1600 |  |  | 2.71/3.34/4.46/5.97 dBm |
| 2400 | 3.63/3.83/5.22/7.12 dBm |  |  | 2400 |  |  | 2.58/3.40/4.50/6.27 dBm |
| With L1 filtering, RRM sub use case 1 PW=1200ms |  |  |  | With L1 filtering, RRM sub use case 2 PW=1200ms |  |  |  |
| OW [ms] | speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  | speed: 30/60/90/120 km/h |
| 1200 | 4.56/4.85/6.79/8.08 dBm |  |  | 1200 |  |  | 4.11/4.42/6.02/7.94 dBm |
| 2400 | 4.43/4.88/6.52/8.86 dBm |  |  | 2400 |  |  | 3.75/4.44/6.08/8.01 dBm |
| 3600 | 4.50/4.92/6.61/8.71 dBm |  |  | 3600 |  |  | 3.25/4.47/6.30/8.02 dBm |

Based on the results it can be seen that the prediction accuracy for FR2 intra-frequency temporal domain case A depends on several factors, in particular: prediction window length, whether L1 filtering was applied or not and whether L1 measurements or L3 measurements were used as an input/output (i.e. RRM sub use case 1 versus RRM sub use case 2). The following observations can be made:

Observation 10 (for Case 4): Applying L1 filtering improves the prediction results as compared to using non-filtered measurements and prediction using RRM sub use case 2 performs better than using RRM sub use case 1, e.g. for PW=400ms:

- Error range for RRM sub-use case 1 without L1 filtering: 2.11~4.51 dBm

- Error range for RRM sub-use case 1 with L1 filtering: 1.93~3.81 dBm

- Error range for RRM sub-use case 2 with L1 filtering: 1.91~3.72 dBm

Observation 11 (for Case 4): With longer prediction window and higher speed, accuracy declines.

Observation 12 (for Case 4): The impact of observation window length depends on the used RRM sub use case and UE speed. Sometimes the results improve with extending the observation, but OW/PW ratios > 1 do not always bring benefits compared to ratio of OW/PW=1 and may even result in worse performance, especially for higher speeds.

Observation 13 (for Case 4): For prediction windows < 400ms it is usually possible to obtain accuracy error < 3dB. Higher prediction windows result in bigger errors.

## 2.5 Conclusions from simulations

Based on the results presented in this section, it can be seen that it is possible to obtain good accuracy of predicted measurement results under some conditions, i.e. with some specific combinations of UE speed and prediction window lengths (for temporal domain) even with 50-66% measurement reduction rate. However, what is still unclear is how the achieved accuracy impacts the performance of mobility, in particular handover failure rate. In order to verify this, RAN2 should perform system level simulations for some of the cases simulated during the initial phase of the study.

Proposal 1: RAN2 should perform system level simulations to verify an impact of measurement predictions inaccuracies on mobility.

We discuss the details of system level simulations in section 3.

# 3   System level simulations

## 3.1	Simulation assumptions

In RAN2#125bis meeting [1], an FFS was captured to discuss whether and what system level performance evaluation is needed. And in RAN2#126 meeting, it was agreed that system performance can be discussed after we see which scenarios have good measurements prediction accuracy. During RAN2#127 meeting, some initial discussion on this topic took place and companies were in general positive on running system level simulations. Also in our view such work is needed. As agreed previously and as captured in the draft TR 38.744 (see [1]), the goal of the study is twofold: measurement overhead reduction and HO performance improvement. For each of these goals, system level simulations are important:

- For the 1st study goal (measurement overhead reduction) – to understand how the prediction inaccuracies impact HO performance

- For the 2nd study goal (HO performance improvement) – to understand whether HO performance can be improved thanks to obtaining the measurement result in advance at the network side

Observation 14: The goal of SLS for RRM measurement prediction is twofold:

- For the 1st study goal (measurement overhead reduction) – to understand how the prediction inaccuracies impact HO performance

- For the 2nd study goal (HO performance improvement) – to understand whether HO performance can be improved thanks to obtaining the measurement result in advance at the network side

As observed in section 2, the accuracy of measurement prediction depends on several factors such as, e.g. UE speed, prediction window length or measurement reduction rate. Hence, to see how system level KPIs are impacted for different values of inaccuracy, it is important to choose scenarios/cases where different inaccuracy levels were obtained. At the same time, in order to limit the workload related to SLS, we should select only a number of representative cases. There is no need to run SLS for all the scenarios in case the inaccuracies for these scenarios are on the similar level as the impact on the mobility performance should be similar.

Observation 15: There is no need to run SLS for all the scenarios in case the inaccuracies for these scenarios are on the similar level as the impact on the mobility performance should be similar.

Based on the above, we believe it is sufficient to choose one scenario for the 1st study goal and one scenario for the 2nd study goal to run SLS.

Proposal 2: RAN2 to prioritize the following scenarios for SLS of RRM measurement prediction:

- SLS case 1: For the 1st study goal (measurement overhead reduction): Case 2 (FR1 to FR1 intra-frequency temporal domain case B) with UE speeds of 30, 60, 90 km/h.

- SLS case 2: For the 2nd study goal (HO performance improvement): Case 4 (FR2 to FR2 intra-frequency temporal domain case A) with UE speeds of 60, 90, 120 km/h.

For SLS case 1, even though it is defined as intra-frequency prediction case, it only makes sense to consider it for inter-frequency handover in the context of system level simulations. The goal is to reduce measurement overhead, hence the assumption is that the predicted cell is on a frequency different than the serving cell as only in this case the measurement overhead can be truly limited due to a reduced number/length of measurement gaps. Based on this RAN2 may select one frequency in FR1 range as the UE’s serving frequency, while another one in FR1 range as the UE’s target frequency (the one to be predicted), e.g. Serving_Freq=2 GHz and Target_freq=4 GHz. In order to simplify the simulations, we think there is no need to actually execute the handover. It is sufficient to just evaluate whether the conditions for handover are met and whether the failure happens or not afterwards. But at the same time, the UE can continue movement on its serving frequency and may be subject to further hypothetical handovers.

Proposal 3: In the simulations, handover is only hypothetical, i.e. after evaluating handover and HOF conditions, the UE is assumed to continue movement on its original frequency and may be subject to further hypothetical handovers.

Furthermore, RAN2 needs to also decide on the values of MRRT and OW/PW for simulations. There is no need to consider as many cases as for intermediate KPI evaluation, since what matters at the moment is to observe how different prediction accuracy values impact the mobility performance. Hence, RAN2 should analyse the results of RSRP prediction accuracy and select the cases allowing to achieve this goal. Based on our results as presented in section 2, we propose the following cases:

| Case 2: MRRT: 50%, with L1 filtering, RRM sub-use case 1 |  |  |
| --- | --- | --- |
| OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |
| 1000 | 200 | 0.18/1.27/2.10/2.98 |
|  | 600 | 0.56/1.52/3.33/4.58 |

| Case 4: With L1 filtering, RRM sub use case 2 |  |  |
| --- | --- | --- |
| OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |  |
| 400 | 400 | 1.91/2.08/2.98/3.72 |
| 800 | 400 | 1.31/1.72/2.35/3.01 |

| Case4: Without L1 filtering, RRM sub use case 1 PW=160ms |  |  |
| --- | --- | --- |
| OW [ms] | PW [ms] | RMSE [dBm] for different speeds: 30/60/90/120 km/h |
| 160 | 160 | 0.81/0.92/1.56/1.88 dBm |
| 320 | 160 | 0.85/1.07/1.43/2.06 dBm |

Lastly, in order to evaluate impact of predictions on handover performance, we need to agree on the handover modelling in our simulations. In our view, we can reuse the modelling from TR 36.839 [3]. In those simulations event A3 was used to determine handover and we can reuse the setting of the event which were proposed there.

Proposal 4: RAN2 to use the following A3 event settings for HO determination in the simulations:

- TTT: 40, 80, 160, 480 ms

- a3-offset: -1, 0, 1, 2, 3 dB

- Handover preparation (decision) delay: 50ms

- Handover execution time: 40ms

Lastly, since we will be relying on predicted measurement results now, we need to clarify how they are intended to be used for mobility event and/or handover determination by the UE/network. In particular, the UE may be able to know that the even is going to happen in advance and depending on the prediction window length used in the simulations, two main cases may happen: PW < TTT and PW >=TTT.

For the case where PW<TTT as shown in Fig. 1, the measurement results between T0 and T2 are actual measurement results while the measurement results between T2 and T1 are predicted measurement result. In this case the mobility event should be triggered the timing of T2, whereas the predicted measurement results and actual measurement results indicate event A3 to be met for the duration of TTT. In short, T2 is the time when handover is executed, i.e. handover preparation time is starting to run from this point in time.

For the case of PW>=TTT as shown in Fig. 2, the UE predicts that an event will be met for the duration of TTT even before the event conditions are met. However, it may be premature to trigger the HO already at this time as the neighbouring cell is not yet sufficiently better than the serving cell. Therefore, the UE/network should wait with the handover execution until the event conditions are actually met and are predicted to be met for the duration of TTT, i.e. the soonest time for handover execution should be T0.

Based on this analysis we propose to agree on the following with respect to the timing of handover execution using the predicted measurement results:

Proposal 5: During the simulations, the HO should be executed (i.e. handover preparation time should be started) when the following two conditions are met:

- A3 event entering condition was met according to actual measurements.

- A3 event is predicted to be met for the duration of TTT (counted from when the event entering condition was met).

Fig. 1: HO preparation for the case of PW<TTT

Fig. 2: HO preparation for the case of PW>=TTT

The table below summarizes the simulation assumptions according to the proposals above.

Proposal 6: RAN2 to apply the following simulation assumptions for SLS, in addition to the ones utilized for intermediate KPI evaluation:

|  | SLS Case 1: FR1 to FR1 intra-frequency temporal domain case B | SLS Case 2: FR2 to FR2 intra-frequency temporal domain case A |
| --- | --- | --- |
| Goal | Verify how RSRP prediction inaccuracies due to measurement reduction affect HO performance | Verify whether HO performance can be improved due to earlier HO event prediction |
| UE speeds | 30, 60, 90 km/h | 60, 90, 120 km/h |
| OW/PW | 1000/200 ms 1000/600 ms | 400/400 ms (RRM sub use case 2) 800/400 ms (RRM sub use case 2) 160/160 ms (RRM sub use case 1) 320/160 ms (RRM sub use case 1) |
| MRRT | 50% for target frequency, no reduction on serving frequency | N/A |
| Frequency layers | Inter-frequency HO: - Serving frequency: 2 GHz - Target Frequency: 4 GHz | Intra-frequency HO: Freq=30 GHz |
| HO determination | Non-AI case: HO preparation starts when A3 is met for TTT duration AI case: HO preparation starts when the following two conditions are met: - A3 event entering condition was met according to actual measurements. - A3 event is predicted to be met for the duration of TTT (counted from when the event entering condition was met). |  |
| A3 event settings | - TTT: 40, 80, 160, 480 ms - a3-offset: -1, 0, 1, 2, 3 dB - Handover preparation (decision) delay: 50ms - Handover execution time: 40ms |  |
| UE handling after hypothetical HO | UE stays on the same frequency and continues movement |  |
| UE handling after HO failure | UE removed from the simulation |  |

## 3.2	KPIs

Finally, RAN2 needs to also decide about the KPIs to consider for SLS. During mobility studies, usually the following ones are considered: handover failure rate, handover ping-pong rate and time of stay. As a baseline RAN2 can reuse the assumptions which were used in for mobility study for HetNet and are captured in TR 36.839 [3]. In this TR, the following three handover states were defined:

#### 5.2.1.1 Definition of Handover states For purpose of modelling, the handover procedure is divided into 3 states as shown in Figure 5.2.1.3.1. State 1: Before the event A3 entering condition, as defined in [5], is satisfied; State 2: After the event A3 entering condition, as defined in [5], is satisfied but before the handover command is successfully received by the UE; and State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE

Further, the handover failure was defined as follows:

#### 5.2.1.3 Handover/PDCCH failure modelling Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3. For calculating the handover failures for the two states: - In state 2: when the UE is attached to the source cell, a handover failure is counted if one of the following criteria is met: 1) Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure)1 or 2) RLF is declared in the state 2 - In state 3: after the UE is attached to the target cell a handover failure is counted if the following criterion is met: - target cell downlink filtered average (the filtering/averaging here is same as that used for starting T310) wideband CQI is less than the threshold Qout (-8 dB) at the end of the handover execution time (Table 5.1.4.1) in state 3.2 (…) Definition 4: The handover failure rate is defined as: Handover failure rate = (number of handover failures) / (Total number of handover attempts). The total number of handover attempts is defined as: Total number of handover attempts = number of handover failures + number of successful handovers. The number of handover failures is in Definition 3.

Another metric worth analysing is ping-pong rate. Handover decisions based on inaccurate predictions may result in unnecessary handovers which in turn may result in the handover back to the previous cell. Similarly, for SLS case 2, if the HO is triggered earlier, it may also lead to too early HO which in turn may lead to handover back to the previous cell. In TR 36.839, ping-pong and ping-pong rate is defined as follows:

### 5.2.2 Ping-pong Modelling The time that a UE stays connected with a cell after a handover is used as the metric to evaluate the ping-pong behaviour. The “Time of stay” in a cell A is the duration from when the UE successfully sends a “handover complete” (i.e. RRCConnectionReconfigurationComplete)-message to the cell A, to when the UE successfully sends a “handover complete” - message to cell B. The minimum time of stay connected with a cell models the time needed to allow a UE to establish a reliable connection with the cell, plus the time required for conducting efficient data transmission. If a UE makes a handover from cell B to cell A and then makes a handover back from cell A to cell B (i.e. the original source cell in the first handover), and the time connected to the cell A was less than the minimum-time-of-stay (MTS), it is considered as a ping-pong. In general, if the UE’s time-of-stay in a cell is less than MTS, the handover may be considered as an un-necessary handover. Definition 5: A handover from cell B to cell A then handover back to cell B is defined as a ping-pong if the time-of-stay connected in cell A is less than a pre-determined MTS. (…) Definition 6: Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

RAN2 can reuse the definitions from TR 36.839 to analyse how predicted RRM measurements impact them for the scenarios as proposed above.

Proposal 7: RAN2 to reuse the definitions of handover failure rate and ping-pong rate as defined in TR 36.839 and check how these KPIs are affected is the simulated scenarios while applying AIML based RRM measurements predictions as compared to non-AIML baseline.

Even though ping-pong rate is a valuable metric as excessive handovers lead to unnecessary overhead, the most important metric remains HOF rate as it is the one which directly translates into user experience. Therefore, we propose:

Proposal 8: RAN2 should focus on evaluating HOF rate for the simulated scenarios. Ping-pong rate evaluation is optional and lower priority.

# 4   Summary

Observations for RSRP prediction accuracy simulations

Observation 1 (for case 2): With the MRRT increasing, the prediction accuracy is declining.

Observation 2 (for case 2): With the UE speed increasing, the prediction accuracy is declining.

Observation 3 (for case 2): When L1 filtering is applied, the prediction accuracy increases.

Observation 4 (for case 2): With the increase of PW length, the accuracy is declining.

Observation 5 (for case 2): Increasing an OW beyond some point results in decreased prediction accuracy for higher UE speeds (>=60 km/h), while for low speed UEs the results for different OW values are comparable (for the simulated OW/PW set).

Observation 6 (for Case 3): 1 With the OW equal to 200ms, the prediction accuracy with L1 filter applied is higher than in the case without L1 filtering for low UE speeds (30 and 60 km/h), while for higher UE speed (90 and 120 km/h), the prediction using non-filtered measurements performs better. This may be justified in the following way:

- For low UE speeds, since both input and output is L1 filtered RSRP, the previous measurement results will still be well correlated with the RSRP of the predicted cell.

- For high UE speeds, the measured results closer to the sampling timestamp are more correlated with the RSRP of the predicted cell than the samples more distant in time.

Observation 7 (for Case 3): With the increase of OW, the prediction accuracy declines for prediction using both filtered and non-filtered results. This may be justified by the fact that the current RSRP of the inter-frequency cell is best correlated with the latest measurements on the serving frequency cell.

Observation 8 (for Case 3): For RRM sub use case 1, the prediction accuracy is very good for all simulated UE speeds (RMSE<1 dBm), even though it declines with the increase of the UE speed.

Observation 9 (for Case 3): Results for RRM sub use case 1 are significantly better than the results using RRM sub-use case 2:

- Error range for sub-use case 1: 0.23~0.91 dBm

- Error range for sub-use case 2: 0.46~3.40 dBm

Observation 10 (for Case 4): Applying L1 filtering improves the prediction results as compared to using non-filtered measurements and prediction using RRM sub use case 2 performs better than using RRM sub use case 1, e.g. for PW=400ms:

- Error range for RRM sub-use case 1 without L1 filtering: 2.11~4.51 dBm

- Error range for RRM sub-use case 1 with L1 filtering: 1.93~3.81 dBm

- Error range for RRM sub-use case 2 with L1 filtering: 1.91~3.72 dBm

Observation 11 (for Case 4): With longer prediction window and higher speed, accuracy declines.

Observation 12 (for Case 4): The impact of observation window length depends on the used RRM sub use case and UE speed. Sometimes the results improve with extending the observation, but OW/PW ratios > 1 do not always bring benefits compared to ratio of OW/PW=1 and may even result in worse performance, especially for higher speeds.

Observation 13 (for Case 4): For prediction windows < 400ms it is usually possible to obtain accuracy error < 3dB. Higher prediction windows result in bigger errors.

System level simulations

Observation 14: The goal of SLS for RRM measurement prediction is twofold:

- For the 1st study goal (measurement overhead reduction) – to understand how the prediction inaccuracies impact HO performance

- For the 2nd study goal (HO performance improvement) – to understand whether HO performance can be improved thanks to obtaining the measurement result in advance at the network side

Observation 15: There is no need to run SLS for all the scenarios in case the inaccuracies for these scenarios are on the similar level as the impact on the mobility performance should be similar.

Proposal 1: RAN2 should perform system level simulations to verify an impact of measurement predictions inaccuracies on mobility.

Proposal 2: RAN2 to prioritize the following scenarios for SLS of RRM measurement prediction:

- SLS case 1: For the 1st study goal (measurement overhead reduction): Case 2 (FR1 to FR1 intra-frequency temporal domain case B) with UE speeds of 30, 60, 90 km/h.

- SLS case 2: For the 2nd study goal (HO performance improvement): Case 4 (FR2 to FR2 intra-frequency temporal domain case A) with UE speeds of 60, 90, 120 km/h.

Proposal 3: In the simulations, handover is only hypothetical, i.e. after evaluating handover and HOF conditions, the UE is assumed to continue movement on its original frequency and may be subject to further hypothetical handovers.

Proposal 4: RAN2 to use the following A3 event settings for HO determination in the simulations:

- TTT: 40, 80, 160, 480 ms

- a3-offset: -1, 0, 1, 2, 3 dB

- Handover preparation (decision) delay: 50ms

- Handover execution time: 40ms

Proposal 5: During the simulations, the HO should be executed (i.e. handover preparation time should be started) when the following two conditions are met:

- A3 event entering condition was met according to actual measurements.

- A3 event is predicted to be met for the duration of TTT (counted from when the event entering condition was met).

Proposal 6: RAN2 to apply the following simulation assumptions for SLS, in addition to the ones utilized for intermediate KPI evaluation:

|  | SLS Case 1: FR1 to FR1 intra-frequency temporal domain case B | SLS Case 2: FR2 to FR2 intra-frequency temporal domain case A |
| --- | --- | --- |
| Goal | Verify how RSRP prediction inaccuracies due to measurement reduction affect HO performance | Verify whether HO performance can be improved due to earlier HO event prediction |
| UE speeds | 30, 60, 90 km/h | 60, 90, 120 km/h |
| OW/PW | 1000/200 ms 1000/600 ms | 400/400 ms (RRM sub use case 2) 800/400 ms (RRM sub use case 2) 160/160 ms (RRM sub use case 1) 320/160 ms (RRM sub use case 1) |
| MRRT | 50% for target frequency, no reduction on serving frequency | N/A |
| Frequency layers | Inter-frequency HO: - Serving frequency: 2 GHz - Target Frequency: 4 GHz | Intra-frequency HO: Freq=30 GHz |
| HO determination | Non-AI case: HO preparation starts when A3 is met for TTT duration AI case: HO preparation starts when the following two conditions are met: - A3 event entering condition was met according to actual measurements. - A3 event is predicted to be met for the duration of TTT (counted from when the event entering condition was met). |  |
| A3 event settings | - TTT: 40, 80, 160, 480 ms - a3-offset: -1, 0, 1, 2, 3 dB - Handover preparation (decision) delay: 50ms - Handover execution time: 40ms |  |
| UE handling after hypothetical HO | UE stays on the same frequency and continues movement |  |
| UE handling after HO failure | UE removed from the simulation |  |

Proposal 7: RAN2 to reuse the definitions of handover failure rate and ping-pong rate as defined in TR 36.839 and check how these KPIs are affected is the simulated scenarios while applying AIML based RRM measurements predictions as compared to non-AIML baseline.

Proposal 8: RAN2 should focus on evaluating HOF rate for the simulated scenarios. Ping-pong rate evaluation is optional and lower priority.

# References

- R2-2406309	Text proposal of 38.744	OPPO	draft TR

- R2-2406935	Simulation results for RRM measurement prediction	Huawei, HiSilicon

- 3GPP TR 36.839	Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 32 | FR1: 1 FR2: 4 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |

In addition, other simulation configurations used to generate the dataset are listed in Table 2, including the number of UEs, UE placement and dataset splitting method.

Table 2: Other simulation configurations

| Parameter | Value for FR1 and FR2 |
| --- | --- |
| UE number | 20 in total |
| UE placement | Uniform by cell |
| Dataset splitting | Data of 19 UEs used for training, data of the remaining UE used for testing |