3GPP TSG-RAN WG2 Meeting #127bis										    R2-2409206

Hefei, China, Oct 14th – 18th, 2024                                   revision of R2-2408021

Source: 	Xiaomi

Title:  	Remaining issues of L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

# Introduction

In RAN2#127 meeting, RAN2 agreed to use MAC CE to transmit the measurement result for the L1 event triggered measurement reporting. However, there are still many detailed issues (e.g. offset, TTT, report content and so on) for the L1 event triggered measurement reporting, which need to be discussed and clarified in RAN2. In this contribution, we provide our understandings and analysis on the remaining issues of the L1 event triggered measurement reporting procedure.

# Discussion

  - Offset

3GPP TS 38.331: Entering condition of measurement event Event 2: Ms + Hys < Thresh Event 3: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Event 4: Mn + Ofn + Ocn – Hys > Thresh Event 5: Mp + Hys < Thresh1 Mn + Ofn + Ocn – Hys > Thresh2

According to the L3 measurement events based on the cell-level measurement results, one event could have the following offset (as the example of Event A3 given above):

- Frequency/measurement object specific offset (e.g. Ofn/ Ofp)

- Cell specific offset (e.g. Ocn/ Ofp)

According to the RAN2#126 meeting discussion, RAN2 agreed to introduce the beam specific offset, and it is still FFS for cell specific offset. According to the RAN2#127 meeting agreement, the cell level measurement result is not be used for the L1 LTM measurement event. Since the gNB is able to control the L1 measurement report for each beam separately by using the beam specific offset and the L1 measurement event is triggered by beam-level measurement result, there is no need to introduce cell specific offset for event evaluation. Thus, the entering/leaving condition for LTM measurement event would be as follows:

Examples of entering/leaving condition of LTM measurement event: LTM Event 2: Entering condition: Ms + Hys < Thresh Leaving condition: Ms – Hys > Thresh LTM Event 3: Entering condition: Mn + Obn – Hys > Mp + Obp + Off Leaving condition: Mn + Obn + Hys < Mp + Obp + Off LTM Event 4: Entering condition: Mn + Obn – Hys > Thresh Leaving condition: Mn + Obn + Hys < Thresh LTM Event 5: Entering condition 1: Mp + Hys < Thresh1 Entering condition 2: Mn + Obn – Hys > Thresh2 Leaving condition 1: Mp – Hys > Thresh1 Leaving condition 2: Mn + Obn + Hys < Thresh2

Proposal 1: For LTM event evaluation, cell specific offset is not needed.

According to the example given above, the beam specific offset (including Obn of the beam specific offset of the candidate beam and Obp of the beam specific offset of the SpCell) will be used for LTM Event 3/4/5. According to the L3 measurement configuration, the cell specific offset can be configured independently for each cell including both the SpCell and the neighbouring cell(s). Thus, we think that the network can provide the beam specific offset for each beam, including both the serving beam and the candidate beams.

Proposal 2: The beam specific offset configuration includes the beam specific offset(s) for each candidate beam and each indicated beam of the SpCell.

  - TTT

In the RAN2#126 meeting, RAN2 agreed to introduce TTT for LTM event evaluation. Then, the details on how the TTT is used for evaluating the entering/leaving condition need to be clarified. According to the L3 measurement reporting procedure, when the TTT is used for the evaluation of the entering/leaving condition. Only when all the measurement results corresponding to a specific cell fulfil the entering condition during TTT, the UE would initiate the measurement reporting procedure.

For the L1 measurement event, the UE could have several candidate/serving beams configured for the measurement event evaluation. The measurement results from different beams could be quite different from each other. This means that during TTT, the UE could have one or more beams with the measurement result fulfilling the entering/leaving condition. From our understanding, the UE should only use the same beam result for evaluating the entering/leaving condition during TTT, so that the measurement event can be triggered to obtain a stable value of a specific-beam result.

Proposal 3: During TTT, the UE uses the measurement result(s) (as indicated by PHY) from the same beam for evaluating the entering/leaving condition.

As the L1 measurement result could change very dynamically and the PHY could provide measurement result to the RRC very frequently for each measurement resource in a slot, it is difficult to have all the measurement results during TTT fulfilling the entering/leaving condition. As such, we consider that the network can configure the maximum number of measurement results fulfilling the entering/leaving condition during TTT. When the number of measurement results fulfilling the entering conditions during TTT is equal to the configured threshold, the UE can initiate the measurement report. When the number of measurement results fulfilling the leaving conditions during TTT is equal to the configured threshold, the UE can remove the beam corresponding to the leaving condition from the measurement report.

Proposal 4: When the number of measurement results (as indicated by PHY) fulfilling the entering conditions during TTT is equal to a threshold configured by the network, the UE initiates the measurement report for the corresponding beam.

Proposal 5: When the number of measurement results (as indicated by PHY) fulfilling the leaving conditions during TTT is equal to a threshold configured by the network, the UE removes the corresponding beam from the measurement report.

  - Report content

Firstly, we think that the measurement result of the candidate beam triggering the measurement report should be reported, so that the LTM Cell Switch Command MAC CE is able to indicate the target serving beam of the candidate cell.

Proposal 6: The measurement result of the candidate beam triggering the LTM measurement event is reported.

With regards to other beams for the candidate cell corresponding to the candidate beam triggering the measurement report, we think that the UE should also be able to report the measurement results of other beams, so that the gNB is able to select another beam when the candidate TRP/beam triggering the LTM measurement event is congested. However, in order to ensure that the candidate beams reported has a good quality, the gNB can configure an RSRP threshold, and only the candidate beams with RSRP above the threshold can be reported.

Proposal 7: The network can configure the report of the measurement results of candidate beams other than the candidate beam triggering the LTM measurement event, for which the RSRP is above a configured threshold.

Furthermore, as the measurement results are used for a handover to specific candidate cell corresponding to the candidate beam triggering the LTM measurement event, the other beams reported and the candidate beam triggering the LTM measurement event reported should be from the same candidate cell.

Proposal 8: The candidate beams reported (including the candidate beam triggering the LTM measurement event and other candidate beams) are from the same candidate cell.

As the UE could be many candidate beams fulfilling the report condition (e.g. above the RSRP threshold), in order to control the reporting signalling, we think that the network can configure a maximum number of candidate beams which can be reported.

Proposal 9: The network can configure the maximum number of candidate beams which can be reported.

With regards to whether to report the measurement results of the beam(s) of the SpCell, we think that same as Rel-18, the gNB can provide a configuration to control whether to report the measurement results of the beam(s) of the SpCell. To control the report number and the report quality of the measurement results of the beam(s) of the SpCell, we think that same as the candidate beam reporting, the network can provide an RSRP threshold to control the report quality, and provide the maximum number of beams of the SpCell which can be reported.

Proposal 10: The network can configure the report of the measurement results of SpCell beams, for which the RSRP is above a configured threshold.

Proposal 11: The network can configure the maximum number of SpCell beams which can be reported.

As for the L3 cell-level measurement results, we think that the network can provide an extra configuration to control whether the L3 cell-level measurement results of either the SpCell and/or the candidate cell can be reported, so that the gNB can have more information (e.g. cell quality) to determine whether to handover the UE to a candidate cell.

Proposal 12: The report of the L3 cell-level measurement result of the SpCell can be configurable.

Proposal 13: The report of the L3 cell-level measurement result of the candidate cell can be configurable.

Since the gNB can configure multiple events for the same beam, the UE should report the event ID in the beam report to differentiate the measurement event used for the measurement report. As the measurement event can be associated with a specific report configuration, linking to a specific measurement resource. The UE can also report the report configuration ID instead.

Proposal 14: The LTM event ID (or report configuration ID) is reported.

## Report on leave

Regarding whether to support the measurement report when the leaving condition is satisfied, we think that the legacy procedure as used for the L3 measurement event procedure can be reused. We also consider that the “Report-on-Leave” function can be useful when the gNB wants to know the current measurement results of the measurement event or wants to adjust the configuration of the measurement event, so as to avoid to potential connection failure.

Proposal 15: The gNB can configure the UE to report the measurement result of a LTM measurement event when the leaving condition is satisfied, same as the legacy reportOnLeave configuration.

## s-Measure

As the LTM procedure is to change the serving cell to another candidate cell, when the quality of the serving SpCell is still good, it is not necessary for the UE to perform the measurement for the candidate beam. For the L3 measurement reporting procedure, s-MeasureConfig can be optionally configured to save the UE power, so that the UE only starts measuring the neighbouring cells when the serving cell quality is below a threshold configured by s-MeasureConfig. From our understanding, same mechanism as L3 s-MeasureConfig can be reused for even-triggered L1 measurement report.

Proposal 16: The UE performs the L1 measurement for the candidate beam(s), only when the RSRP of the SpCell is below the threshold configured by RRC, same as the legacy s-MeasureConfig.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: For LTM event evaluation, cell specific offset is not needed.

Proposal 2: The beam specific offset configuration includes the beam specific offset(s) for each candidate beam and each indicated beam of the SpCell.

Proposal 3: During TTT, the UE uses the measurement result(s) (as indicated by PHY) from the same beam for evaluating the entering/leaving condition.

Proposal 4: When the number of measurement results (as indicated by PHY) fulfilling the entering conditions during TTT is equal to a threshold configured by the network, the UE initiates the measurement report for the corresponding beam.

Proposal 5: When the number of measurement results (as indicated by PHY) fulfilling the leaving conditions during TTT is equal to a threshold configured by the network, the UE removes the corresponding beam from the measurement report.

Proposal 6: The measurement result of the candidate beam triggering the LTM measurement event is reported.

Proposal 7: The network can configure the report of the measurement results of candidate beams other than the candidate beam triggering the LTM measurement event, for which the RSRP is above a configured threshold.

Proposal 8: The candidate beams reported (including the candidate beam triggering the LTM measurement event and other candidate beams) are from the same candidate cell.

Proposal 9: The network can configure the maximum number of candidate beams which can be reported.

Proposal 10: The network can configure the report of the measurement results of SpCell beams, for which the RSRP is above a configured threshold.

Proposal 11: The network can configure the maximum number of SpCell beams which can be reported.

Proposal 12: The report of the L3 cell-level measurement result of the SpCell can be configurable.

Proposal 13: The report of the L3 cell-level measurement result of the candidate cell can be configurable.

Proposal 14: The LTM event ID (or report configuration ID) is reported.

Proposal 15: The gNB can configure the UE to report the measurement result of a LTM measurement event when the leaving condition is satisfied, same as the legacy reportOnLeave configuration.

Proposal 16: The UE performs the L1 measurement for the candidate beam(s), only when the RSRP of the SpCell is below the threshold configured by RRC, same as the legacy s-MeasureConfig.

# Reference

[1] RAN2#125 meeting minutes

[2] RAN2#126 meeting minutes

[3] RAN2#127 meeting minutes

# Annex

## RAN2#125 meeting agreements

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

## RAN2#126 meeting agreements

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

## RAN2#127 meeting agreements

Agreements on L1 MR event evaluation: - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. Agreements on L1 measurement reporting: - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.