3GPP TSG-RAN2 Meeting #127bis	R2-2409207

Hefei, China, Oct 14th – 18rd, 2024

Source: 			ZTE Corporation

Title: 	Evaluation on RRM measurement prediction

Agenda item:	8.3.2

Document for: 	Discussion and Decision

- Introduction

In the RAN2 126 meeting, the simulation scenario and the corresponding priority for RRM measurement prediction are defined [1]:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this document, we’d like to provide our simulation results and the corresponding analysis for the case with high priority.

- Discussion

## 2.1 General

In the post email discussion [POST127][030][AI mobility], companies reached a consensus that for temporal domain prediction case B, example 2 are recommended to be baseline. In the example 2, the measurement results from actual measurement within observation window is not continuous, as shown in Fig. 1:

Figure 1: Temporal domain case B example 2

For example 2, from simulation perspective, one key issue is how to perform L3 filtering in the case that old L3 filtered result is from prediction (e.g. at T3). For example, the input data of RRM sub use case 2 is L3 filtered cell results, while how to generate L3 cell results at T3, the following three options can be considered:

- Filtering option 1: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T2;

- Filtering option 2: L3 cell result at T3 is filtered based on the L1 filtered result at T3; in other words, not consider old L3 filtered result.

- Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement);

Observation 1: For temporal domain prediction case B, the following filtering scheme can be considered:

- Filtering option 1: L3 filtering is based on the L1 filtered result and the last L3 filtered cell result;

- Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction;

- Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

In the existing specification, the L3 filtering is based on the latest received measurement result and old filtered measurement result. Option 1 follows the same way. From the perspective of real implementation, it is feasible for the UE to take the AI model output in the previous predictions (i.e. filtered result) as AI model input of the following predictions. However, in the simulation, this requires lots of computing resources. Because all the prediction must be performed in order, and our simulation shows that compared with parallel prediction, the serial prediction requires more computing resources and consumes more simulation time. Besides, with option 1, the prediction errors will be accumulated. For example, if the cell result at T6 is not accurate due to prediction error, the filtered cell result at T7 is also inaccurate, since the filtered cell result at T7 is calculated based on the cell result at T6. Taking an inaccurate cell result as model input, the predicted result at T8 is also inaccurate.

Option 2 and Option 3 don’t have the problem of prediction error accumulation since all results are from actual measurement. And from the perspective of simulation, the required computing resources and simulation time are less than option 1. For option 2, only L1 filtering is performed at T3, so compared with option 1 and option 3, the channel quality of option 2 may change more quickly. For option 3, the measurement interval between current measurement result and adopted old measurement result (i.e. T3-T1) is longer than that with option 1 (T2-T1), so the channel correlation may be impacted. In our understanding, it is up to companies to select and report which filtering option is used for generating the input data in case of RRM sub use case 2.

Proposal 1: Companies can select and report which filtering option is used for generating the input data in case of RRM sub use case 2.

In our simulation, option 2 and option 3 are adopted considering the consumption of computing resources. Specifically, in the procedure of modelling training, the cell results used as model input takes filtering option 2 and option 3 above, while the cell result used as model output uses the ground true result (i.e. no measurement is reduced); and in the procedure of modelling inference, the cell result used as model input also uses filtering option 2 and filtering option 3, and the predicted result is compared with ground true value to calculate average RSRP difference.

In addition, in the post email discussion, we find that companies have different understandings for average RSRP difference in the case that there are multiple predicted results within prediction window. The possible understandings are as follows:

- Understanding1: the average RSRP is to average RSRP difference between all predicted results within prediction window and corresponding ground true cell results, and only one average RSRP difference is provided.

- Understanding 2: the average RSRP is to average RSRP difference between the cell result from the last prediction within prediction window and the ground true value, and only one average RSRP difference is provided

- Understanding 3: the average RSRP is to average RSRP difference between each predicted result and the corresponding ground true value separately, and multiple average RSRP differences are provided.

For example, in temporal domain prediction case A, the cell results at future two time instances are predicted by the historical measurement results. With understanding 1, the average RSRP is calculated for the first prediction and the second prediction together. With understanding 2, the average RSRP is calculated for the second prediction only. With understanding 3, the average RSRP is calculated for the first prediction and the second prediction, separately. The relationship between three understanding is that: average RSRP difference in the understanding 1 is the average of all average RSRP difference in the understanding 3, i.e. average the average RSRP difference for the first prediction and average RSRP difference for the second prediction. The average RSRP difference value with understanding 1 is smaller than that with understanding 2 and understanding 3. So, in order to better compare the simulation results among companies, we suggest to clarity how to calculate the average RSRP difference. Our understanding is that: With understanding 1, even if the average RSRP difference is acceptable, the prediction accuracy of last prediction, the second-to-last prediction may be un-acceptable, since the prediction accuracy will decrease with the increase of prediction window length. Both understanding 2 and understanding 3 are reasonable, and understanding 3 can provide more information. So, we prefer understanding 3.

Proposal 2: RAN2 to clarity which of the following understanding for average RSRP difference is correct:

- Understanding1: the average RSRP is to average RSRP difference between all predicted results within prediction window and corresponding ground true cell results, and only one average RSRP difference is provided.

- Understanding 2: the average RSRP is to average RSRP difference between the cell result from the last prediction within prediction window and the ground true value, and only one average RSRP difference is provided

- Understanding 3: the average RSRP is to average RSRP difference between each predicted result and the corresponding ground true value separately, and multiple average RSRP differences are provided.

Our preference is understanding 3.

Finally, as discussed in the post email discussions, all simulation results and the corresponding simulation assumptions are provided in the corresponding spreadsheet, which can be seen in the attachment. These simulation results need to be captured in the TR.

Proposal 3: Capture the simulation results in the spreadsheet provided in the attachment into TR.

## 2.2 Simulation results for FR2 to FR2 intra-frequency temporal domain case A

The motivation of case A is to improve handover performance via predicting the measurement results in the future. Specifically, the continuous measurement results in prediction window are predicted by continuous historical measurement results in observation window. In this section, we provide simulation results for temporal domain case A in difference scenarios, which can be seen in the table 1. The detailed simulation assumptions can be seen in the corresponding spreadsheet.

Table 1: simulation results for temporal domain case A

| RRM sub use case | Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- | --- |
| RRM sub use case 1 | 60 | [800ms, 1200ms] | 0.795, 1.361, 2.042 |
|  |  | [800ms, 800ms] | 0.810, 1.391 |
|  |  | [800ms, 400ms] | 0.758 |
|  |  | [1200ms,400ms] | 0.739 |
|  |  | [1600ms,400ms] | 0.746 |
|  | 120 | [800ms, 1200ms] | 1.109, 1.987, 2.920 |
|  |  | [800ms, 800ms] | 1.087, 1.945 |
|  |  | [800ms, 400ms] | 0.992 |
|  |  | [1200ms,400ms] | 0.961 |
|  |  | [1600ms,400ms] | 0.963 |
| RRM sub use case 2 | 60 | [800ms, 1200ms] | 0.665, 1.394, 2.178 |
|  |  | [800ms, 800ms] | 0.687, 1.400 |
|  |  | [800ms, 400ms] | 0.680 |
|  |  | [1200ms,400ms] | 0.672 |
|  |  | [1600ms,400ms] | 0.666 |
|  | 120 | [800ms, 1200ms] | 0.936, 2.059, 3.193 |
|  |  | [800ms, 800ms] | 0.941, 2.015 |
|  |  | [800ms, 400ms] | 0.946 |
|  |  | [1200ms,400ms] | 0.916 |
|  |  | [1600ms,400ms] | 0.914 |
| Note: xn in x1,…xn represents the average RSRP difference at time instance 400ms * n |  |  |  |

- The impact of prediction window length

Based on the simulation result above, we find that with same observation window length (i.e. 800ms), the prediction accuracy of temporal domain prediction case A is increased with the decrease of prediction window length. For example, in the case with RRM sub use case 1 and 120km/h UE speed, the average RSRP differences for cell results at future time instance 1200ms, 800ms and 400ms are 2.920dB, 1.945dB and 0.992 dB, respectively. This is because with the increase of prediction window length, the correlation between the historical channel and future channel becomes weaker, and the UE location change is more obvious, which degrades prediction performance.

Observation 2: For temporal domain prediction case A, with the same observation window length, the prediction accuracy is decreased with the increase of predicted window length.

- The impact of observation window length

In our simulation, we also studied the impact of observation window length on the prediction accuracy. We find that with RRM sub use case 1, as observation window length increases, the prediction accuracy increases first, then decreases. But the increase and decrease is very small, where the maximum decrease for average RSRP difference is only 0.031dB and the maximum increase for average RSRP difference is only 0.007dB. The observation window length with optimum prediction performance is 1200ms. For RRM sub use case 2, as observation window length increases, the prediction accuracy also improves, while the speed of increase slows down. For example, in the scenario with 120km/h UE speed and 400ms prediction window length, the average RSRP difference for temporal domain prediction with 800ms, 1200ms and 1600ms observation window length are 0.946dB, 0.916dB and 0.914dB, respectively. We can see that as the observation window length increases by the first 400ms (i.e. observation window 800ms 1200ms), the average RSRP difference decreases 0.3dB; while as observation window length increases by the second 400ms (i.e. observation window 1200ms  1600ms), the average RSRP difference decreases only 0.002dB.

Observation 3: For temporal domain prediction case A, with RRM sub use case 1, as observation window length increases, the prediction accuracy first increases and then decreases, while the increase and decrease are small.

Observation 4: For temporal domain prediction case A, with RRM sub use case 2, as observation window length increases, the prediction accuracy increases, while the speed of increase will slow down.

- The impact of RRM sub use case

Comparing the simulation results of RRM sub use case 1 and RRM sub use case 2, we can find that in some scenarios, RRM sub use case 1 can provide better prediction performance; while in other scenario, RRM sub use case 2 can provide better prediction performance. For example, in the case with 800ms observation window, 1200ms prediction window and 60km/h UE speed, the average RSRP difference with RRM sub use case 1 is 2.042dB, while the average RSRP difference with RRM sub use case 2 is 2.178dB, so RRM sub use case 1 can provide better prediction performance in this case; while in the case with 1600ms observation window length, 400ms prediction window length and 120km/h UE speed, the average RSRP difference with RRM sub use case 1 is 0.963dB, while the average RSRP difference with RRM sub use case 2 is 0.914dB, so RRM sub use case 2 can provide better prediction performance in this case. In our understanding, this may be because from the perspective of model inference, RRM sub use case 1 have multiple optimization goal (32 L1 filtered beam results as optimization goal), so the prediction accuracy may be worse than RRM sub use case 2. But the final metric is L3 filtered cell results, when performing L3 filtering, with RRM sub use case 1, the old filtered cell result is from actual measurement, which may reduce prediction error. In different scenario, the proportion of the two factors are different. So sometimes RRM sub use case 1 can provide better prediction accuracy; sometimes RRM sub use case 2 can provide better prediction accuracy. Based on the analysis above, we think no need to down-select among RRM sub use case 1 and RRM sub use case 2.

Observation 5: For temporal domain case A, there is no fixed conclusion that which of RRM sub use case 1 or 2 can provide better prediction accuracy.

Proposal 4: No need to down-select among RRM sub use case 1 and RRM sub use case 2.

- The impact of UE speed

As can be seen, in the same condition, the temporal domain prediction with 30 km/h UE speed can achieve better prediction accuracy than that with 90km/h. Compared with 30km/h UE speed, the average RSRP difference with 90km/h UE speed increases up to 1.015dB. This is consistent with our intuition, the channel condition in high speed scenario changes more rapidly, which makes measurement results prediction more difficult.

Observation 6: As UE speed decreases from 90km/h to 30km/h, the prediction accuracy of temporal domain measurement prediction Case A improves. The average RSRP difference decreases up to 1.015dB.

Proposal 5: Capture the observation 2~6 for temporal domain prediction case A in the TR.

## 2.3 Simulation results for FR1 to FR1 intra frequency temporal domain case B

In this section, we provide simulation results for temporal domain case B in difference scenarios, which can be seen in the table 2. The detailed simulation assumptions can be seen in the corresponding spreadsheet.

Table 2: Simulation results for temporal domain prediction case B

| MRRT | [observation window, prediction window] and detailed skipping pattern | RRM sub use case | ARSRP difference with UE speed = 30km/h | ARSRP difference with UE speed = 90km/h |
| --- | --- | --- | --- | --- |
| 50% | [600ms, 200ms] skipping pattern: 1:1 | RRM sub use case 1 | 0.502 | 0.693 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.522 | 0.673 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.521 | 0.701 |
|  | [1000ms,200ms] skipping pattern: 1:1 | RRM sub use case 1 | 0.498 | 0.679 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.508 | 0.668 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.508 | 0.669 |
|  | [400ms,400ms] skipping pattern: 2:2 | RRM sub use case 1 | 0.452, 0.592 | 0.618, 0.930 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.469, 0.611 | 0.607, 0.912 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.482, 0.644 | 0.658, 0.974 |
|  | [1200ms,400ms] skipping pattern: 2:2 | RRM sub use case 1 | 0.449, 0.579 | 0.595, 0.895 |
|  |  | RRM sub use case 2(filtering option 2) | 0.453, 0.609 | 0.599, 0.891 |
|  |  | RRM sub use case 2(filtering option 3) | 0.452, 0.598 | 0.598, 0.877 |
| 60% | [400ms,600ms] skipping pattern: 2:3 | RRM sub use case1 | 0.465, 0.612, 0.722 | 0.625, 0.917, 1.203 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.474, 0.623 0.742 | 0.601, 0.896, 1.195 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.499, 0.663, 0.788 | 0.666, 0.960, 1.234 |
|  | [1400ms,600ms] Skipping pattern: 2:3 | RRM sub use case 1 | 0.463, 0.589, 0.695 | 0.598, 0.844, 1.075 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.461, 0.613, 0.739 | 0.592, 0.879, 1.141 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.458, 0.603 0.712 | 0.590, 0.887, 1.125 |
| 66.7% | [800ms, 400ms] Skipping pattern: 1:2 | RRM sub use case 1 | 0.544, 0.674 | 0.779, 1.050 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.544, 0.692 | 0.700, 0.987 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.553, 0.709 | 0.725, 0.994 |
|  | [400ms, 800ms] skipping pattern: 2:4 | RRM sub use case1 | 0.464, 0.599, 0.708, 0.812 | 0.650, 0.953 1.231, 1.517 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.470, 0.613 0.727, 0.840 | 0.598, 0.908 1.208, 1.493 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.497, 0.648 0.771, 0.890 | 0.691, 0.983 1.256, 1.516 |
|  | [1600ms,800ms] skipping pattern: 2:4 | RRM sub use case 1 | 0.458, 0.583, 0.677, 0.772 | 0.616, 0.871, 1.105, 1.356 |
|  |  | RRM sub use case 2 (filtering option 2) | 0.471, 0.619, 0.733, 0.835 | 0.593, 0.887, 1.156, 1.407 |
|  |  | RRM sub use case 2 (filtering option 3) | 0.459, 0.604, 0.719, 0.826 | 0.592, 0.878, 1.130, 1.377 |

- The impact of MRRT on the prediction accuracy

As can be seen in the table 2, we find that in some cases, even the MRRT increases, the prediction accuracy also improves. For example, in the case that 60% MRRT, 400ms observation window and 600ms prediction window, the average RSRP difference with RRM sub use case 1 is 1.203dB; while in the case that 66.7% MRRT, 800ms observation window and 400ms prediction window, the average RSRP difference with RRM sub use case 1 is 1.050dB, whose prediction accuracy is better. This is because with different MRRT, the observation and prediction window length are different, which also impacts prediction accuracy. For the example above, the increase of MRRT will decrease the prediction accuracy; while the decrease of prediction window length will increase the prediction accuracy.

Observation 7: For temporal domain case B, the increase of MRRT may not result in the decrease of prediction accuracy due to the impact of prediction window length.

- The impact of skipping pattern on the prediction accuracy

In the post email discussion, companies reached a consensus that the companies need to report MRRT, observation window length and prediction window length when providing their simulation results, and detailed skipping pattern are optional to report. In this meeting, we study the impact of the skipping pattern on the prediction accuracy with the same MRRT. Based on the simulation results, we find that with even with the same MRRT, different skipping pattern results in different prediction accuracy. For instance, we simulate 3 skipping patterns with 66.7% MRRT, including [800ms, 400ms], [400ms, 800ms], and [1600ms, 800ms] case. And their average RSRP differences with RRM sub use case 1 and 90km/h UE speed are 1.050 dB, 1.517dB and 1.356dB, respectively. One possible reason is that different skipping pattern have different prediction window length and observation window length, which will impact prediction accuracy.

Observation 8: Regarding temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance.

- The impact of filtering

As mentioned above, filtering option 2 and option 3 are adopted in our simulation. And the simulation results show that the two filtering scheme provide similar prediction accuracy, the maximum difference between the average prediction error with the two filtering scheme in the same condition is less than 0.062dB.

Observation 9: Regarding temporal domain case B, filtering option 2 and filtering option 3 provide similar prediction performance.

- The impact of UE speed and RRM sub use case.

Finally, we can find that the impact of UE speed and RRM sub use case on the temporal domain prediction case B are the same as on the temporal domain prediction case A. So, observation 5 and 6 are also applicable for temporal domain prediction case B.

Observation 10: The observation 5~6 for temporal domain measurement prediction case B are applicable for temporal domain prediction case A.

Proposal 6: Capture the Observation 7~10 for temporal domain prediction case B in the TR.

## 2.4 Simulation results for FR1 to FR1 inter-frequency (frequency domain)

The simulation results for frequency domain prediction and the corresponding CDF curve is as follows:

Table 3: Simulation results for frequency domain prediction

|  | Inter-frequency shadow fading correction | ARSRP difference with UE speed = 30km/h | ARSRP difference with UE speed = 90km/h |
| --- | --- | --- | --- |
| 2G4G | full | 0.99 | 0.99 |
|  | no | 1.38 | 1.4 |
| 4G2G | full | 1 | 1 |
|  | no | 1.39 | 1.43 |

| - CDF for RSRP difference in the case of 30km/h and 2G4G prediction | - CDF for RSRP difference in the case of 90km/h and 2G4G prediction |
| --- | --- |
| (c) CDF for RSRP difference in the case of 30km/h and 4G2G prediction | (d) CDF for RSRP difference in the case of 90km/h and 4G2G prediction |

Figure 2: CDF curve for RSRP difference in the frequency domain prediction

Comparing the simulation results with full correlation and no correlation inter-freq shadowing fading, we find that no matter average RSRP difference value or RSRP difference CDF curve, the prediction accuracy of frequency domain prediction with full correlation inter-freq shadowing fading is better than that with no correlation model. This is because the uncertainty of shadowing fading brings additional difficulty for prediction. But even with no correlation inter-freq shadowing model, the maximum average RSRP difference is only 1.43dB, which is acceptable.

Observation 11: The prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation shadowing fading model.

Observation 12: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.43dB, which is acceptable.

Besides, we also find that the accuracy for prediction from low frequency to high frequency and the prediction from high frequency to low frequency are similar, and their maximum average RSRP difference between them is only 0.003dB. In addition, their CDF curve are almost the same.

Observation 13: Regarding frequency domain prediction, the accuracy for prediction from low frequency to high frequency and the prediction from high frequency to low frequency are similar

Proposal 7: Capture the observation 11~13 for frequency domain prediction in the TR.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1: For temporal domain prediction case B, the following filtering scheme can be considered:

- Filtering option 1: L3 filtering is based on the L1 filtered result and the last L3 filtered cell result;

- Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction;

- Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

- Temporal domain prediction case A:

Observation 2: For temporal domain prediction case A, with the same observation window length, the prediction accuracy is decreased with the increase of predicted window length.

Observation 3: For temporal domain prediction case A, with RRM sub use case 1, as observation window length increases, the prediction accuracy first increases and then decreases, while the increase and decrease are small.

Observation 4: For temporal domain prediction case A, with RRM sub use case 2, as observation window length increases, the prediction accuracy increases, while the speed of increase will slow down.

Observation 4: For temporal domain prediction case A, with RRM sub use case 2, as observation window length increases, the prediction accuracy increases, while the speed of increase will slow down.

Observation 5: For temporal domain case A, there is no fixed conclusion that which of RRM sub use case 1 or 2 can provide better prediction accuracy.

Observation 6: As UE speed decreases from 90km/h to 30km/h, the prediction accuracy of temporal domain measurement prediction Case A improves. The average RSRP difference decreases up to 1.015dB.

- Temporal domain prediction case B

Observation 7: For temporal domain case B, the increase of MRRT may not result in the decrease of prediction accuracy due to the impact of prediction window length.

Observation 8: Regarding temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance.

Observation 9: Regarding temporal domain case B, filtering option 2 and filtering option 3 provide similar prediction performance.

Observation 10: The observation 5~6 for temporal domain measurement prediction case B are applicable for temporal domain prediction case A.

- Frequency domain prediction

Observation 11: The prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation shadowing fading model.

Observation 12: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.43dB, which is acceptable.

Observation 13: Regarding frequency domain prediction, the accuracy for prediction from low frequency to high frequency and the prediction from high frequency to low frequency are similar

Proposal 1: Companies can select and report which filtering option is used for generating the input data in case of RRM sub use case 2.

Proposal 2: RAN2 to clarity which of the following understanding for average RSRP difference is correct:

- Understanding1: the average RSRP is to average RSRP difference between all predicted results within prediction window and corresponding ground true cell results, and only one average RSRP difference is provided.

- Understanding 2: the average RSRP is to average RSRP difference between the cell result from the last prediction within prediction window and the ground true value, and only one average RSRP difference is provided

- Understanding 3: the average RSRP is to average RSRP difference between each predicted result and the corresponding ground true value separately, and multiple average RSRP differences are provided.

Our preference is understanding 3.

Proposal 3: Capture the simulation results in the spreadsheet provided in the attachment into TR.

Proposal 4: No need to down-select among RRM sub use case 1 and RRM sub use case 2.

Proposal 5: Capture the observation 2~6 for temporal domain prediction case A in the TR.

Proposal 6: Capture the Observation 7~10 for temporal domain prediction case B in the TR.

Proposal 7: Capture the observation 11~13 for frequency domain prediction in the TR.

- References

- Report of 3GPP TSG RAN WG2 meeting #126