3GPP TSG-RAN2 Meeting #127bis	R2-2409240

# Hefei, China, 14 - 18 Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5076 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR NTN (R18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC, Sequans Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-10-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#124, it was agreed that the reference point for epoch time of neighbour cell is the serving cell ULTSRP and the reference point for epoch time of target cell is the target cell ULTSRP (based on R2-2313554). This is not clarified in the spec. Since the neighbour cell epoch time refers to the start of a DL subframe of the serving cell, and the orbital propagator needs the absolute time at the satellite for propagating the real-time satellite position, it should be clarified clearly in the spec that neighbour cell ephemeris is provided based on the UL time sync reference point of the serving satellite Similarly for target cell in HO and CHO, since the epoch time refers to the start of a DL subframe of the target cell, the reference point for epoch time is the UL time sync reference point of the target satellite. Impact analysis Impacted functionality: NTN Inter-operability: If the network is implemented according to the CR and the UE is not, or vice versa, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the UL time sync reference point for epoch time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether the UL time sync reference point of which satellite is used as the reference point for epoch time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

–	EpochTime

The IE EpochTime is used to indicate the epoch time for the NTN assistance information, and it is defined as the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for EpochTime of the serving or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell when this field is provided in an NTN cell and the gNB when this field is provided in a TN cell. In case of handover or conditional handover , the reference point for EpochTime of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell. In case of handover or conditional handover, EpochTime is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the target cell. Otherwise, EpochTime is used based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the serving cell.

EpochTime information element

-- ASN1START

-- TAG-EPOCHTIME-START

EpochTime-r17 ::=              SEQUENCE {

sfn-r17                        INTEGER(0..1023),

subFrameNR-r17                 INTEGER(0..9)

}

-- TAG-EPOCHTIME-STOP

-- ASN1STOP

EpochTime field descriptions

sfn For serving cell, it indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. For neighbour or target cell, it indicates the SFN nearest to the frame where the message indicating the epochTime is received.

End of Changes