3GPP TSG-RAN WG2 Meeting #127b	R2-2409264

# Heifei, China, 14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0037 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to error handling for U2U operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2024.10.14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.4, the description for error handling is too generic, and the some of the cases listed below are not covered: For U2U Remote UE: In current clause 5.4, the error handling covers the following two cases: 1. An SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList . 2. An SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or the 5 bits LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure. However, it can not cover the following case (case 3): 3. An SRAP Data PDU with SRAP header that contains UE ID fields which matches sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList, but the BEARER ID field does not match the concerned sidelink DRB for the paired Remote UE and Peer Remote UE. For U2U Relay UE: In current clause 5.4, the error handling covers the following three cases: 1. An SRAP Data PDU with SRAP header that contains a UE ID (for DST) field which does not match sl-PeerRemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList. 2. An SRAP Data PDU with SRAP header contains BEARER ID field which does not match 0/1/2/3 or the 5 bits LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure is received. 3. An SRAP Data PDU that contains a UE ID (for SRC) field which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link However, it can not cover the following case (case 4): 4. An SRAP Data PDU with SRAP header that contains UE ID fields which matches sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity in sl-LocalID-PairToAddModList, but BEARER ID field does not match the concerned sidelink DRB for the paired Remote UE and Peer Remote UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.4, modify the description for U2U operation, to cover the following cases: - For a U2U Remote UE, it shall discard the SRAP PDU with SRAP header that contains UE ID fields which matches sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList, but the BEARER ID field does not match the concerned sidelink DRB for the paired Remote UE and Peer Remote UE. - For a U2U Relay UE, it shall discard the SRAP PDU with SRAP header that contains UE ID fields which matches sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity in sl-LocalID-PairToAddModList, but BEARER ID field does not match the concerned sidelink DRB for the paired Remote UE and Peer Remote UE Impact analysis Impacted functionality: SRAP error handling Impacted architecture options: NR SA Inter-operability: - There are no inter-operability issues, considering this CR is only to clarify UE behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the change, the current error handling description cannot cover all the cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Start of Change-----------------------------------------------------------------

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Remote UE, if sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity are both configured, when an SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList or an SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or any of the 5 bits LSBs of slrb-PC5-ConfigIndex used on the end-to-end PC5 connection indicated by sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Relay UE, when an SRAP Data PDU with SRAP header that contains a UE ID (for DST) field which does not match sl-PeerRemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList is received, or when an SRAP Data PDU that contains a UE ID (for SRC) field which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link in any one of the entries in sl-LocalID-PairToAddModList with matched sl-PeerRemoteUE-LocalIdentity and sl-PeerRemoteUE-L2Identity is received, or an SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or any of the 5 bits LSBs of slrb-PC5-ConfigIndex used on the end-to-end PC5 connection indicated by sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity is received by U2U Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives an SRAP Data PDU with invalid or reserved values, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

-----------------------------------------------------------End of Change-------------------------------------------------------------------