3GPP TSG-RAN WG2 #127bis	R2-2409268

# Hefei, China, Oct 14th – 18th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.355 | CR | 0008 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of location time stamp, RSTD and RTOA report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | #1. There is no time stamp for the location information in CommonIEsProvideLocationInformation however the location information is always associated with time since location may change anytime. #2. There is no RSTD request in SL-TDOA-RequestLocationInformation which means RSTD is mandatory reported. To align with the RSTD report in LPP, RSTD should be reported as mandatory. #3. There is no RTOA request in SL-TOA-RequestLocationInformation which means RTOA is mandatory reported. To align with the RTOA report in LPP, RTOA should be reported as mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1. Add SL time stamp for the location information in CommonIEsProvideLocationInformation. #2. The description that UE always includes sl-RSTD-Result in this release. #3. The description that UE always includes sl-RTOA-Result in this release. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: SL Positioning Inter-operability: If UE supports the feature and NW does not - NW may not be able to decode the associated reported time stamp from UE - Network may not decode the measurement report correctly. If NW supports the CR and UE does not - UE will not report the timestamp assciated with the location information. - UE may not report RSTD in SL-TDOA method or RTOA in SL-TOA method. If one UE is implemented according to the CR and the other UE is not, - The other UE may not be able to decode the timestamp assciated with the location information. - the other UE may not decode the measurement report correctly. If one UE is not implemented according to the CR and the other UE is, - The first UE will not report the timestamp assciated with the location information. - The first UE may not report RSTD in SL-TDOA method or RTOA in SL-TOA method. Both the NW and UE if supporting SL positioning feature must support this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support the time stamp associated with location information nor RSTD/RTOA always report in SL-TDOA/SL-TOA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5, 6.9, 6.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2409255 |  |  |  |  |  |  |  |  |

START OF CHANGE

6.5	SLPP PDU Common Contents

–	SLPP-PDU-CommonContents

This ASN.1 segment is the start of the SLPP PDU Common Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-COMMONCONTENTS-START

SLPP-PDU-CommonContents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

NCGI,

NR-PhysCellID,

SL-TimeStamp

FROM

SLPP-PDU-Definitions;

-- TAG-SLPP-PDU-COMMON-CONTENTS-STOP

-- ASN1STOP

***********************************Skip the unchanged part***************************************

–	CommonIEsProvideLocationInformation

The IE CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information SLPP message Type.

-- ASN1START

-- TAG-COMMONIESPROVIDELOCATIONINFORMATION-START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate                        LocationCoordinates            OPTIONAL, -- locationTargetUe-sl-pos

rangeAndOrDirection                     RangeAndOrDirection            OPTIONAL,

velocityEstimate                        Velocity                       OPTIONAL,

relativeLocationEstimate                RelativeLocationCoordinates    OPTIONAL,

locationError                           LocationError                  OPTIONAL,

...,

[[

locationTimeStamp-r18                       SL-TimeStamp                       OPTIONAL

]]

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint                                      EllipsoidPoint,

ellipsoidPointWithUncertaintyCircle                 EllipsoidPointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse                EllipsoidPointWithUncertaintyEllipse,

polygon                                             Polygon,

ellipsoidPointWithAltitude                          EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid   EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc                                        EllipsoidArc

}

RelativeLocationCoordinates ::= CHOICE {

relative2D-LocationWithUncertaintyEllipse                                      Relative2D-LocationWithUncertaintyEllipse,

relative3D-LocationWithUncertaintyEllipsoid                                    Relative3D-LocationWithUncertaintyEllipsoid,

...

}

Relative2D-LocationWithUncertaintyEllipse ::=   SEQUENCE {

x                                               INTEGER (-134217728..134217727),   -- 28 bit field

y                                               INTEGER (-134217728..134217727),   -- 28 bit field

uncertaintySemiMajor                            INTEGER (0..255),

uncertaintySemiMinor                            INTEGER (0..255),

orientationMajorAxis                            INTEGER (0..179),

confidence                                      INTEGER (0..100)

}

Relative3D-LocationWithUncertaintyEllipsoid ::= SEQUENCE {

x                                               INTEGER (-134217728..134217727),     -- 28 bit field

y                                               INTEGER (-134217728..134217727),     -- 28 bit field

z                                               INTEGER (-16777216..16777215),       -- 25 bit field

uncertaintySemiMajor                            INTEGER (0..255),

uncertaintySemiMinor                            INTEGER (0..255),

orientationMajorAxis                            INTEGER (0..179),

uncertaintyAltitude                             INTEGER (0..255),

confidence                                      INTEGER (0..100)

}

Velocity ::= CHOICE {

horizontalVelocity                              HorizontalVelocity,

horizontalWithVerticalVelocity                  HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty               HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty    HorizontalWithVerticalVelocityAndUncertainty,

relativeVelocityWithUncertainty                 RelativeVelocityWithUncertainty

}

LocationError ::= SEQUENCE {

locationfailurecause        LocationFailureCause

}

LocationFailureCause ::= ENUMERATED { undefined, requestedMethodNotSupported, positionMethodFailure, periodicLocationMeasurementsNotAvailable}

EllipsoidPoint ::= SEQUENCE {

latitudeSign        ENUMERATED {north, south},

degreesLatitude     INTEGER (0..8388607),        -- 23 bit field

degreesLongitude    INTEGER (-8388608..8388607)  -- 24 bit field

}

EllipsoidPointWithUncertaintyCircle ::= SEQUENCE {

latitudeSign                             ENUMERATED {north, south},

degreesLatitude                          INTEGER (0..8388607),        -- 23 bit field

degreesLongitude                         INTEGER (-8388608..8388607), -- 24 bit field

uncertainty                              INTEGER (0..127)

}

EllipsoidPointWithUncertaintyEllipse ::= SEQUENCE {

latitudeSign                             ENUMERATED {north, south},

degreesLatitude                          INTEGER (0..8388607),        -- 23 bit field

degreesLongitude                         INTEGER (-8388608..8388607), -- 24 bit field

uncertaintySemiMajor                     INTEGER (0..127),

uncertaintySemiMinor                     INTEGER (0..127),

orientationMajorAxis                     INTEGER (0..179),

confidence                               INTEGER (0..100)

}

EllipsoidPointWithAltitude ::= SEQUENCE {

latitudeSign                   ENUMERATED {north, south},

degreesLatitude                INTEGER (0..8388607),        -- 23 bit field

degreesLongitude               INTEGER (-8388608..8388607), -- 24 bit field

altitudeDirection              ENUMERATED {height, depth},

altitude                       INTEGER (0..32767)           -- 15 bit field

}

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid ::= SEQUENCE {

latitudeSign                                          ENUMERATED {north, south},

degreesLatitude                                       INTEGER (0..8388607),        -- 23 bit field

degreesLongitude                                      INTEGER (-8388608..8388607), -- 24 bit field

altitudeDirection                                     ENUMERATED {height, depth},

altitude                                              INTEGER (0..32767),          -- 15 bit field

uncertaintySemiMajor                                  INTEGER (0..127),

uncertaintySemiMinor                                  INTEGER (0..127),

orientationMajorAxis                                  INTEGER (0..179),

uncertaintyAltitude                                   INTEGER (0..127),

confidence                                            INTEGER (0..100)

}

EllipsoidArc ::= SEQUENCE {

latitudeSign                ENUMERATED {north, south},

degreesLatitude             INTEGER (0..8388607),        -- 23 bit field

degreesLongitude            INTEGER (-8388608..8388607), -- 24 bit field

innerRadius                 INTEGER (0..65535),          -- 16 bit field,

uncertaintyRadius           INTEGER (0..127),

offsetAngle                 INTEGER (0..179),

includedAngle               INTEGER (0..179),

confidence                  INTEGER (0..100)

}

RangeAndOrDirection ::= SEQUENCE {

range                   Range       OPTIONAL,

azimuth                 Azimuth     OPTIONAL,

elevation               Elevation   OPTIONAL

}

Range ::= SEQUENCE {

rangeResult                  INTEGER (0..134217727),

uncertainty                  INTEGER (0..255),

confidence                   INTEGER (0..100)             OPTIONAL

}

Azimuth ::= SEQUENCE {

azimuthResult                INTEGER (0..3599),

uncertainty                  INTEGER (0..127),

confidence                   INTEGER (0..100)             OPTIONAL

}

Elevation ::= SEQUENCE {

elevationResult              INTEGER (0..1800),

uncertainty                  INTEGER (0..63),

confidence                   INTEGER (0..100)             OPTIONAL

}

HorizontalVelocity ::= SEQUENCE {

bearing               INTEGER(0..359),

horizontalSpeed       INTEGER(0..2047)

}

HorizontalWithVerticalVelocity ::= SEQUENCE {

bearing                            INTEGER(0..359),

horizontalSpeed                    INTEGER(0..2047),

verticalDirection                  ENUMERATED{upward, downward},

verticalSpeed                      INTEGER(0..255)

}

HorizontalVelocityWithUncertainty ::= SEQUENCE {

bearing                               INTEGER(0..359),

horizontalSpeed                       INTEGER(0..2047),

uncertaintySpeed                      INTEGER(0..255)

}

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing                                          INTEGER(0..359),

horizontalSpeed                                  INTEGER(0..2047),

verticalDirection                                ENUMERATED{upward, downward},

verticalSpeed                                    INTEGER(0..255),

horizontalUncertaintySpeed                       INTEGER(0..255),

verticalUncertaintySpeed                         INTEGER(0..255)

}

Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

PolygonPoints ::= SEQUENCE {

latitudeSign      ENUMERATED {north, south},

degreesLatitude   INTEGER (0..8388607),        -- 23 bit field

degreesLongitude  INTEGER (-8388608..8388607)  -- 24 bit field

}

RelativeVelocityWithUncertainty ::=        SEQUENCE {

radialVelocityComponent                    SEQUENCE {

unitsRadialVelocity                        ENUMERATED { mPerS, cmPerS, ... },

radialVelocity                             INTEGER (-2048..2047),

uncertaintyRadialVelocity                  INTEGER (0..255),

confidenceUncertaintyRadialVelocity        INTEGER (0..100)

}                                                                                             OPTIONAL,

transverseVelocityComponent                SEQUENCE {

unitsTransverseVelocity                    ENUMERATED { degPerSec1, degPerSec0-1, ... },

azimuth                                    SEQUENCE {

azimuthRateOfChange                        INTEGER (0..1023),

uncertaintyAzimuthRateOfChange             INTEGER (0..255),

confidenceUncertaintyAzimuthRateOfChange   INTEGER (0..100)

}                                                                                         OPTIONAL,

elevation                       SEQUENCE {

elevationRateOfChange                      INTEGER (0..1023),

uncertaintyElevationRateOfChange           INTEGER (0..255),

confidenceUncertaintyElevationRateOfChange INTEGER (0..100)

}                                                                                         OPTIONAL

}                                                                                             OPTIONAL,

...

}

-- TAG-COMMONIESPROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

CommonIEsProvideLocationInformation field descriptions

locationError This field shall be included if and only if a location estimate and measurements are not included in the SLPP PDU. The field includes information concerning the reason for the lack of location information. The enumerated value 'periodicLocationMeasurementsNotAvailable' shall be used by the UE if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

locationTimeStamp This field provides the time instance when the location information (e.g. locationEstimate and/or rangeAndOrDirection and/or velocityEstimate) is valid.

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [7]. Coding of the values of the various fields internal to each geographic shape follows the rules in TS 23.032 [7]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message.

rangeAndOrDirection This field provides a range and/or direction estimate as defined in TS 23.032 [7] for the "Range and Direction". - range provides the range component and comprises the following subfields: - rangeResult provides the range estimate in units of milli-metres, as defined in TS 23.032 [7] for the "Range and Direction". - uncertainty provides the uncertainty of the rangeResult and corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - confidence provides the confidence level for the uncertainty and corresponds to confidence as defined in TS 23.032 [7]. - azimuth provides the horizontal direction component and comprises the following subfields: - azimuthResult provides the horizontal direction (azimuth) as defined in TS 23.032 [7] for the "Range and Direction". Scale factor 0.1 degree; range 0 to 359.9 degrees. - uncertainty provides the single-sided uncertainty of the azimuthResult. Scale factor 1 degree; range 0 to 127 degrees. - confidence provides the confidence level for the uncertainty and corresponds to confidence as defined in TS 23.032 [7]. - elevation provides the vertical direction component and comprises the following subfields: - elevationResult provides the vertical direction (elevation) as defined in TS 23.032 [7] for the "Range and Direction". Scale factor 0.1 degree; range 0 to 180.0 degrees. - uncertainty provides the single-sided uncertainty of the elevationResult. Scale factor 1 degree; range 0 to 63 degrees. - confidence provides the confidence level for the uncertainty and corresponds to confidence as defined in TS 23.032 [7].

relativeLocationEstimate This field provides a relative location estimate. - relative2D-LocationWithUncertaintyEllipse provides the "relative 2D location with uncertainty ellipse" as defined in TS 23.032 [7] and comprises the following subfields: - x, y provides the value (in the unit of milli-metres) on x-axis and y-axis, respectively, of the relative location in the local cartesian system, as defined in TS 23.032 [7]. The origin of the cartesian system is the reference location of the relative positioning. - uncertaintySemiMajor provides the semi-major axis of the uncertainty ellipse. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - uncertaintySemiMinor provides the semi-minor axis of the uncertainty ellipse. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - orientationMajorAxis provides the orientation angle of the major axis as defined in TS 23.032 [7]. - confidence provides the confidence level for the uncertainty ellipse and corresponds to confidence as defined in TS 23.032 [7]. - relative3D-LocationWithUncertaintyEllipsoid provides the "relative 3D location with uncertainty ellipsoid" as defined in TS 23.032 [7] and comprises the following subfields: - x, y, z provides the value (in the unit of milli-meters) on x-axis, y-axis and z-axis, respectively, of the relative location in the local cartesian system, as defined in TS 23.032 [7]. The origin of the cartesian system is the reference location of the relative positioning. - uncertaintySemiMajor provides the semi-major axis of the uncertainty ellipsoid. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - uncertaintySemiMinor provides the semi-minor axis of the uncertainty ellipsoid. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - orientationMajorAxis provides the orientation angle of the major axis as defined in TS 23.032 [7]. - uncertaintyAltitude provides uncertainty altitude. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - confidence provides the confidence level for the uncertainty ellipsoid and corresponds to confidence as defined in TS 23.032 [7].

relativeVelocityWithUncertainty This field provides the relative velocity with uncertainty as defined in TS 23.032 [7] and comprises the following subfields: - radialVelocityComponent provides the radial velocity component characterised by a rate of change of range between the endpoint A and endpoint B: - unitsRadialVelocity provides the unit for the radialVelocity. Enumerated values 'mPerS' and 'cmPerS' indicate units m/s and cm/s, respectively. - radialVelocity provides the radial velocity as defined in TS 23.032 [7] in units given in the unitsRadialVelocity field. Positive values indicate increasing range between endpoint A and B; negative values indicate decreasing range between endpoint A and B. - uncertaintyRadialVelocity provides the (single-sided) uncertainty of the radialVelocity in increments of 1 the unit given in the unitsRadialVelocity field. - confidenceUncertaintyRadialVelocity provides the confidence of the uncertaintyRadialVelocity, as defined in TS 23.032 [7] for the "Confidence". - transverseVelocityComponent provides the transverse velocity component characterised by a rate of change of direction to the endpoint B from the endpoint A: - unitsTransverseVelocity provides the unit for the azimuth and elevation components. Enumerated values 'degPerSec1' and 'degPerSec0-1' indicate units 1-degree per second and 0.1 degree per second, respectively. - azimuthRateOfChange provides the rate of change of azimuth measured clockwise from North in a horizontal plane through the endpoint A as defined in TS 23.032 [7] in units given in the unitsTransverseVelocity field. - uncertaintyAzimuthRateOfChange provides the (single-sided) uncertainty of the azimuthRateOfChange in increments of 1 units given in the unitsTransverseVelocity field. - confidenceUncertaintyAzimuthRateOfChange provides the confidence of the uncertaintyAzimuthRateOfChange, as defined in TS 23.032 [7] for the "Confidence". - elevationRateOfChange provides the rate of change of elevation measured from Zenith in a vertical plane through the endpoint A and B, as defined in TS 23.032 [7] in units given in the unitsTransverseVelocity field. - uncertaintyElevationRateOfChange provides the (single-sided) uncertainty of the elevationRateOfChange in increments of 1 in units given in the unitsTransverseVelocity field. - confidenceUncertaintyElevationRateOfChange provides the confidence of the uncertaintyElevationRateOfChange, as defined in TS 23.032 [7] for the "Confidence".

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [7]. Coding of the values of the various fields internal to each velocity shape (except for relativeVelocityWithUncertainty) follows the rules in TS 23.032 [7].

NEXT CHANGE

6.9	SLPP PDU SL-TDOA Contents

***********************************Skip the unchanged part***************************************

–	SL-TDOA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-START

SL-TDOA-ProvideLocationInformation ::= SEQUENCE {

sl-RSTD-ReferenceUE-Info               SEQUENCE {

applicationLayerID                     OCTET STRING

}                                                                              OPTIONAL,

sl-TDOA-SignalMeasurementInformation   SL-TDOA-SignalMeasurementInformation    OPTIONAL,

sl-TDOA-Error                          SL-TDOA-LocationInformationError        OPTIONAL,

...

}

SL-TDOA-SignalMeasurementInformation ::= SEQUENCE {

sl-TDOA-MeasList                         SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-TDOA-MeasElementPerARP-ID-Rx,

...

}

SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement

SL-TDOA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-RSTD-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                                OPTIONAL,  -- sl-PRS-RSTD

sl-TDOA-AdditionalPathList            SL-TDOA-AdditionalPathList OPTIONAL,

sl-TimeStamp                          SL-TimeStamp               OPTIONAL,  -- sl-Timestamp

sl-TimingQuality                      SL-TimingQuality           OPTIONAL,  -- sl-TimingQuality

...

}

SL-TDOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TDOA-AdditionalPath

SL-TDOA-AdditionalPath  ::= SEQUENCE {

sl-RSTD-AdditionalPathResult          CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                               OPTIONAL,  -- additionalPath-SL-PRS-RSTD

sl-PRS-AdditionalPathRSRPP-Result      INTEGER (0..126)         OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                       SL-TimingQuality         OPTIONAL,  -- sl-TimingQuality

...

}

SL-TDOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-TDOA-MeasElement in IE SL-TDOA-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-TDOA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the sidelink PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TDOA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-RSTD-Result This field specifies the SL-RSTD measurement based on first path of arrival. In this release, this field is always included.

sl-RSTD-ReferenceUE-Info This field indicates reference UE information for SL-PRS based RSTD measurement report.

sl-TimeStamp This field specifies the time instance at which the SL RSTD and SL-PRS RSRP (if included) measurement is performed.

NEXT CHANGE

6.10	SLPP PDU SL-TOA Contents

***********************************Skip the unchanged part***************************************

–	SL-TOA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-START

SL-TOA-ProvideLocationInformation ::= SEQUENCE {

sl-TOA-SignalMeasurementInformation   SL-TOA-MeasElementPerARP-ID-Rx         OPTIONAL,

sl-TOA-Error                          SL-TOA-LocationInformationError        OPTIONAL,

...

}

SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TOA-MeasElement

SL-TOA-MeasElement ::= SEQUENCE {

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-RTOA-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                               OPTIONAL,  -- sl-PRS-RTOA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TOA-AdditionalPathList             SL-TOA-AdditionalPathList OPTIONAL,

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-TimingQuality                      SL-TimingQuality          OPTIONAL,  -- sl-TimingQuality

...

}

SL-TOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TOA-AdditionalPath

SL-TOA-AdditionalPath  ::= SEQUENCE {

sl-RTOA-AdditionalPathResult               CHOICE {

k0                                         INTEGER (0..16351),

k1                                         INTEGER (0..8176),

k2                                         INTEGER (0..4088),

k3                                         INTEGER (0..2044),

k4                                         INTEGER (0..1022),

k5                                         INTEGER (0..511)

}                                                                OPTIONAL,  -- additionalPath-SL-PRS-RTOA

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)      OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                           SL-TimingQuality      OPTIONAL,  -- sl-TimingQuality

...

}

SL-TOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TOA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-TOA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the PRS sidelink resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-RTOA-Result This field specifies the SL-RTOA measurement based on first path of arrival. In this release, this field is always included.

sl-TimeStamp This field specifies the time instance at which the SL RTOA and SL-PRS RSRP (if included) measurement is performed.

END OF CHANGE