3GPP TSG-RAN WG2 Meeting #127bis	R2-2409396

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.1.2.3

Source:	InterDigital Inc. (Rapporteur)

Title:	Offline on LCM for positioning

Document for:	Discussion

# 1. Introduction

In RAN2-127, LCM for UE-sided model for the beam management use case was discussed and the following agreements were made [1]:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signalling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

In this offline discussion, we will discuss on the high-level LCM procedure for positioning.

# 2. Discussion

The LCM procedure for the AIML positioning use case 1 was discussed and companies have agreed that the current LPP signaling can be used for the capability exchange as well as for inference configuration and applicability reporting.

Based on the comments during the discussion, the following steps have been agreed as a baseline:

The steps:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: The network sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

Proposals:

Proposal 1: The following procedures for LCM for UE sided model for AI positioning case 1 can be considered:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: The network sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

Proposal 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

Proposal 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

# 3. Conclusions

In this offline discussion, the LCM procedure for the positioning use case (specifically use case 1) were discussed and the following proposals are made:

Proposal 1: The following procedures for LCM for UE sided model for AI positioning case 1 can be considered:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: The network sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

Proposal 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

Proposal 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

# 4. References

[1]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[2]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[3]	RAN2-125bis Chairman Notes, Changsha, China, April 2024