3GPP TSG RAN WG2#127bis	R2-2407914

Hefei, China, 14th - 18th October 2024

3GPP TSG RAN WG1 #118	R1-2407438

Maastricht, NL, August 19th – 23rd, 2024

Title:	LS to RAN2 for on-demand SSB SCell operation

Release:	Release 19

Work Item:	Netw_Energy_NR_enh-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact person:	Seonwook Kim

sseonwook.kim@lgepartner.com

# 1	Overall description

RAN1 has discussed signaling mechanisms to indicate on-demand SSB transmission and has reached agreements to support indication by both RRC and MAC CE signalling.

Agreement (RAN1#118)

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement (RAN1#118)

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement (RAN1#116bis)

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

To provide more context for RAN2, the following are relevant RAN1 agreements on Cases and Scenarios for on-demand SSB SCell operation.

Agreement (RAN1#116)

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement (RAN1#116)

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement (RAN1#116bis)

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate on-demand SSB transmission for Scenarios #2 and #2A.

RAN1 is working on the further details needed for RRC and MAC CE based signaling to indicate on-demand SSB transmission and will inform RAN2 as further agreements are made.

# 2	Actions

To TSG RAN2

ACTION:	RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate on-demand SSB transmission for Scenarios #2 and #2A.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#118bis	14 - 18 Oct 2024	Hefei, CN

RAN1#119	18 - 22 Nov 2024	Orlando, US