3GPP TSG RAN WG2#127bis	R2-2407927

Hefei, China, 14th - 18th October 2024

# 3GPP TSG-RAN WG3 Meeting #125	R3-244844

# Maastricht, Netherlands, 19 - 23 Aug 2024

Title:	Response LS to SA2 on FS_XRM Ph2

Response to:	R3-244045/S2-2407351

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	RAN3

To:	SA2

Cc:	RAN2, SA4

Contact person:	Yansheng Liu

yansheng.liu@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA2 for the LS n FS_XRM Ph2(R3-244045/S2-2407351) and kindly asks SA2 to consider the following RAN3 responses.

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

[RAN3 answer to Question1]:

From RAN3 perspective, the control plane based solution providing periodicity per QoS flow as part of the TSCAI is sufficient for semi-static periodicity case. Other potential cases (e.g. dynamic case) need to be clarified by SA2, if any.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

[RAN3 answer to Question3]:

For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate.

RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting.

# 2	Actions

To SA2

ACTION: 	RAN3 kindly asks SA2 to take above answers into account and provide feedback if any.

# 3	Dates of next RAN3 meetings

RAN3#125bis	14th – 18th October 2024	Hefei, China

RAN3#126	18th - 22rd November 2024 	Orlando, US