3GPP TSG RAN WG2#127bis	R2-2409256

Hefei, China, 14 – 18 October 2024

Title:	LS on Additional Measurements for SL-TDOA and SL-TOA

Response to:	-

Release:	Rel-18

Work Items:	NR_pos_enh2

Source:	RAN WG2

To:	RAN WG1

CC:	RAN WG4

Contact Person:

#### Name:	Sven Fischer

E-mail Address:	sfischer@qti.qualcomm.com

Attachment:	-

1. Overall Description:

RAN2 observed that the LS in R2-2407909 (R1-2407387) with the latest RAN1 feature list for Rel-18 NR (R1-2407385) contains the following note:

For NR_pos_enh2, with regards to FGs 41-1-7a and FG 41-1-7b, RAN1 respectfully informs RAN2 that UE capability components have been specified, but there is no corresponding report specified in the ProvideLocationInformation message of TDOA and TOA methods.

However, there was no specific action for RAN2 other than “take into account the RAN1 NR UE features”. RAN2 discussed the above issue at RAN2#127bis and have the following questions for RAN1:

Question 1:	Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)?

Question 2:	If the answer to Question 1 is ‘yes’, what should the additional SL-TDOA/SL-TOA measurements comprise? Do the additional SL-TDOA/SL-TOA measurements include the SL-RSTD/SL-RTOA measurements only (as FGs 41-1-7a and FG 41-1-7b seems to imply), or can the additional SL-TDOA/SL-TOA measurements also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18),  LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)?

Question 3:	If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, should the encoding of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement)? I.e., analogous to LPP additional measurement reporting where the e.g., additional RSTD measurements are defined relative to the 1st/main RSTD measurement (e.g., nr-RSTD-ResultDiff in LPP). If so, what would be the value range of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements relative to the 1st/main SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement?

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to provide answers to the above questions.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG2 Meeting #128	November 18 to 22, 2024	Orlando, US

TSG-RAN WG2 Meeting #129	February 17 to 21, 2025	Athens, GR