3GPP TSG RAN WG2 #127bis                                                                              R2-2407948

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on the Random Access for Ambient IoT

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

This paper further discusses the remaining issues related to RACH for A-IoT. In details, the following issues are covered,

- Steps/content for 2-step/3-step CBRA

- Determination of access occasion

- Consideration on re-access

- Additional aspects related to RACH

Section 2 gives our discussion, and followed by Section 3 that summarizes the conclusion of this paper.

- Discussion

- Steps/content for 2-step/3-step CBRA

A-IoT Msg3 failure in 3-step CBRA

For the failure indication of A-IoT Msg3 in 3-step CBRA, it was further discussed in the post email discussion [1],

When the indication can be absent (i.e. implicit indication on the success): - Case 1: The reader has the subsequent R2D data to transmit for this device (e.g. command after inventory), i.e. - After D2R data transmission, if device receives its R2D data transmission, it considers the success of previous D2R data transmission by default. When the indication is needed: - Case 2: The reader has no more subsequent R2D data to transmit for this device (e.g. after the device sends feedback to the command), where we have several options: - Option 1: 1-bit indication with two code-points as “success” and “failure”; - Option 2: 1-bit indication for success indication (while its absence means failure); - Option 3: 1-bit indication for failure indication (while its absence means success); - Option 4: no AS feedback (success or failure) indication

For case 1, most companies accepted that the subsequent R2D data from the reader for this device (e.g. command after inventory) can be used to implicitly indicate the successful reception of A-IoT Msg3 by the reader. Companies had controversy on whether to need such explicit “success/failure” indication for A-IoT Msg3.

From our view, the multiple access mechanism is under RAN1 discussion, so even multiple devices are successfully decoded in A-IoT Msg1, the subsequent PDRCH transmission should be allocated with orthogonal resource to avoid interference. That is to say, the failure of Msg3 may occur only when the channel quality of PDRCH is bad enough, instead of conflicted resource.

Observation 1: The failure of A-IoT Msg3 transmission is mainly caused by the bad channel quality of PDRCH.

Nevertheless, this issue can also be mitigated by the physical layer repetition,

Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times For D2R, study at least block-level and bit-level repetition type 1 and type 2.

So from RAN2 perspective, we see no sufficient motivation to introduce Msg4 for Msg3 failure handling.

Observation 2: RAN1 supports the PDRCH repetition for AS reliability.

Even Msg3 failure occurs with physical repetition, it can be left to implementation, i.e., the CN can re-trigger the inventory or command service. By this way, the device does not need to be aware of Msg3 failure. Note that upon the post email discussion, we saw the objection on the CN implementation as some companies pointed out that the CN has no knowledge of correct full list of device ID. This may be the key point, as from our view the Rel-19 inventory or command services focus on the case where the AF has the full information for a certain service. For example, the AF initiates an inventory service, and it knows the whole device IDs that needs to be inventoried. Then it triggers CN to execute the inventory service and checks whether all these devices are found based on the feedback from CN.

Proposal 1a: RAN2 to align with the understanding that the CN/AF has the full knowledge of device information targeted for the inventory or command service.

Proposal 1b: Do not introduce explicit “success/failure” indication to handle the A-IoT Msg3 transmission failure. It/s up to CN implementation re-trigger the inventory or command service, if the CN/AF has full knowledge of device information.

Whether A-IoT Msg2 is always needed for 2-step CBRA

Regarding whether the A-IoT Msg2 is always needed for 2-step CBRA, the key point is whether we use a common procedure for inventory or command service. For example, some companies argued that,

Inventory only cases – device ID sent to reader and if you don’t receive it you can trigger the device to send the ID again. For command – it may be needed

If the A-IoT Msg2 is not needed in above mentioned case, whether the device needs to include the random number in A-IoT Msg1,  as the procedure indeed involves the contention resolution. From our view, we prefer an unified design for both 3-step CBRA and 2-step CBRA.

Proposal 2: A-IoT Msg2 is always needed for 2-step CBRA.

- Determination of access occasion

As per the current TR of 38.769, based on the previous agreements and the post email discussion, the modeling of RACH now is captured as follows,

Based on above modeling, the access occasion and access occasion set were defined as follows,

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device). Access occasion set: A set of access occasion(s) for difference A-IoT device(s), which is scheduled via the R2D message by the reader.

FDMA impact

With consideration of FDMA agreed by RAN1, one access occasion set includes multiple access occasions.

Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

For RACH procedure, the potential impact introduced by FDMA may be that the device needs to consider the frequency-shift to determine the corresponding access occasion within one selected access occasion set, which depends on further RAN1 discussion. From RAN2 perspective, the FDMA does not impact the detailed procedure of slot-ALOHA for A-IoT.

Proposal 3: Wait for RAN1 progress on how UE determines the access occasion within the selected access occasion set.

The trigger of access round

According to the post email discussion on RACH, there is the discussion on whether to use a separated msg from A-IoT paging to trigger the start of access round. Our view is that a separate R2D msg to trigger new round is more flexible, as there may be the situation where the A-IoT paging indicates the devices that need to response, but the access occasion is delayed until some time duration after the A-IoT paging. In this situation, separate round trigger msg helps the acquisition of timing information.

Proposal 4a: Introduce round-trigger msg to indicate the start of the access round.

Regarding the content of the round-trigger msg, it mainly depends on the way by which the device determines the access occasion. Since we had agreed to take slot-ALOHA as baseline, we can take the RFID for reference.

- Number of access occasions

In RFID, the device is aware of the number of access occasions (i.e., the access occasion sets) in one access round indicated by the reader. Since the access round is triggered by a new round-trigger msg as suggested by our Proposal 4a, the number of access occasions within one access round should be indicated by the round-trigger msg.

- Whether to join the current access round

In addition, the device needs to determine whether to join the current round upon reception of the round trigger msg. In RFID, the device generates a random number within the size [0 2^Q-1], where Q is indicated by the reader. If the device generates the value of 0, it will join the current access round. The similar principle can be applied for A-IoT.

- Other aspects

For other potential information such as frequency-shift related parameters (as mentioned by our Proposal 3) and dedicated parameters for re-access (as mentioned by our Proposal 7), we can wait further progress on these issues.

Proposal 4b: The number of access occasion sets (based on the current TR modeling) within one access round and the Q-like parameter for device to determine whether to join the current access round should be included in the round trigger msg. FFS others such as frequency-shift related parameters.

Determination of access occasion

On how to determine the access occasion, it seems straightforward to reuse the RFID principle, i.e., the device randomly selects a access occasion set if the device determines to join the current access round. Regarding on how to determine the specific access occasion within the access occasion set with consideration of FDMA (possibly based on the frequency-shift related parameters indicated in the round trigger msg), it needs more RAN1 involvement with the further progress on the design of FDMA.

Proposal 5: The device randomly selects a access occasion set if the device determines to join the current access round. FFS how to determine the access occasion within the access occasion set, pending on RAN1 design of FDMA.

- Consideration on re-access

Baseline for re-access

In the post email discussion, the candidate solutions on when to perform re-access are provided as follows,

- Option 1: In the same access occasion - Proponent companies may need to clarify whether the reader will extend additional sub-access occasions in this access occasion (something like “adding more sub-access occasions specific for re-access purpose”). - Option 2: In the following access occasion of the same access round - Proponent companies may need to clarify: - Option 2a: whether the reader will extend additional access occasions in this access round. (something like “adding more access occasions specific for re-access purpose”, i.e. adaptive length/number of access occasions of this access round), or - Option 2b: whether the device just re-accesses in the later already allocated access occasions, which were originally intended for the initial access of other devices. - Option 3: In the next access round - Option 4: In the next paging round - Option 5: the round length is adaptive. The round can be adjusted by increasing its length or terminating earlier upon detection of too high collision. More time occasions are added in current round or in the new round (if current one terminated)

Option 1 – The access occasion needs to be variable in time-domain to allow multiple access attempts (sub-access occasions), which depends on further RAN1 design;

Option 2 –  Option 2b may aggravate the burden for contention resolution in the subsequent access occasions, so Option 2a is a further enhancement for Option 2b, for example, define specific access occasions for re-access within one access round, which is indicated by the reader;

Option 3/4/5 –  These options aim to not allow the device to perform the re-access in the current round;

Since the current round has multiple access occasions for the device to use, no motivation is observed to delay the random access for a device to next round or next paging round. That is to say, at least the device to perform the re-access in the current round should not be excluded.

Proposal 6: The device should be allowed to perform re-access in the current access round.

On the other hand, if we aim to optimize the aggravation issue for Option 2b, the reader can indicate some specific access occasions within one round for re-access purpose, i.e., no need adaptive number of access occasions within one access round.

Proposal 7: The reader can indicate the specific access occasions within one round for re-access purpose.

Whether to support power ramping for A-IoT Msg1

In legacy, the RACH procedure supports power ramping for PRACH transmission. The UE performs the power ramping for Msg1 based on the local counter and the ramping step-size. For A-IoT, whether to support this feature should be further considered by RAN1, since there may be the situation where the device cannot meet the power requirement after ramping due to a limited energy storage. Fortunately, whether to have power ramping does not impact the procedure design for re-access, so it is suggested that RAN2 firstly design the re-access procedure without consideration of power ramping.

Proposal 8: RAN2 to firstly design the re-access procedure without consideration of power ramping for A-IoT Msg1.

- Additional aspects related to RACH

How to determine 2-step or 3-step CBRA

Upon the initialization of the CBRA, how to determine the 2-step or 3-step CBRA by the device needs to be discussed. In last meeting, two candidate solutions are mentioned from companies’ contributions, i.e., indicated by the reader or selected by the device based on a certain criteria.

In NR, the 2-step and 4-step CBRA are supported, and if the NW provides both 2-step and 4-step CBRA configurations in the UL BWP, UE will down-select one according to the criteria whether the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold,

1> else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources within the selected set of Random Access resources (as specified in clause 5.1.1b) and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or 1> … 2> set the RA_TYPE to 2-stepRA.

The reason is that the 2-step RACH in NR is suitable for better channel quality which facilitates the decoding of MsgA in the gNB.

When considering A-IoT, it seems not feasible to reuse above solution as the device is not required to perform L1/L3 measurement. Besides, even we introduce new criteria for the device to evaluate the RACH type, it will increase the reader decoding complexity as the reader does not know which type the device selects. This may be acceptable by gNB reader in Topology-1 due to enough ability, but not sure about UE reader in Topology-2.

From our view, since the reader can be aware of the approximate number of target devices that need to response the A-IoT paging msg, the reader can roughly evaluate the contention situation with joint consideration of the allocated access occasions by slot-ALOHA.

At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional

Thus, the reader can indicate whether to use 2-step RACH or 3-step RACH to the device.

Proposal 9: Whether to perform 2-step or 3-step CBRA is indicated by the reader. Do not support device-based solution to determine the RACH type.

Then another issue is how to indicate the RACH type to the device, i.e., whether the indication is included in the A-IoT paging msg and/or round-trigger msg.

It is straightforward that the A-IoT paging msg can include the RACH type indication so that the device knows whether to perform 2-step or 3-step CBRA in the corresponding access round by slot-ALOHA. However, the number of devices will reduce due to the successful access by part of devices in previous access occasions. Thus the reader may change the RACH type in the subsequent access round since it knows the approximately remaining number of devices that has not been successfully accessed.

Proposal 10: RAN2 to discuss whether the RACH type indication can be included in the A-IoT paging msg and/or the round-trigger msg.

Consideration on RACH Fallback for CBRA

In NR 2-step CBRA, upon receiving MsgB that matches the preamble index in MsgA, if the MsgB further contains fallback RAR, UE will use the UL grant provided in the fallback RAR to transmit Msg3, i.e., performing the RACH fallback from 2-step CBRA to 4-step CBRA. The intention is to let UE re-transmist Msg3 because there may be the case where the gNB only decodes PRACH successully, but not PUSCH.

Observation 3: In legacy, the fallback from 2-step CBRA to 4-step CBRA allows UE to re-tranmist Msg3 (i.e., PUSCH) because the MsgA content is carried over PRACH (i.e., preamble) and PUSCH (i.e., Msg3 in 4-step CBRA) separately.

When considering A-IoT, since we now support both 2-step CBRA and 3-step CBRA, then the following issues should be further discussed by RAN2,

- Whether to support RACH fallback for A-IoT;

- How to execute the fallback (if supported);

Whether to support RACH fallback for A-IoT

For A-IoT, upon performing re-access in the subsequent access occasion, the device will further colides with other devices allocated in the corresponding subsequent access occasion by slot-ALOHA. Note that it’s corner case that the other devices are not alloated within the access occasion selected by a certain device for performing re-access, since the slot-ALOHA aims to distribute the devices among all the access occasions homogeneously as much as possible. Even with specific access occasion for re-access (one candidate option for re-access in the post email discussion), there will be the situation where multiple devices share the same access occasion for re-access. That means the device may suffer multi-time access failures, which is power-consuming due to a large A-IoT Msg1 content of 2-step RACH and may cause the device to be re-charged after several access attempts in vain. From this perspective, the RACH fallback allows the device to perform re-access by using a small-size PDRCH, which facilitates the reader decoding and accelerates the contention resolution.

Proposal 11: RAN2 to discuss whether to support the RACH fallback from 2-step CBRA to 3-step CBRA.

How to execute the fallback

If we support RACH fallback for A-IoT, then how to execute the fallback should be further considered.

In NR 2-step CBRA, upon contention resolution failure, the re-transmission of MsgA allows UE to re-select the RACH resource including preamble and RO. Even a UE re-selects to the same RO with another UE, the preambles (for example, with different preamble index) used by two UEs may be orthogonal so that the gNB can still identify these two UEs. This allows the gNB to further send fallback RAR associated with the decoded preamble index to schedule Msg3, if the PUSCH is still colided.

While for A-IoT, the random number and the data in A-IoT Msg1 of 2-step CBRA are carried over PDRCH. From our view, the reader cannot do the partial decoding, thus if the device determines to perform RACH fallback, it should perform random access starting from the A-IoT Msg1 of 3-step CBRA.

Proposal 12: If the device performs RACH fallback from 2-step CBRA to 3-step CBRA, the device should perform random access by sending A-IoT Msg1 of 3-step CBRA.

Whether to allow parallel CBRA and CFRA among devices

In last meeting, it was proposed by the company contribution that for all devices that need to response the A-IoT paging msg, the reader can allocate the CFRA resource to part of these devices while the remaining devices without CFRA resource perform CBRA.

Note that whether to have this case depends on the use case for CFRA, i.e., whether it applies to single device case or multi-device case, pending on RAN1 further discussion. If the CFRA only applies to single device case, there is no issue here. If it can be applied to multi-device case, the reader needs to ensure that the other devices that perform CBRA should not select the access occasions allocated to specific devices. From our view, there is no need to do so. As per the SA1 use cases for the inventory or command service, there is no further requirement that part of devices have higher priority than the others on the access to the reader. Since anyway the device will perform re-access to the reader upon CBRA, no need to do such special handling for some specific devices.

Proposal 13: Do not support parallel CBRA and CFRA among devices within one service request.

- Conclusion

In this paper, we further discuss the RACH-related issues for A-IoT. The observations and proposals of this paper are summarized as follows,

Observation 1: The failure of A-IoT Msg3 transmission is mainly caused by the bad channel quality of PDRCH.

Observation 2: RAN1 supports the PDRCH repetition for AS reliability.

Observation 3: In legacy, the fallback from 2-step CBRA to 4-step CBRA allows UE to re-tranmist Msg3 (i.e., PUSCH) because the MsgA content is carried over PRACH (i.e., preamble) and PUSCH (i.e., Msg3 in 4-step CBRA) separately.

Steps/content for 2-step/3-step CBRA

Proposal 1a: RAN2 to align with the understanding that the CN/AF has the full knowledge of device information targeted for the inventory or command service.

Proposal 1b: Do not introduce explicit “success/failure” indication to handle the A-IoT Msg3 transmission failure. It/s up to CN implementation re-trigger the inventory or command service, if the CN/AF has full knowledge of device information.

Proposal 2: A-IoT Msg2 is always needed for 2-step CBRA.

Determination of access occasion

Proposal 3: Wait for RAN1 progress on how UE determines the access occasion within the selected access occasion set.

Proposal 4a: Introduce round-trigger msg to indicate the start of the access round.

Proposal 4b: The number of access occasion sets (based on the current TR modeling) within one access round and the Q-like parameter for device to determine whether to join the current access round should be included in the round trigger msg. FFS others such as frequency-shift related parameters.

Proposal 5: The device randomly selects a access occasion set if the device determines to join the current access round. FFS how to determine the access occasion within the access occasion set, pending on RAN1 design of FDMA.

Consideration on re-access

Proposal 6: The device should be allowed to perform re-access in the current access round.

Proposal 7: The reader can indicate the specific access occasions within one round for re-access purpose.

Proposal 8: RAN2 to firstly design the re-access procedure without consideration of power ramping for A-IoT Msg1.

Additional aspects related to RACH

Proposal 9: Whether to perform 2-step or 3-step CBRA is indicated by the reader. Do not support device-based solution to determine the RACH type.

Proposal 10: RAN2 to discuss whether the RACH type indication can be included in the A-IoT paging msg and/or the round-trigger msg.

Proposal 11: RAN2 to discuss whether to support the RACH fallback from 2-step CBRA to 3-step CBRA.

Proposal 12: If the device performs RACH fallback from 2-step CBRA to 3-step CBRA, the device should perform random access by sending A-IoT Msg1 of 3-step CBRA.

Proposal 13: Do not support parallel CBRA and CFRA among devices within one service request.

- References

[1] Report of [POST127][033][AIoT] Random Access