3GPP TSG-RAN WG2 Meeting #127bis	R2-2407949

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.5

Source: 	CATT

Title: 	Discussion on Topology 2 for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

In RAN2#127, the following agreements on topology 2 were achieved.

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

This contribution will continue discussing the identified issues.

2	Discussion

2.1	Resource allocation for the A-IoT interface

In RAN#127, it was agreed that the air resource used by A-IoT interface is under control of NW. This section will discuss detailed solutions on the resource allocation mechanism for the A-IoT interface.

According to the use case given by SA1 [1], different A-IoT service procedure may have different requirement on the A-IoT radio resource. For example, one inventory procedure is for a single device and another inventory procedure is for a group of devices, the latter needs more radio resource than the former. Hence, to avoid congestion or resource waste on the A-IoT interface, the resource allocated for each procedure should be suitable for the current service. For this purpose, three way forwards are given as following:

Option 1: UE reader request resource towards RAN node based on the requirement of different A-IoT services

After the UE reader receives inventory/command service request from CN/AF, it request the needed radio resource towards the RAN node based on the characteristic of this service, e.g., number of the target devices and service type. Then the RAN node allocates appropriate resource to the UE reader.

Option 2: RAN node acquires assistance info from CN and allocates appropriate resource to UE reader

When CN/AF sends an A-IoT service to UE reader, it also indicates assistance info towards RAN node (There will be impacts on RAN3). Based on the assistance info, the RAN node assigns appropriate resource to UE reader. Take CP solution as an example, the CN/AF could send the assistance info (e.g., number of target devices, service type) to RAN node within the N2 message which contains the NAS packet of inventory/command.

Option 3: RAN node configures resource pool via SIB, UE select resource from the pool as needed

The RAN node broadcast a common resource pool for A-IoT interface in SIB and the UE reader could select resource from the resource pool. How to select resource can depend on UE reader and the selected resource can be directly used by UE reader without confirmation from the RAN node.

All these options have pros and cons, option1 can assign resource more accurately, but brings additional Uu signalling overhead; option 2 has less Uu impact than option 1, but RAN node can only roughly estimate the required resources; option 3 has advantage on signalling overhead and applicable to all RRC states, but this option may cause collision and interference among UE readers (the inventory/command failure caused by collision and interference can be fixed by NW implementation, e.g., inventory/command for another time). RAN2 could further discuss and down select from these options.

Proposal 1: Study solutions on resource allocation mechanism for the A-IoT interface, e.g.,

- UE reader requests resource to RAN node, and the RAN node assign suitable resource for the UE reader;

- RAN node acquires assistance info from CN/AF and allocates suitable resource for the UE reader;

- RAN node broadcast resource pool in SIB and UE reader selects resource from the resource pool as needed.

The detailed radio resource configured for the A-IoT interface is still under discussion of RAN1, including dimensions of time domain, frequency domain and code domain. RAN2 can wait progress of RAN1.

Proposal 2: The detailed resource configuration of the new A-IoT interface depends on RAN1.

2.2	RRC state of UE

If the UE keeps in RRC_CONNECTED state during the A-IoT service procedure, the A-IoT service can be supported normally based on connection (including RRC, NAS and PDU session). Whether the UE could stay in RRC_IDLE or RRC_INACTIVE needs further consideration. The discussion will be carried with considering two scenarios, when A-IoT service request arrives from CN/AF and when UE needs forwarding the received A-IoT data towards CN/AF.

Based on existing mechanisms, the possible RAN behavior and UE reader behavior are listed in the following table.

| RRC state of UE | Scenario A: when A-IoT service request arrives from CN/AF | Scenario B: when UE needs forwarding the received A-IoT data towards CN/AF |
| --- | --- | --- |
| RRC_INACTIVE | The RAN pages the UE into RRC_CONNECTED state and sends the service request in RRC_CONNECTED; or RAN sends the A-IoT service request to UE via SDT procedure. | The UE resumes to RRC_CONNECTED firstly and finish data forwarding in RRC_CONNECTED; or UE sends the received device data via SDT procedure. |
| RRC_IDLE | The CN pages the UE into RRC_CONNECTED, and establish NAS connection and PDU session (for UP solution). | The UE setups RRC connection, NAS connection and establish PDU session (for UP solution) for A-IoT service, and forward the received device data towards CN/AF. |

For UE in RRC_INACTIVE, the A-IoT service can be performed normally under scenario A and scenario B based on legacy mechanisms, e.g., page UE enters RRC_CONNECTED or via SDT. For UE in RRC_IDLE, under scenario A, the NW could page the UE enters RRC_CONNECTED firstly as legacy, and then triggers A-IoT service. But problem may exist in scenario B. When the UE enters RRC_IDLE, the CN will release the A-IoT service context for the UE. After the UE collects data from A-IoT devices, it re-establishes NAS connection and/or PDU session to forward the received data towards CN/AF. Whether and how the CN/AF could recognize the response is for which A-IoT service before should be determined by SA2.

Observation 1: Whether RRC_IDLE can be supported when performing A-IoT service should base on SA2, that because if UE reader collects A-IoT device data in RRC_IDLE and forwards the data to CN/AF when enters RRC_CONNECTED, the CN/AF cannot recognize the response, for the connection is re-established and when UE enters RRC_IDLE the A-IoT context was released.

Proposal 3: Do not support the case that UE reader performs A-IoT service in RRC_IDLE state. An LS to SA2 is needed for confirmation and solution, if the case is wanted by RAN2.

Even if this problem is solvable by SA2, this procedure will bring significant signalling overhead for the UE, which is not benefit for power saving. From our perspective, it is expected that the UE reader keeps in RRC_CONNECTED/ RRC_INACTIVE state during ongoing A-IoT service. However, the RRC state of UE reader cannot be fully controlled by the network as expected, e.g., when UE detects RLF failure and fails to select suitable cell for re-establishment, the UE enters RRC_IDLE.

Observation 2: The UE reader may enter RRC_IDLE under abnormal conditions, e.g., the UE reader detects RLF on Uu and fails to select suitable cell for re-establishment.

Based on observation 1/2 and proposal 3, it is proposed that if the UE enters RRC_IDLE, the UE terminates the ongoing A-IoT procedures in both Uu and A-IoT interface.

Proposal 3a: When UE enters RRC_IDLE, the UE terminates the ongoing A-IoT procedures over both Uu and A-IoT interface.

2.3	Mobility aspects

In last meeting it was agreed to support in-coverage case and study temporarily out of coverage case caused by HO/RLF (and other similar operations). This section analyses the mobility impacts in topology 2.

1) Analysis on mobility impact when there is ongoing A-IoT procedure

The following two cases are taken into consideration. Case 1 and case 2 may occurs simultaneously or separately.

- Case 1: UE has ongoing procedures over the A-IoT interface, while HO/RLF/cell reselection occurs;

- Case 2: UE is performing data forwarding/signalling transmission for A-IoT device over the Uu interface, while HO/RLF/cell reselection occurs.

For case 1, if UE handover to target cell, or if RLF detected and UE selects a cell and re-establish successful, or if UE reselects to another cell, the A-IoT interface resource configured by source cell may become invalid. In this situation, to continue the A-IoT procedure, the UE needs new resources from the target cell. And during HO, how to handle the ongoing procedure over the A-IoT interface needs to be discussed.

Proposal 5: Discuss A-IoT interface radio resource management and how the UE reader handles the ongoing procedure over the A-IoT interface when the serving cell changes (e.g. HO ).

For case 2, if UE handover/re-establish/reselect to a new cell, the NW will perform path switch or context retrieve procedure for the UE following the current spec. By this way, the data forwarding or signalling transmission for A-IoT service can be continued by the target cell. No spec impact is identified so far.

Observation 3: When the service cell of UE reader changes, legacy mechanisms can support data forwarding/ signalling transmission for A-IoT service between UE reader and CN. No spec impact is identified so far.

If failure occurs during these procedures of case 1 or case 2, e.g., the UE fail to select a suitable cell after RLF declared and it enters RRC_IDLE, based on proposal 4, the UE terminates the ongoing A-IoT procedure over both A-IoT interface and Uu. Additional impact is not needed.

Observation 4: If failure occurs during HO/re-establish/cell reselection, the UE enters RRC_IDLE according to legacy mechanisms, the UE will terminates the ongoing A-IoT procedures based on proposal 4. Additional spec impact is not needed.

2) Impacts on UE (re-)selection mechanism

The RAN node in topology 2 should be able to allocate A-IoT radio resource for the UE reader, which is an optional capability in Rel-19. We could not assume all RAN nodes supports this capability. If UE camps on the cells which do not support A-IoT topology 2, the UE reader could not perform A-IoT related progresses. Hence, for a RRC_IDLE/ RRC_INACTIVE UE supports A-IoT topology 2, when it operating as a normal UE, there is no limitation on its mobility management progress, but when the UE operating as a reader, it should select a cell which supports A-IoT topology 2.

Proposal 6: The UE operating as a reader when in RRC_IDLE/RRC_INACTIVE state should camp on the cell which supports A-IoT topology 2.

With respect to how to indicate the cell supports A-IoT topology 2 to UE reader. One solution is to introduce an indication in system information, e.g., SIB1. The occurrence of the indication shows the cell supports A-IoT topology 2 related capabilities. The UE could determine whether the cell supports A-IoT topology 2 by reading system information.

Proposal 7: The RAN node should indicate to UE whether it supports A-IoT topology 2, e.g. via an indication in SIB1.

2.4	How the RAN node recognize an UE reader

Apart from CN, the RAN node also needs to know the UE supports the capability of operating as a reader. For example, if the RAN node knows the UE’s radio capability on the new A-IoT interface, it could configure suitable radio resource for the UE. Hence, it is proposed that UE supports A-IoT reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

Proposal 8: UE reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

2.5	UE reader selection

In practical needs, the inventory/command request raised by AF may be in format of geographical area. How to select suitable UE reader(s) and which network element make the decision need to be discussed. Reference to the solutions proposed in TR 23700-13 [2], the related mechanisms on reader selection are summarized as the following three kinds.

- AF raised inventory/command request towards AMF/AIoTF, the request include area information optionally. Then AMF/A-IoTF selects reader(s) for this service accordingly. Reference to solution #4, #6, #11, #21, #33.

- AF raised inventory/command request towards AMF/AIoTF, the request include area information optionally. AMF/A-IoTF selects some candidate reader(s) accordingly and sends the candidate reader(s) list to RAN node. The reader who is used to perform this A-IoT service is finally determined by RAN node. Reference to solution #30.

- AF determines reader directly based on the location of UE reader. Reference to solution #22.

After analysing the issue with reference to these candidate solutions, it can be seen that the decision can be totally depends on CN/AF and optional RAN node. The only requirement on UE reader is the NW needs to know UE reader’s location, which can be supported by legacy mechanisms, e.g., GNSS location report or 3GPP positioning feature. No additional RAN2 impact is identified so far.

Proposal 9: How to selected suitable UE reader(s) to perform A-IoT service depends on other groups, i.e. RAN3/SA2. RAN2 may further discuss the potential impact if there is within the progress of RAN3/SA2.

3	Conclusion

In this contribution, A-IoT topology 2 is discussed with giving corresponding observations and proposals:

Resource allocation for the A-IoT interface

Proposal 1: Study solutions on resource allocation mechanism for the A-IoT interface, e.g.,

- UE reader requests resource to RAN node, and the RAN node assign suitable resource for the UE reader;

- RAN node acquires assistance info from CN/AF and allocates suitable resource for the UE reader;

- RAN node broadcast resource pool in SIB and UE reader selects resource from the resource pool as needed.

Proposal 2: The detailed resource configuration of the new A-IoT interface depends on RAN1.

RRC state of UE

Observation 1: Whether RRC_IDLE can be supported when performing A-IoT service should base on SA2, that because if UE reader collects A-IoT device data in RRC_IDLE and forwards the data to CN/AF when enters RRC_CONNECTED, the CN/AF cannot recognize the response, for the connection is re-established and when UE enters RRC_IDLE the A-IoT context was released.

Proposal 3: Do not support the case that UE reader performs A-IoT service in RRC_IDLE state. An LS to SA2 is needed for confirmation and solution, if the case is wanted by RAN2.

Observation 2: The UE reader may enter RRC_IDLE under abnormal conditions, e.g., the UE reader detects RLF on Uu and fails to select suitable cell for re-establishment.

Proposal 3a: When UE enters RRC_IDLE, the UE terminates the ongoing A-IoT procedures over both Uu and A-IoT interface.

Mobility aspects

Proposal 5: Discuss A-IoT interface radio resource management and how the UE reader handles the ongoing procedure over the A-IoT interface when the serving cell changes (e.g. HO ).

Observation 3: When the service cell of UE reader changes, legacy mechanisms can support data forwarding/ signalling transmission for A-IoT service between UE reader and CN. No spec impact is identified so far.

Observation 4: If failure occurs during HO/re-establish/cell reselection, the UE enters RRC_IDLE according to legacy mechanisms, the UE will terminates the ongoing A-IoT procedures based on proposal 4. Additional spec impact is not needed.

Proposal 6: The UE operating as a reader when in RRC_IDLE/RRC_INACTIVE state should camp on the cell which supports A-IoT topology 2.

Proposal 7: The RAN node should indicate to UE whether it supports A-IoT topology 2, e.g. via an indication in SIB1.

How the RAN node recognize an UE reader

Proposal 8: UE reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

UE reader selection

Proposal 9: How to selected suitable UE reader(s) to perform A-IoT service depends on other groups, i.e. RAN3/SA2. RAN2 may further discuss the potential impact if there is within the progress of RAN3/SA2.

4	Reference

- 3GPP TR 22.840: "Study on Ambient power-enabled Internet of Things ".

- 3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things".

- 3GPP TR 38.331: "Radio Resource Control (RRC) protocol specification".