3GPP TSG-RAN WG2 Meeting #127bis                                                              R2-2407951																						xxx

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on other aspects for SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

The objectives of Sub-band full duplex (SBFD) which are related with RAN2 except random access can be found as below:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols,

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - UL resource muting for PUSCH[RAN1, RAN2, RAN4]

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework,[RAN1, RAN2, RAN4]

We will summarize the progress in RAN1 on SBFD configuration for UE transmission, reception and the new measurement in this contribution.

2. Discussion

RAN1 has reached the configuration of SBFD symbols within a TDD-UL-DL pattern period as below.

Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

RAN2 reached the Cell-specific configuration at #127 meeting, so the data structure of TDD-UL-DL-Pattern in RRC may be updated to follow the agreement in RAN1.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration).

Proposal 1: The detailed cell-specific configuration parameters (slot and symbol index) defined by RAN1 is captured by TDD-UL-DL-Pattern in RRC.

RAN1 has discussed the signal and channel-specific procedures. The configurations of PDSCH, CSI-RS, PUCCH, PUSCH will be impacted according to the conclusion in RAN1.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc. Agreement Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

Here is the summary of progress in RAN1 on these configurations.

| Channel | Required configurations in High layer | RAN1 progress |
| --- | --- | --- |
| ### Physical channel/signal across SBFD and non-SBFD symbols in same slot | ### New configurations for SBFD | ### On going |
| ### Physical channel/signal across SBFD and non-SBFD symbols in multiple slots | ### New configurations for SBFD | ### On going |
| ### SPS PDSCH | ### TBD | ### On going |
| ### CSI-RS | ### TBD | ### On going |
| ### PUCCH | ### Required | ### Separate configurations of startingPRB and secondHopPRB |
| ### PUSCH | ### Required | ### Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols. |
| ### PDCCH | ### Not required | ### Not applicable to PDCCH |

We can find that RAN1 have been discussing the detailed configurations step by step.

Proposal 2: The configurations of signal and channel on SBFD shall be sent to UE via RRC procedure. The detailed configurations rely on the design by RAN1.

RAN1 reached some agreement on L1 UE-to-UE CLI measurement and reporting at RAN1#118 meeting. New additional measurements have been defined by {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity.

Agreement For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity. - FFS: configuration of number of reported CLI resources in the reportConfig - FFS: reporting criteria

Observation 1: Two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} will be captured by the higher layer parameter reportQuantity.

Proposal 3: Wait for the LS from RAN1 on the design of UE-to-UE CLI measurement and reporting.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

Observation 1: Two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} will be captured by the higher layer parameter reportQuantity.

Proposal 1: The detailed cell-specific configuration parameters (slot and symbol index) defined by RAN1 is captured by TDD-UL-DL-Pattern in RRC.

Proposal 2: The configurations of signal and channel on SBFD shall be sent to UE via RRC procedure. The detailed configurations rely on the design by RAN1.

Proposal 3: Wait for the LS from RAN1 on the design of UE-to-UE CLI measurement and reporting.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- 3GPP TR 38.858 Study on Evolution of NR Duplex Operation (Release 18) V2.0.0 (2023-12)

- RAN1 Chair Notes WG1 #118