3GPP TSG-RAN WG2 Meeting #127bis	R2-2407961

Hefei, China, Oct. 14th – 18th, 2024

Agenda Item:	8.8.4

Source: 	CATT

Title: 	Discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

In RAN2#127 [1], the following agreements were achieved, where how to signal the intended service area and intended UE behaviour of MCCH acquisition are still left open:

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this contribution, we discuss these two remaining issues.

Discussion

Where to include the service area info: MCCH vs. SIB20

RAN2 has discussed two candidate solutions on where to contain the service area info, i.e. in MCCH vs. in SIB20. In the following texts, we provide analyses on why MCCH is a preferred option but SIB20 is not.

Why not SIB20?

SIB20 contains the information required to acquire the MCCH/MTCH configuration but it does not contain the MBS session related information directly. If the service area were to be included in SIB20, the MBS session ID would also need to be included to indicate the mapping between the service area and MBS session. Possible signalling structures are given in [2]. Based on these given structures, we have a rough calculation on the size of the current SIB20 and the required size for the service area and TMGI. The details can be founded in Annex A and Annex B.

On the one hand, according to the calculation in Annex A, the current SIB20 size can be as large as approximate 2684 bits already, and this has almost reaches the upper bound of SI message size, i.e., 2976 bits [3]. That is, there is not much space left in SIB20 to afford further information extension.

NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Observation 1a: The maximum SI message size is 2976 bits, and the size of the current SIB20 can be up to approximately 2684 bits already. That is, there is not much room (e.g. ~ 292 bits) left for SIB20 extension.

On the other hand, the bit consumption of a service area and of a TMGI, is shown in Annex B:

- One TMGI needs a space of 50 bits.

- One service area in format of reference location and radius needs a space of 64 bits.

- One service area in format of polygon needs a space of 144~720 bits.

In one cell, at most 1024 MBS sessions can be configured [3], and for one MBS session in NR NTN, at least one TMGI and one service area is needed. However, per the calculation in Annex A, the SI size limit for SIB20 has already been already exceeded, even if we consider only a few (e.g. 3 or 4) MBS sessions with their service areas are provided by a NR NTN cell, not even mentioning the typical cases that e.g. tens of MBS sessions are served (which requires even thousands of bits containing the service areas and corresponding TMGI to be added).

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

maxNrofMBS-Session-r17                  INTEGER ::= 1024    -- Maximum number of MBS sessions provided in MBS broadcast or multicast in

-- a cell

Observation 1b: Large space is needed to contain the service areas and corresponding TMGIs, with one MBS session needing at least one TMGI (50 bits) and additionally one or more service areas/service area indexes needed. These make current SIB20 too limit to include MBS sessions with the intended service areas for NR NTN (e.g. no more than 3 sessions) due to the SI size limitation.

Obviously, SIB20 has no enough space to contain the TMGI and service area information, we should exclude this option.

Why MCCH?

For the option of using MBSBroadcastConfiguration message on MCCH, considering the MBS session ID has already existed in this message, only a list of intended service areas needs to be indicated additionally for each MBS-SessionInfo-r17. This option will bring less signalling overhead. Additionally, there is no limitation on the size of MBSBroadcastConfiguration message, because of the RLC segmentation supported for the MCCH logical channel [4], and therefore we do not need to concern whether the MBSBroadcastConfiguration message have enough space to contain the service area info.

MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 MBS-SessionInfo-r17 ::= SEQUENCE { mbs-SessionId-r17 TMGI-r17, g-RNTI-r17 RNTI-Value, mrb-ListBroadcast-r17 MRB-ListBroadcast-r17, mtch-SchedulingInfo-r17 DRX-ConfigPTM-Index-r17 OPTIONAL, -- Need S mtch-NeighbourCell-r17 BIT STRING (SIZE(maxNeighCellMBS-r17)) OPTIONAL, -- Need S pdsch-ConfigIndex-r17 PDSCH-ConfigIndex-r17 OPTIONAL, -- Need S mtch-SSB-MappingWindowIndex-r17 MTCH-SSB-MappingWindowIndex-r17 OPTIONAL -- Cond MTCH-Mapping }

Observation 2a: If the service area info is contained in message MBSBroadcastConfiguration on the MCCH, the signalling overhead is less, because the MBS session ID has already existed. Also, there is no concern on whether this message has enough space for the service area info, since there is no limitation on MCCH message size due to RLC segmentation.

Using MBSBroadcastConfiguration message has also below merits, with respect to the association between service areas and MBS sessions. Firstly, by this way, the UE does not need to jointly consider the configurations from two different messages, compared with inclusion of service area in SIB20, and it is also more convenient for NW management and updates. Secondly, the location dependent MBS service [5] can be supported without additional ASN.1 impact. If the NTN cell provides services of more than one area sessions for the same MBS session, the NW could configure multiple entries that have the same mbs-SessionId but are associated with different service areas. Then, the UE can determine which entry's configuration to apply by comparing its location and the service area included in each entry. By this means, explicit indication of area session ID is not needed, which is aligned with the green highlight text shown as below in legacy.

### 6.2.3 Location dependent MBS service A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE.

Observation 2b: If the service area is added in the message MBSBroadcastConfiguration on the MCCH, the association between service area and MBS session is more concise and can support location dependent service without additional ASN.1 impact.

As per above analysis, we suggest to contain the service area in the message MBSBroadcastConfiguration on the MCCH, but not in SIB20. The service area is associated with each entry of the MBS-SessionInfoList.

Proposal 1: The service area information is provided via MBSBroadcastConfiguration message on the MCCH, and the service area (i.e. by a (set of) referenceLocation and radius or by a (set of) polygon(s)) is associated with each entry of the MBS-SessionInfoList).

How to avoid MCCH acquisition when out of service area

In RAN2#127, another leftover issue indicating that how to avoid UE (re)acquire up-to-date MCCH when it is not in any intended service area of its interested broadcast services.

RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

Firstly, it is worth mentioning that the problem of acquiring MCCH when outside service area also exists in legacy TN scenario. According to TS 38.331 [3] (marked in yellow in the citation box below), if UE interested to receive any MBS broadcast service and the cell is providing SIB20 for any MBS broadcast service, the UE shall apply MCCH acquisition procedure upon entering the cell providing SIB20. The intention is the UE could determine whether the cell is broadcasting the service it interested in by checking if the TMGI of this service is broadcast in MCCH.

Companies may wonder whether the UE could use the stored service area in USD to avoid MCCH acquisition when it is outside the service area of the MBS service it interested in. In TS 38.331 [3], only one NOTE is added for that (marked in pink in the citation box below). That is, in legacy MBS broadcast mechanism, how to addressed/alleviated this problem is up to UE implementation, relying on the service area in USD, rather than fully avoided relying on specified UE behaviour.

#### 5.9.2.2 Initiation A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s). NOTE 1: It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification. NOTE 2: It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

Observation 3: According to TS 38.331, the problem of acquiring MCCH when outside the service area also existed in legacy MBS broadcast mechanism and how to address/alleviate this problem is left to UE implementation relying on the service area in USD, rather than being avoided via specified UE behaviour.

For the MBS broadcast service in NTN scenario, a similar method can be used, i.e., UE could use stored service area to avoid acquiring MCCH when outside the service area. But different from legacy mechanism, the stored service area is acquired and stored by reading MCCH upon entering the cell, rather than acquired via higher layers and stored in USD. When UE enters an NTN cell, it reads the MCCH according to the current spec and acquires the service area of MBS session. Once the UE acquires the service area info in MCCH, it could store this information to determine whether to reacquire up-to-date MCCH when receiving MCCH change notification later, i.e., the UE is not required to acquire the MCCH when it is outside any service area of its interested services. How to use the stored service area to avoid acquiring MCCH, as illustrated above, can be up to UE implementation, and this does not need to be specified in normative text.

Proposal 2: It is up to UE implementation to use the stored service area info acquired in the MCCH to avoid acquiring the MCCH when the UE is outside the service area of the MBS broadcast service.

Conclusion

In this contribution, supportive of broadcast service via NR NTN is discussed with giving corresponding proposals:

Where to contain the service area info: MCCH vs SIB20

Observation 1a: The maximum SI message size is 2976 bits, and the size of the current SIB20 can be up to approximately 2684 bits already. That is, there is not much room (e.g. ~ 292 bits) left for SIB20 extension.

Observation 1b: Large space is needed to contain the service areas and corresponding TMGIs, with one MBS session needing at least one TMGI (50 bits) and additionally one or more service areas/service area indexes needed. These make current SIB20 too limit to include MBS sessions with the intended service areas for NR NTN (e.g. no more than 3 sessions) due to the SI size limitation.

Observation 2a: If the service area info is contained in message MBSBroadcastConfiguration on the MCCH, the signalling overhead is less, because the MBS session ID has already existed. Also, there is no concern on whether this message has enough space for the service area info, since there is no limitation on MCCH message size due to RLC segmentation.

Observation 2b: If the service area is added in the message MBSBroadcastConfiguration on the MCCH, the association between service area and MBS session is more concise and can support location dependent service without additional ASN.1 impact.

Proposal 1: The service area information is provided via MBSBroadcastConfiguration message on the MCCH, and the service area (i.e. by a (set of) referenceLocation and radius or by a (set of) polygon(s)) is associated with each entry of the MBS-SessionInfoList).

How to avoid MCCH acquisition when out of service area

Observation 3: According to TS 38.331, the problem of acquiring MCCH when outside the service area also existed in legacy MBS broadcast mechanism and how to address/alleviate this problem is left to UE implementation relying on the service area in USD, rather than being avoided via specified UE behaviour.

Proposal 2: It is up to UE implementation to use the stored service area info acquired in the MCCH to avoid acquiring the MCCH when the UE is outside the service area of the MBS broadcast service.

Reference

- R2-2407573	Report from Break-out session on NR-NTN and IoT-NTN 	Session Chair (ZTE Corporation)

- R2-2407619	Report of [AT127][307][R19 NR NTN] service area information	Apple	discussion	Rel-19	NR_NTN_Ph3-Core.

- 3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".

- 3GPP TS 38.300: " NR; NR and NG-RAN Overall Description; Stage 2"

- 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

- Annex A: Size of the current SIB20

NOTE: Only the necessary IEs in SIB20 which will be configured by NTN cell are counted below, i.e., cfr-ConfigMCCH-MTCH-RedCap-r18, mcch-ConfigRedCap-r18 and lte-CRS-ToMatchAround-r17 are not in scope.

-- SIB20

-- mcch-Config-r17

4+8+7+3+1+4=27 bits

-- cfr-ConfigMCCH-MTCH-r17

1 bit

-- locationAndBandwidthBroadcast-r17

1+1+max{1, 16}=18 bits

-- pdsch-ConfigMCCH-r17

1 bit

(10+1+16+1+2+1)*16=496 bits    -- pdschConfigList-r17

1+(6+1+1+7+3+1+7+1+3+1)*16=497 bits    -- pdsch-TimeDomainAllocationList-r17

1+(2+1+max{275+29+43+1+1, 4}+3+1+1+5)*4=1449 bits    -- rateMatchPatternToAddModList-r17

2+3=5 bits

2447 bits

--commonControlResourceSetExt-r17

1 bit

4+45+2+1+max{2+2+9+1,1}+1+7*6+1+7*6+1+2+17+1+4+3+2+2+2+3=189 bits

189 bits

2657 bits

2684 bits

- Annex B: IE size of service area and TMGI

1) TMGI (Refer to TS 38.331)

#### –	TMGI

The IE TMGI is used to identify the MBS session.

TMGI information element

-- ASN1START

-- TAG-TMGI-START

TMGI-r17 ::=                     SEQUENCE {

plmn-Id-r17                      CHOICE {   						--	1 bit

plmn-Index                       INTEGER (1..maxPLMN),		--	4 bits

explicitValue                    PLMN-Identity     			--	25 bits

},

serviceId-r17                    OCTET STRING (SIZE (3))  		--	24 bits

}

-- TAG-TMGI-STOP

-- ASN1STOP

1+25+24=50 bits

2) Reference location + radius (Refer to TS 38.331 and TS 37.355, take format of TN area as example)

CoverageAreaInfo-r18 ::=       SEQUENCE {

tn-AreaId-r18                  TN-AreaId-r18,

tn-ReferenceLocation-r18       ReferenceLocation-r17,  	--	48 bits

tn-DistanceRadius-r18          INTEGER(0..65535)    		--	16 bits

}

48+16=64 bits

#### –	ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

#### –	Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

3) Polygon (Refer to TS 37.355)

#### –	Polygon

The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

PolygonPoints ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

48*(3~15)=144~720 bits

NOTE that this is just a computation towards the existing SIB20, not having considered the service area related information yet.

This observation still holds even if the service area index as in [2] is included in SIB20 other than the service area itself, as the index can consume several extra bits (perhaps up to 10 bits) on top of the necessarily included TMGI.