3GPP TSG-RAN WG2 Meeting #127bis	R2-2407966

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.9.2

Source: 	CATT

Title: 	Discussion on RAN2 impacts due to the satellite ID list from MME in S&F operation

Document for: 	Discussion and Decision

Introduction

Based on the conclusion in TR 23700-29 [1], for both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs which identify the satellites the UE is allowed to communicate with, and the format of the Satellite ID aligns with the satellite ID indicated in the system information.

This contribution will discuss what information the eNB needs to provide in SIB with respect to the indicated satellite IDs by MME, and potential impact on the cell (re-)selection procedure.

Discussion

2.1	 How to use assistance info of satellite ID list from MME in AS

Considering the use case of the S&F operation, it is highly likely that the satellites indicated by MME are not the satellites which provide neighbour cell coverage (i.e. not the next satellite that provides the immediate neighbour cell coverage). Similar to the discontinuous coverage mechanism, if the UE could start/stop idle mode tasks related to S&F operation upon the indicated satellite coming and leaving, it is beneficial for UE power saving. In more detail, if the UE determines that it is out of coverage of any target satellite indicated by MME, the UE need not perform idle mode tasks (e.g. cell (re)selection, paging monitoring, etc.) related to S&F operation. Only when it is in coverage of a target satellite indicated by MME, shall the UE perform idle mode tasks related to S&F operation. How the UE determines whether it is in coverage or out of coverage of the indicated satellite(s) using satellite assistance information can be left up to UE implementation (similar to legacy discontinuous coverage mechinism).

Proposal 1a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" of a target satellite is up to UE implementation.

Proposal 1b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

2.2	 How to signal the satellite ID list from MME in Uu signalling

It is noticed by SA2 that the MME only provides Satellite IDs to UE for monitoring and accessing without the related satellite assistance information. So UE still needs to acquire the satellite assistance information corresponding to theses MME-indicated satellites from SIB broadcast by eNB. RAN2 needs to discuss what information the eNB should provide to UE for the indicated Satellite IDs.

Obviously, it would be helpful if eNB provides some information for UE to determine the serving time of these indicated satellites, so that unnecessary power consumption can be avoided. In Rel-17, SIB32 was introduced for the prediction of discontinuous coverage, where ephemeris info and footprint info are provided [2]. As mentioned above, as the satellites indicated by the MME to provide the subsequent service may not be the next satellite which provides the same coverage as the current satellite, the satellite assistance information needed for S&F operation is logically similar with that for discontinuous coverage, so that similar configuration can be used.

SatelliteInfo-r17 ::= SEQUENCE { satelliteId-r17 INTEGER (0..255), serviceInfo-r17 SEQUENCE { tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17 OPTIONAL, -- Need OR t-ServiceStart-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR }, footprintInfo-r17 SEQUENCE { referencePoint-r17 SEQUENCE { longitude-r17 INTEGER (-131072..131071), latitude-r17 INTEGER (-131072..131071) } OPTIONAL, -- Need OR elevationAngles-r17 SEQUENCE { elevationAngleRight-r17 INTEGER (-14..14), elevationAngleLeft-r17 INTEGER (-14..14) OPTIONAL -- Need OP } OPTIONAL, -- Need OR radius-r17 INTEGER (1..256) OPTIONAL -- Need OR } }

However, it is better to introduce a separate list specifically for S&F use, instead of reusing the existing list in SIB32 directly. That is because discontinuous coverage and S&F are corresponding to different functionalities and serving different purposes. For discontinuous coverage, the NW tends to indicate the first upcoming satellite which will provide same coverage of the current cell in future. It only cares about when the satellites could provide RRC connection to UE. But for S&F, the satellite indicated by MME is the satellite which may provide service (e.g. attach procedure) for the specific UE and is designated by the CN, and it may not be the adjacent future satellite which will provide coverage for the UE.

Besides, the UE behaviour for the operation towards these two kinds of satellites is different either. While in coverage of any satellites in SIB32, the UE shall perform idle mode tasks related to NTN. However, for the satellites indicated by MME for the S&F purpose, the UE needs to search for the specific satellites. Only in coverage of these specific satellites, does the UE need to perform related tasks. Based on the current procedure in [3], if current SIB32 were to be reused for S&F use, besides S&F operation, the UE might also perform discontinuous coverage mechanism, i.e., perform idle mode tasks related to NTN for all the satellites broadcast in SIB32, even if the satellite is not belongs to the Satellite ID list provided by MME. This will bring unnecessary power consumption.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

Furthermore, the maximum number of satellites for discontinuous coverage in SIB32 is 4. If it were directly reused to provide IDs of several satellites not limited to the next hop, this maximum number would be too limited.

Observation 1: If the SatelliteInfo-r17 in SIB32 were directly reused for S&F operation, the configured satellites might not meet the S&F requirement of the UE and might bring unnecessary power consumption for UE.

Hence, it is better to introduce a separate list to indicate the assistance information of the satellite(s) indicated by MME for S&F purpose. But the format of SatelliteInfo-r17 in SIB32 can be taken as the baseline for the signalling design.

Proposal 2: Introduce a separate list in system information to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the serving time of the satellites indicated by MME.

Proposal 3: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

2.3	 How cell (re)selection procedure is impacted by the satellites indicated by MME

For the satellites indicated by MME, there may be some stored DL signalling or data for the UE. The UE behaviours of receiving possible DL transmission are different.

- For split MME architecture, the UE should camp on the cells served by these satellites to perform paging monitoring, initiate RRC connection setup/resume procedure, etc.

- For full CN on-board architecture, the UE should perform attach procedure on the satellites indicated by MME, so that the satellite knows whether the UE is under coverage and determines whether to transmit DL signalling/data towards the UE.

For both of these two architectures, a common issue is how the UE selects the cells belonging to the indicated satellites by the MME. Even when the UE initiates cell (re)selection measurement based on the above satellite assistance information, it is also possible that satellites other than the MME-indicated ones are detected by the UE. In this case, if the existing cell (re-)selection mechanism based on RSRP/RSRQ is directly reused, UE may select a cell which is not served by a satellite indicated by the MME to camp on and thus miss the MT data or signalling from the indicated satellites.

Observation 2: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected). However, existing cell (re-)selection mechanism based on RSRP/RSRQ cannot ensure that.

RAN2 should consider some enhancements on cell (re-)selection with the consideration of the MME-indicated satellites introduced by SA2. For example, UE considers the frequencies served by the indicated satellites as highest priority. As another example, the UE should select one of the cells which belong to the indicated satellite by MME to camp on, taking precedence over the RSRP/RSRQ ranking.

Proposal 4: Enhance cell (re-)selection mechanism to ensure that UE (re-)selects a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the certain cell which belongs to the indicated satellite to camp on, etc.).

Conclusion

In this contribution, some potential issues on supporting S&F operation are discussed with corresponding observations and proposals listed as follows:

- How to use assistance info of satellite ID list from MME in AS

Proposal 1a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" of a target satellite is up to UE implementation.

Proposal 1b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

- How to signal the satellite ID list from MME in Uu signalling

Observation 1: If the SatelliteInfo-r17 in SIB32 were directly reused for S&F operation, the configured satellites might not meet the S&F requirement of the UE and might bring unnecessary power consumption for UE.

Proposal 2: Introduce a separate list in system information to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the serving time of the satellites indicated by MME.

Proposal 3: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

- How can the UE select the cells belonging to the satellites indicated by MME

Observation 2: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected). However, existing cell (re-)selection mechanism based on RSRP/RSRQ cannot ensure that.

Proposal 4: Enhance cell (re-)selection mechanism to ensure that UE (re-)selects a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the certain cell which belongs to the indicated satellite to camp on, etc.).

Reference

- 3GPP TR 23700-29: "Study on integration of satellite components in the 5G architecture ".

- TS 36.331

- TS 36.304