3GPP TSG-RAN2 Meeting # 127bis	R2-2407968

# Hefei, China, 14th - 18th Oct. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0903 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on coexistence between CHO and satellite switching with re-synchronization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#124, the following agreement was achieved when satellite switching with UE re-synchronization procedure was introduced in Rel-18 NR NTN. - When both CHO (for a different cell) and satellite switching procedure are configured, the UE initiates the procedure that triggers earlier; it's up to UE implementation if both procedures are triggered at the same time. However, this agreement has not been reflected in any specifications. Actually, when different mobility mechanisms are configured to a UE, how to deal with the co-existence are usually captured in Stage-2 specifications. As a typical example, when LTM was introduced in Rel-18, a similar agreement was reached for the co-existence of CHO and HO/LTM command, and is now captured in subclause 9.2.3.4.1 of TS 38.300: - Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration) or LTM cell switch command MAC CE, the UE executes the HO procedure as described in clause 9.2.3.2 or LTM cell switch procedure as described in clause 9.2.3.5, regardless of any previously received CHO configuration.. Hence, similar to the coexistence of other mobility mechanisms, the above agreement for the coexistence of CHO and satellite switching with UE re-synchronization procedure also needs to be reflected in the Stage-2 specification. |
| Summary of change: |  | In 16.14.3.2.3, capture above RAN2 agreement for the coexistence of CHO and satellite switching with UE re-synchronization procedure, when both are configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One RAN2 agreement on coexistence of CHO and satellite switching with UE re-synchronization procedure is missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

16.14.3.2.3	Satellite switch with re-synchronization

Upon both hard and soft satellite switch over in the quasi-Earth fixed scenario with the same SSB frequency and the same gNB, the satellite switch with re-synchronization procedure is supported. The satellite switch with re-sync avoids a L3 mobility for UEs in the cell by maintaining the same PCI on the geographical area covered by quasi-Earth fixed beam. CHO can be configured simultaneously with the satellite switch with re-sync procedure. When both CHO and satellite switch with re-synchronization procedure are configured, the UE initiates the procedure that triggers earlier and it is up to UE implementation if both procedures are triggered at the same time.

For soft satellite switch over, the UE can start synchronizing with the target satellite before the source satellite ends to serve the cell. It is not required for the UE to be connected to source satellite when the UE switches to target satellite.

For hard satellite switch over, the UE can only start synchronizing with the target satellite after the switch to the target satellite is initiated.

For the re-synchronization to the target satellite, random access can be triggered by a PDCCH order via the target satellite.

CHANGE END