3GPP TSG-RAN2 Meeting #127bis	R2-2407973

# Hefei, China, 14th October 2024 – 18th October 2024

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES, for which R2#127 agreed that

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

- Discussion

2.1 Issue-1: OD-SSB Activation/Deactivation

In 127 meeting, there was an FFS point on whether to allow the RRC signaling to reconfigure the non-initial (de)activation state of OD-SSB.

On the one hand, it is already sufficient to support scenario-2/2A by limiting RRC signaling to initial state configuration, since

- For scenario-2, the OD-SSB can be activated before SCell activation, via either RRC-based initial state setting or MAC-CE-based activation (before SCell activation afterwards);

- For scenario-2A, the OD-SSB and SCell can be activated simultaneously via either RRC-based initial state setting or MAC-CE (e.g., the two OD-SSB and SCell activation MAC-CE sent together);

On the other hand, one argument against the initial state setting was whether the R1 conclusion meant RRC signaling can also reconfigure the OD-SSB non-initial state.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

So, it seems R1 only concluded on the usage of RRC signaling for the case where RRC signaling being used for SCell activation, which for now is limited to initial SCell (de)activation state setting. Thus, it is limited to Scenario-2A RRC-based approach, while other cases are FFS, i.e., no explicit R1 conclusion to cover the non-initial state setting.

While the RRC configuration + dynamic MAC-CE control is very commonly used, for (de)activation of SCell, duplication, NES, in order to benefit from the dynamic control via MAC-CE after the initial RRC reconfiguration. So from our perspective, it would be helpful if R2 can clarify more on this detailed aspect.

- RRC based OD-SSB transmission indication is used to indicate only the initial activation/deactivation state of OD-SSB configuration, but not the non-initial state.

For the MAC-CE design: It is hard to go into detailed design of the MAC-CE, since R1 has not yet concluded on the content of the MAC-CE design.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

On the other hand, for scenario-2A, there was another alternative, i.e., to design another MAC-CE including functionality of both OD-SSB and SCell (de)activation, with the argument that separate MAC-CE may be sent in different MAC PDUs and thus fail to enable scenario-2A. But MAC PDU size wise, we do not observe clear performance gap between the two approaches (two MAC-CEs with separate functionalities, and one MAC-CE covering two functionalities), so eventually the key point is a DL assignment with sufficient space to carry the two MAC-CEs, i.e., it boils down to a restriction to network DL scheduling.

- R2 not pursue a second MAC-CE format for scenario-2A, but rely on separate MAC-CEs for OD-SSB (de)activation and SCell (de)activation.

2.2 Issue-2: Measurement of OD-SSB

127 agreed that

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

Based on the latest R1 conclusion, R1 leave L3 measurement to R2/R4, yet L1 measurement is still to be covered by R1.

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

So good to clarify that the 127 agreement is limited to L3 measurement

- R2 confirm the agreement of “Measurement based on OD-SSB in both case 1 and case 2 will be considered” is limited to L3 measurement. While for L1 measurement, it relies on R1 and R4 to conclude.

For L3 measurement, since it is possible that serving cell and neighbouring cell share the same MO configuration, e.g., in an intra-frequency scenario, it is good to understand the behavior for neighboring and serving cell scenario.

For neighbouring cell, it anyway needs to rely on SMTC window to identify the time location of SSB searching. Besides, when it comes to OD-SSB, it is anyway infeasible for UE to know the neighboring cell OD-SSB (de)activation status. Combing the two, it seems more realistic to rely on Case-2, if network would like to configure the UE to perform neighboring cell measurement. Anyway, there seems no much work that R2 can do for the neighbouring cell measurement under OD-SSB objective.

- For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work, but rely on R4 to conclude on the feasibility and detailed design.

For serving cell measurement, 127 concluded that

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

It would be helpful to firstly clarify the details of case-1, before looking into case-2.

In case-1, for the L3 measurement of serving cell based on OD-SSB, in both scenario-2 and scenario-2A.

- On the one hand, for scenario-2, where the OD-SSB before SCell activation is mainly for fast SCell activation, it is hard to restrict the L3 measurement to SMTC window, considering the SMTC window is more designed for static SSB transmission, while now the OD-SSB can be dynamically (de)activated with various periodicity (FFS other parameters), so it is hard to always restrict the serving cell L3 measurement into a configured SMTC window (with measurement cycle), i.e., cannot wait for a coming SMTC window for L3 measurement, which may further delay the SCell activation.

- On the other hand, for scenario-2/2A, after the SCell has been activated, it seems OK to limit the L3 serving cell measurement to the SMTC window as in legacy.

So, combining the two aspects together, it seems necessary at least to design a serving cell L3 measurement mechanism for OD-SSB out of SMTC window at least before SCell activation, for which the detailed scheme can be up to R4.

- R2 not pursue SMTC-based OD-SSB L3 measurement for SCell before activation, and whether to support L3 measurement for SCell before activation without relying on SMTC is up to R4.

2.2 Issue-3: UL-triggered OD-SSB

The other issue is about UL/DL triggering.

For non-collocated scenario, there would be different levels/types of backhaul connection, e.g.,

Table 1 Categorization of Backhaul Latency

| Backhaul Technology | Latency (One way) | Throughput |
| --- | --- | --- |
| Fiber Access 1 | 10-30ms | 10M-10Gbps |
| Fiber Access 2 | 5-10ms | 100-1000Mbps |
| Fiber Access 3 | 2-5ms | 50M-10Gbps |
| Fiber Access 4 (NOTE 1) | less than 2.5 us (NOTE2) | Up to 10Gbps |
| DSL Access | 15-60ms | 10-100 Mbps |
| Cable | 25-35ms | 10-100 Mbps |
| Wireless Backhaul | 5-35ms | 10Mbps – 100Mbps typical, maybe up to Gbps range |

Apparently, the latency due to non-ideal backhaul would alleviate the gain from on-demand SSB, i.e., it would slow down the overall procedure of SCell activation, and also would waste SSB transmission (the SSBs from SCell being aware of it, till UE being notified by PCell).

On the contrary, a UL-based triggering would be helpful to adapt with this kind of backhaul.

- For on-demand SSB SCell operation, R2 confirm to include the support of UL based triggering.

Conclusion

Proposal 1	RRC based OD-SSB transmission indication is used to indicate only the initial activation/deactivation state of OD-SSB configuration, but not the non-initial state.

Proposal 2	R2 not pursue a second MAC-CE format for scenario-2A, but rely on separate MAC-CEs for OD-SSB (de)activation and SCell (de)activation.

Proposal 3	R2 confirm the agreement of “Measurement based on OD-SSB in both case 1 and case 2 will be considered” is limited to L3 measurement. While for L1 measurement, it relies on R1 and R4 to conclude.

Proposal 4	For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work, but rely on R4 to conclude on the feasibility and detailed design.

Proposal 5	R2 not pursue SMTC-based OD-SSB L3 measurement for SCell before activation, and whether to support L3 measurement for SCell before activation without relying on SMTC is up to R4.

Proposal 6	For on-demand SSB SCell operation, R2 confirm to include the support of UL based triggering.