3GPP TSG-RAN WG2 Meeting #127bis	R2-2407980

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Discussion on measurement event prediction

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

At the RAN2#125bis meeting[1], RAN2 agreed on the methodology for the measurement event prediction:

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

In this contribution, we will provide further analysis of the methodology and simulation assumption for the measurement event prediction.

- Discussion

  - Methodology

2.1.1	Definition

According to RAN2#125bis meeting, at least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed. Taking RLF prediction as a reference, we may discuss indirect/direct measurement event prediction based on the agreements in previous meeting [2].

Agreements

-	Both direct and indirect are allowed.  Companies should indicate what they used and what inputs they are using

-	Output for indirect: predicted SINR.  Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

-	Companies should report the prediction time window they have used in the simulations

KPI

For indirect

-	SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction.

Direct

-	missed RLF detection, false RLF detection, F1 score.

-	Whether the RLF will happen based on the following methodology.   Above a probability threshold we assume that RLF will happen.   This is then compared with true RLF.

-	For the time being we don’t need HO procedure simulation in RLF simulation

Indirect measurement event prediction

In our understanding, similar to indirect RLF prediction, indirect measurement event prediction is based on RRM measurement prediction, i.e., the temporal domain serving cell measurement predictions(e.g. RSRP). The following Figure 2.1-1 shows the basic methodology of indirect measurement event prediction based on RRM measurement results.

Figure 2.1-1: Indirect measurement event evaluation

In details, the measurement event evaluation result is derived by evaluating the predicted RRM measurement results, e.g., RSRP or RSRQ, according to the measurement configuration, such as entering condition, leaving condition and TTT(timeToTrigger), etc. This methodology judges whether the measurement event will be fulfilled in a time instance, which is the same as one of the time instances of the predicted RSRP values. That is, no AI/ML models are introduced to further determine the time instance of prediction (except the model for RRM measurement prediction).

Hence, we propose that:

Proposal 1. The output of AI/ML Model for indirect measurement event prediction is RRM measurement prediction (e.g., RSRP). Based on RRM measurement prediction, the time instance that measurement event will be fulfilled can be determined without further AI/ML model.

Direct measurement event prediction

In direct measurement event prediction, the AI/ML model directly outputs the measurement event prediction results. The following Figure 2.1-2 shows the basic methodology of direct measurement event prediction.

Figure 2.1-2: Direct measurement event prediction

In this option, we need to consider the input and output parameters of the AI Model, which may be different from the AI Model for RRM measurement result prediction. For example, the output may be a flag of whether the measurement event is fulfilled, and the probability of measurement event within a window, etc. Furthermore, the KPI of direct measurement event prediction should be defined.

Based on the above analysis, taking advantage of the study product on RRM measurement prediction method, indirect measurement event prediction can be studied as the baseline. Although direct measurement event prediction is also allowed, extra work on dataset generation and model training is needed. Considering the workload to study both direct/indirect methods, we would like to focus on indirect measurement event prediction in the following discussion.

2.1.2	Performance KPI

Regarding the KPI of indirect measurement event prediction, we may reuse the conclusions for the case of indirect RLF prediction as much as possible to save the study effort. According to indirect RLF prediction, the KPI includes “SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction”.

Therefore, we suggest that the KPI of indirect measurement event prediction includes: RSRP difference, missed measurement event detection rate, false measurement event detection rate, F1 score, time difference between true time and predicted time, and true measurement event prediction rate.

For the KPI ‘RSRP difference’, the same KPI defined for RRM measurement prediction can be reused for indirect measurement event prediction. The KPI ‘Time difference between true time and predicted time’ can be defined as the time difference between predicted measurement event fulfillment time and actual measurement event fulfillment time. The definitions of the other performance metrics should be further clarified.

Measurement event prediction is a typical binary classification problem. When using AI/ML to address binary classification problems, it’s essential to assess their performance accurately to ensure they make reliable predictions. This is where confusion matrices and performance metrics come into play. The Confusion Matrix is shown in Table 2.1-1.

Table 2.1-1 Confusion Matrix of binary classification

| Confusion Matrix |  | True value |  |
| --- | --- | --- | --- |
|  |  | Positive | Negative |
| Predicted value | Positive’ | True Positive (TP) | False Positive (FP) |
|  | Negative’ | False Negative (FN) | True Negative (TN) |

- True Positive (TP): Instances that were correctly predicted as positive by the AI/ML model.

- False Positive (FP): Instances that were incorrectly predicted as positive by the AI/ML model (when they are, in fact, negative).

- False Negative (FN): Instances that were incorrectly predicted as negative by the AI/ML model (when they are, in fact, positive).

- True Negative (TN): Instances that were correctly predicted as negative by the AI/ML model.

The typical performance metrics for binary classification are clarified based on the confusion matrix:

- Accuracy: This metric measures the overall correctness of the AI/ML model and is calculated as (TP + TN) / (TP + TN + FP + FN).

For measurement event prediction, accuracy tells us the proportion of correctly classified instances over the total number of instances, and can be defined as true measurement event prediction rate.

- Precision: Precision quantifies the accuracy of positive predictions made by the AI/ML model. It is calculated as TP / (TP + FP).

The opposite definition of precision is false detection rate, i.e., FP / (TP + FP). For measurement event prediction, the false detection rate answers the question: Of all the instances classified as positive, how many were genuinely negative? With the false detection, too-early HO or HO to wrong cell may occur.

- Recall (Sensitivity or True Positive Rate): Recall measures the ability of the AI/ML model to identify all positive instances. It is calculated as TP / (TP + FN).

The opposite definition is missed detection rate, i.e., FN / (TP + FN). For measurement event prediction, the missed detection rate answers the question: Of all the positive instances in the dataset, how many did the AI/ML model miss the detection? Upon miss detection, too-late HO may happen.

- F1 Score: The F1 score is the harmonic mean of precision and recall, providing a balance between the two metrics. It is calculated as 2 * (Precision * Recall) / (Precision + Recall).

For measurement event prediction, F1 Score is a useful metric as there is an imbalance between positive and negative classes.

Proposal 2. The KPI of indirect measurement event prediction includes: RSRP difference, time difference between true time and predicted time, true measurement event prediction rate, false measurement event detection rate, missed measurement event detection rate, F1 score.

Proposal 3a. Capture the confusion matrix of binary classification in the TR with the clarification of TP, FP, TN and FN.

Proposal 3b. The definitions of the KPIs of indirect measurement event prediction are:

- Time difference between true time and predicted time: the time difference between predicted measurement event fulfillment time and actual measurement event fulfillment time;

- True measurement event prediction rate: the rate of correctly predicted instances over the total number of instances, i.e., (TP+TN) / (TP+TN+FP+FN);

- False measurement event detection rate: the rate of falsely predicted instances over the total number of instances predicted as positive, ie., FP / (FP+TP);

- Missed measurement event detection rate: the rate of missed instances over the total number of positive instances that are fulfilled, i.e., FN / (TP+FN);

- F1 score = 2* Precision*Recall/( Precision+ Recall).

2.1.3	Modeling of Prediction

Another aspect that will have an impact on the simulation performance is the modeling of prediction, including the inference timing and the duration to derive the event prediction.

Inference timing

In intra-frequency temporal domain case A, continuous measurement results in the prediction window are predicted by continuous historical measurement result(s) in the observation window. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. One example is illustrated in Figure 2.1-2:

Figure 2.1-3 Example of intra-frequency temporal domain case A

For the inference timing, at least the following 2 candidates can be considered:

- Option 1: Continuous inference. Consistent with Case A, a measurement event prediction result will be derived for each sampling period/measurement period;

- Option 2: Event-triggered inference. Inference is only executed when the entering condition for an event is met.

Option 1 will result in a higher number of inferences, and in most cases, the UE is located at the center of the cell and the measurement event conditions are not fulfilled. On the contrary, Option 2 can effectively reduce the number of inferences and is sufficient to avoid too-late handover. Therefore, Option 2 should be the baseline.

Proposal 4. The inference timing should be aligned in the simulation, the following two options can be considered:

- Option 1: Continuous inference. Consistent with Case A, a measurement event prediction result will be derived for each sampling period/measurement period;

- Option 2: Event-triggered inference. Model inference is only executed when the entering condition for an event is met.

The time window to derive the event prediction

Regarding the indirect measurement event prediction, our understanding is that the event prediction is derived based on the RSRP prediction instance(s) within a time window. The starting point of this time window can be in one of the following three options:

- Option 1: The starting point of the time window is a historical time, so the time window includes both historical measurements and future predictions;

- Option 2: The starting point of the time window is the current time, and the time window only includes future predictions;

- Option 3: The starting point of the time window is a future time;

Figure 2.1-4 Options of the time window to derive the event prediction

For these three options, Option 1 and Option 2 can directly reuse the pattern of Case A, while Option 3 requires additional parameters, e.g., the interval between the observation window and the prediction window.

In addition, if Option 1 is used, the duration for advanced triggering events will be shorter, and the benefits of AI/ML are not obvious. Furthermore, Option 2 can effectively reduce the probability of ping-pong handover.

Therefore, Option 2 should be the baseline.

Proposal 5. RAN2 to clarify the starting point of time window to derive the event prediction, the following three options can be considered:

- Option 1: The starting point of the time window is a historical time, so the time window includes both historical measurements and future predictions;

- Option 2: The starting point of the time window is the current time, and the time window only includes future predictions;

- Option 3: The starting point of the time window is a future time.

  - Simulation assumption

2.2.1	Common simulation assumptions

At the RAN2#126 and RAN2#127 meetings, the general simulation assumptions to acquire the dataset for RRM prediction were discussed. Similar to RLF prediction, the common simulation assumption should be reused for all use cases as much as possible.

RRM measurement prediction considers two frequency ranges (i.e., FR1 and FR2) for different purposes. Among them, the mobility performance of FR1 is already quite good, thus the main purpose of the simulation on FR1 is measurement reduction. In contrast, the mobility performance of the FR2 is greatly affected by environment changes and UE mobility, so the main purpose of the simulation on FR2 is to enhance mobility performance. Our understanding is that the main purpose of measurement event prediction is enhancing mobility performance as well, therefore, the measurement event prediction should also focus on FR2. Furthermore, measurement event prediction only needs to consider intra-frequency scenarios.

Proposal 6: For the measurement event prediction, the simulation evaluation should focus on FR2 and intra-frequency scenarios.

To reuse the simulation assumptions of RRM prediction as much as possible, the simulation assumption of ‘FR2 to FR2 intra-frequency temporal domain case A’ can be the baseline for measurement event prediction, especially for indirect measurement event prediction.

Proposal 7. Taking the simulation assumptions of RRM prediction ‘FR2 to FR2 intra-frequency temporal domain case A’ as the baseline for measurement event prediction.

2.2.2	Mobility-specific parameters

The following tables capture the additional recommended HetNet mobility-specific parameters in [3]:

Table 2.2-1: HetNet mobility specific parameters

| Items | Description |
| --- | --- |
| Pico cell placement | At fixed location(s) e.g., at 0.5 ISD, 0.3 ISD on the boresight direction. Or randomly placed. |
| Cell loading (NOTE 1) | 100%, 50% |
| UE speed | 3 km/h, 120km/h, 30km/h, 60km/h |
| Channel model | Either one of the models, TU or ITU, could be used. (fast fading included) |
| TimeToTrigger [ms] | 40, 80, 160, 480 |
| a3-offset [dB] | -1, 0, 1, 2, 3 |
| TMeasurement_Period, Intra, L1 filtering time in TS36.133 [2] | 200ms (other values could be added later) |
| Layer3 Filter Parameter K | 4, 1, 0 |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

Table 2.2-2: Summary of Mobility related simulation parameters for the MSE

| HO Parameter | Value |
| --- | --- |
| Time To Trigger (TTT) | Dynamic, 480 ms in normal Mobility |
| A3 Offset | 3 dB Macro and Pico |
| Ping-Pong-Time | 1 s |
| Measurements Rate | 0.2 s |
| HO Execution Time (including Preparation) | 0.15 s |
| RSRP error – zero mean Gaussian | 1 dB std |
| Filtering Factor K | 4 |
| RLF: Qout Threshold | - 8 dB |
| RLF: Qin Threshold | - 6 dB |

We think that most of the parameters can be reused in AI-based mobility simulation. For intra-frequency temporal domain case A, the prediction window is initially set as 400ms for FR2. To reuse the simulation result as much as possible, the value of Time to Trigger can be set close to 400ms, e.g., 320 ms or 480 ms.

Proposal 8: Take the mobility-specific simulation parameters in TR 36.839 as the baseline for measurement event prediction:

| Parameter | Value |
| --- | --- |
| A3 Offset | 3 dB |
| TimeToTrigger | 320/480 ms |
| Ping-Pong-Time/short time of stay | 1 s |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

  - Initial Simulation result

In this initial simulation, the time window to derive the event prediction is Option 2 in Figure 2.1-4 .The baseline procedure is as follows:

- When the entering condition for a measurement event is met, model inference based on the pattern of Case A is performed.

- Based on the predicted results within the prediction window, whether the measurement event is fulfilled can be determined.

The simulation results of UE speeds 60, 90, and 120 km/h are summarized in the following Table 2.3-1, Table 2.3-2, and Table 2.3-3.

Table 2.3-1 Confusion Matrix of measurement event prediction (60km/h)

| Confusion Matrix |  | True value |  |
| --- | --- | --- | --- |
|  |  | Positive (52484) | Negative (6193) |
| Predicted value | Positive’ | 51544 | 2822 |
|  | Negative’ | 940 | 3371 |

Table 2.3-2 Confusion Matrix of measurement event prediction (90km/h)

| Confusion Matrix |  | True value |  |
| --- | --- | --- | --- |
|  |  | Positive (63619) | Negative (10445) |
| Predicted value | Positive’ | 61772 | 4267 |
|  | Negative’ | 1847 | 6178 |

Table 2.3-3 Confusion Matrix of measurement event prediction (120km/h)

| Confusion Matrix |  | True value |  |
| --- | --- | --- | --- |
|  |  | Positive (69524) | Negative (12270) |
| Predicted value | Positive’ | 67258 | 5083 |
|  | Negative’ | 2266 | 7187 |

The corresponding simulation results of KPI values are summarized in Table 2.3-4:

Table 2.3-4 Simulation results of KPI of measurement event prediction

| KPI | 60km/h | 90km/h | 120km/h |
| --- | --- | --- | --- |
| True measurement event prediction rate | 93.59% | 91.74% | 91.02% |
| False measurement event detection rate | 5.19% | 6.46% | 7.03% |
| Missed measurement event detection rate | 1.79% | 2.90% | 3.26% |
| F1 score | 0.96 | 0.95 | 0.95 |

To study the impact of UE speed on performance KPI, our simulation considers the UE speed of 60 km/h, 90km/h and 120km/h. As can be seen in Table 2.3-4, the prediction with 60km/h UE speed can achieve the best KPIs. As UE speed increases from 60km/h to 120km/h, the true measurement event prediction rate decreases from 93.59% to 91.02%; the false measurement event detection rate increases from 5.19% to 7.03%; the missed measurement event detection rate increases from 1.79% to 3.26%; the overall F1 score is quite close, i.e., 0.96 and 0.95.

Observation 1: As UE speed increases from 60km/h to 120km/h, the true measurement event prediction rate decreases from 93.59% to 91.02%; the false measurement event detection rate increases from 5.19% to 7.03%; the missed measurement event detection rate increases from 1.79% to 3.26%; the overall F1 score is quite close, i.e., 0.96 and 0.95.

During the simulation process based on the agreed simulation assumptions, one issue arose: how to evaluate the gain of AI-based prediction. From our understanding, non-AI mechanisms can be compared as a benchmark. One of the simplest non-AI mechanisms is that: once the entering condition is met, the measurement event is predicted as fulfilled.

Proposal 9: To evaluate the gain of AI-based prediction, introduce a non-AI mechanism as the benchmark, e.g., once the entering condition is met, the measurement event is predicted as fulfilled. Other non-AI mechanisms are up to company’s implementation and report.

The corresponding KPIs of non-AI mechanism are as Table 2.3-5:

Table 2.3-5 Results of KPI of non-AI measurement event prediction

| KPI | 60km/h | 90km/h | 120km/h |
| --- | --- | --- | --- |
| True measurement event prediction rate | 89.45% | 85.90% | 85.00% |
| False measurement event detection rate | 10.55% | 14.10% | 15.00% |
| Missed measurement event detection rate | 0.00% | 0.00% | 0.00% |
| F1 score | 0.94 | 0.92 | 0.92 |

It can be absolved that the true measurement event prediction rate and false measurement event detection rate of the non-AI mechanism are worse than that of AI-based prediction. However, the missed measurement event detection rate of non-AI mechanism is 0 since there is no event is missed. And the overall F1 score also decreases, i.e., 0.94 and 0.92.

In general, the AI-based prediction can balance the false measurement event detection rate and missed measurement event detection rate compared with legacy HO. Besides, AI-based prediction can achieve better overall true measurement event prediction rate and F1 score.

Observation 2: Compared with non-AI mechanism, the AI-based prediction can balance the false measurement event detection rate and missed measurement event detection rate. Besides, AI-based prediction can achieve better overall performance, i.e., with higher true measurement event prediction rate and F1 score.

From our understanding, the above KPIs are intermediate KPIs and can be utilized to compare the performance of AI-based and non-AI-based solutions. However, if the system-level performance is not considered, the impact of introducing AI/ML for mobility cannot be fully assessed.

For the specific system-level performance KPI, we suppose most of the mobility-related performance KPIs listed in the SID can be considered, such as Ping-pong HO, HOF, Time of stay, Handover interruption, prediction accuracy, measurement reduction, etc. As to the RLF frequency, we think it’s not needed in measurement event prediction as quite few RLF may occur during our simulation.

As to the definition, the following KPIs are captured in TR 36.839 and can be reused:

- Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

- A Short ToS rate is defined as the number of Short ToS occurrences divided by the number of successful handovers. I.e., Short ToS rate = (number of Short ToS occurrences)/(total number of successful handovers).

- The HOF rate is defined as Handover failure rate = (number of handover failures) / (Total number of handover attempts).

For the Handover interruption, modeling the detailed RACH procedure and the corresponding interruption during handover presents a complex challenge for simulation. Assuming that each handover procedure may introduce close interruption, one simple approach is to introduce an equivalent KPI, such as handover frequency, which can be defined as the number of HO occurrences per UE per second.

Based on the above analysis, we provide the system-level performance evaluation results for UE speed of 60 km/h, 90 km/h and 120 km/h. In the simulation, three types of HO mechanism are considered:

- Set 1: legacy HO with TTT = 0ms (i.e., non-AI mechanism in Proposal 9);

- Set 2: legacy HO with TTT = 320ms;

- Set 3: AI-based HO with measurement event prediction

The simulation results of each set are summarized in the following Table 2.3-6. The HOF rate and Short-ToS rate are further illustrated in Figure 2.3-1 and Figure 2.3-2.

Table 2.3-6: Simulation results of system-level KPI for each parameter set

| Parameters | KPI | 60km/h | 90km/h | 120km/h |
| --- | --- | --- | --- | --- |
| Set 1: Legacy HO TTT = 0ms | HOF rate | 2.3% | 3.2% | 5.2% |
|  | Ping-pong rate | 18.1% | 22.0% | 30.1% |
|  | Short ToS rate | 48.6% | 61.6% | 65.3% |
|  | handover interruption | 0.41/UE/s | 0.49/UE/s | 0.59/UE/s |
| Set 2: Legacy HO TTT = 320ms | HOF rate | 8.7% | 10.7% | 11.7% |
|  | Ping-pong rate | 8.0% | 8.3% | 8.5% |
|  | Short ToS rate | 36.9% | 42.2% | 43.5% |
|  | handover interruption | 0.30/UE/s | 0.32/UE/s | 0.32/UE/s |
| Set 3: AI-based HO with measurement event prediction | HOF rate | 3.4% | 4.2% | 5.5% |
|  | Ping-pong rate | 11.3% | 18.6% | 21.4% |
|  | Short ToS rate | 41.5% | 48.2% | 51.1% |
|  | handover interruption | 0.36/UE/s | 0.40/UE/s | 0.49/UE/s |

Figure 2.3-1: HOF rate of legacy HO and AI-based HO

Figure 2.3-2: Short-ToS rate of legacy HO and AI-based HO

With system-level performance, it can be observed that the AI-based HO can balance the HOF rate and Short-ToS rate compared with legacy HO.

Observation 3: Compared with legacy HO, The AI-based HO relies on measurement event prediction can balance the HOF rate and Short-ToS rate.

Proposal 10: System-level performance evaluation is needed to reflect the impact on mobility performance after measurement event prediction is introduced.

Proposal 11: The system-level performance KPIs for measurement event prediction include Ping-pong HO rate, short ToS rate, HOF rate, and handover interruption.

Proposal 12a: Reuse the definition of the following system-level performance KPIs in 36.839:

- Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

- Short ToS rate is defined as the number of Short ToS occurrences divided by the number of successful handovers. I.e., Short ToS rate = (number of Short ToS occurrences)/(total number of successful handovers).

- HOF rate is defined as Handover failure rate = (number of handover failures) / (Total number of handover attempts).

Proposal 12b: The KPI of handover interruption can be reflected as HO occurrences per UE per second.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Methodology

Proposal 1. The output of AI/ML Model for indirect measurement event prediction is RRM measurement prediction (e.g., RSRP). Based on RRM measurement prediction, the time instance that measurement event will be fulfilled can be determined without further AI/ML model.

Proposal 2. The KPI of indirect measurement event prediction includes: RSRP difference, time difference between true time and predicted time, true measurement event prediction rate, false measurement event detection rate, missed measurement event detection rate, F1 score.

Proposal 3a. Capture the confusion matrix of binary classification in the TR with the clarification of TP, FP, TN and FN.

Proposal 3b. The definitions of the KPIs of indirect measurement event prediction are:

- Time difference between true time and predicted time: the time difference between predicted measurement event fulfillment time and actual measurement event fulfillment time;

- True measurement event prediction rate: the rate of correctly predicted instances over the total number of instances, i.e., (TP+TN) / (TP+TN+FP+FN);

- False measurement event detection rate: the rate of falsely predicted instances over the total number of instances predicted as positive, ie., FP / (FP+TP);

- Missed measurement event detection rate: the rate of missed instances over the total number of positive instances that are fulfilled, i.e., FN / (TP+FN);

- F1 score = 2* Precision*Recall/( Precision+ Recall).

Proposal 4. The inference timing should be aligned in the simulation, the following two options can be considered:

- Option 1: Continuous inference. Consistent with Case A, a measurement event prediction result will be derived for each sampling period/measurement period;

- Option 2: Event-triggered inference. Model inference is only executed when the entering condition for an event is met.

Proposal 5. RAN2 to clarify the starting point of time window to derive the event prediction, the following three options can be considered:

- Option 1: The starting point of the time window is a historical time, so the time window includes both historical measurements and future predictions;

- Option 2: The starting point of the time window is the current time, and the time window only includes future predictions;

- Option 3: The starting point of the time window is a future time.

Simulation assumption

Proposal 6: For the measurement event prediction, the simulation evaluation should focus on FR2 and intra-frequency scenarios.

Proposal 7. Taking the simulation assumptions of RRM prediction ‘FR2 to FR2 intra-frequency temporal domain case A’ as the baseline for measurement event prediction.

Proposal 8: Take the mobility-specific simulation parameters in TR 36.839 as the baseline for measurement event prediction:

| Parameter | Value |
| --- | --- |
| A3 Offset | 3 dB |
| TimeToTrigger | 320/480 ms |
| Ping-Pong-Time/short time of stay | 1 s |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

Initial Simulation result

Observation 1: As UE speed increases from 60km/h to 120km/h, the true measurement event prediction rate decreases from 93.59% to 91.02%; the false measurement event detection rate increases from 5.19% to 7.03%; the missed measurement event detection rate increases from 1.79% to 3.26%; the overall F1 score is quite close, i.e., 0.96 and 0.95.

Observation 2: Compared with non-AI mechanism, the AI-based prediction can balance the false measurement event detection rate and missed measurement event detection rate. Besides, AI-based prediction can achieve better overall performance, i.e., with higher true measurement event prediction rate and F1 score.

Observation 3: Compared with legacy HO, The AI-based HO relies on measurement event prediction can balance the HOF rate and Short-ToS rate.

Proposal 9: To evaluate the gain of AI-based prediction, introduce a non-AI mechanism as the benchmark, e.g., once the entering condition is met, the measurement event is predicted as fulfilled. Other non-AI mechanisms are up to company’s implementation and report.

Proposal 10: System-level performance evaluation is needed to reflect the impact on mobility performance after measurement event prediction is introduced.

Proposal 11: The system-level performance KPIs for measurement event prediction include Ping-pong HO rate, short ToS rate, HOF rate, and handover interruption.

Proposal 12a: Reuse the definition of the following system-level performance KPIs in 36.839:

- Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

- Short ToS rate is defined as the number of Short ToS occurrences divided by the number of successful handovers. I.e., Short ToS rate = (number of Short ToS occurrences)/(total number of successful handovers).

- HOF rate is defined as Handover failure rate = (number of handover failures) / (Total number of handover attempts).

Proposal 12b: The KPI of handover interruption can be reflected as HO occurrences per UE per second.

- Reference

- R2_125bis_ChairNotes

- R2_127_ChairNotes

- TR 36.839		Mobility enhancements in heterogeneous networks