3GPP TSG-RAN WG2 Meeting #127bis	R2-2407981

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Discussion on reply LS on the maximum number of devices supported in SLPP

Agenda Item:	7.2.1

Document for:	Discussion and Decision

- Introduction

At RAN2#126 meeting, LS on the maximum number of devices supported in SLPP is sent to CT1/CT4 for alignment [1].

1. Overall Description: RAN2 would like to inform that the maximum number of devices that is supported in the SLPP messages ProvideLocationInformation and ProvideAssistanceData is 256; for the other SLPP messages, the maximum number of devices that is supported is 1. 2. Actions: To CT1, CT4: ACTION: RAN2 respectfully asks CT1 and CT4 to take the above into account in the future work.

In the reply LS [2], CT1 provided the following feedback:

For ranging and sidelink positioning, the information transmitted over NAS via payload container IE has the limitation of a maximum length of 65536 octets from CT1 protocol point of view (the maximum size of a NAS message carrying payload container IE is specified in subclause 7.2.2 of TS 24.501 for further information). The related information includes e.g. SLPP messages, the result of the positioning request. So CT1 finds out that: - The payload container IE may not be able to encapsulate a complete SLPP message with information for other UEs (UE2-UEn) with the maximum number of devices is 256 as defined by RAN2. - The payload container IE may also not be able to encapsulate the absolute location result for other UEs (UE2-UEn) with the maximum number of devices is 256. This limitation also applies to the messages exchanged between UEs. To address the above issue, for RSPP supplementary messages, CT1 evaluated and achieved the consensus that the supported maximum number of other UEs (UE2-UEn) shall not be greater than 63 in R18 (see attachment), considering the associated information to be exchanged between UEs or between UE1 and LMF for ranging and sidelink positioning. 2. Actions: To RAN2 ACTION: CT1 kindly asks RAN2 to take above information into account, update the corresponding specification if needed, and keep CT1 informed of the final decision.

In this contribution, we intend to further discuss the alignment and try to draft a reply LS from RAN2 perspective.

- Discussion

As indicated in the LS from CT1, the supported maximum number of other UEs (UE2-UEn) shall not be greater than 63 in R18. Correspondingly, the maximum number of SLPP containers of other UEs (UE2-UEn) is updated as 63 in TS 24.080 [3].

LCS-DLRSPPTransportArg ::= SEQUENCE { rangingSLPPList [0] RangingSLPPList OPTIONAL, scheduledLocTime [1] DateTime OPTIONAL, ueBased [2] UEBased OPTIONAL, relatedUEInfo [3] RelatedUEInfo OPTIONAL, ...} LCS-ULRSPPTransportArg ::= SEQUENCE { rangingSLPPList [0] RangingSLPPList OPTIONAL, ...} RangingSLPPList ::= SEQUENCE (SIZE (1..maxNumSLPPMsg)) OF RangingSLPPInfo maxNumSLPPMsg INTEGER ::= 63

In downlink positioning based on PRS, the maximum number of TRPs to transmit PRS is 256. RAN2's initial consideration is that the number of anchor UEs can be the same as the number of TRPs, i.e., 256.

CT1's new limitation on the number of other UEs (UE2-UEn) is mainly out of the capacity for PC5 connections between UEs and the maximum number of NAS message bearers between UE and LMF. We think this limitation is reasonable, therefore we recommend that RAN2 align with CT1's conclusion, clarifying in the SLPP protocol that the maxNrOfUEs is 63.

Since the SLPP specification has been frozen, the most simple way to make such clarification is to add a note that the maximum number of Tx UEs or Rx UEs is 63 in this version of specification.

Proposal 1: RAN2 to align with CT1 on the maximum number of other UEs (UE2-UEn) in SLPP specification as 63.

Proposal 2: Adding a note in the SLPP spec that ‘the Max number of Tx UEs or Rx UEs is 63 in this version of specification’. Adopt TP in Annex 1.

Since CT1 requests RAN2 to keep CT1 informed of the final decision, we therefore propose that:

Proposal 3: Adopt Draft Reply LS to CT1 in Annex2.

- Conclusion

Proposal 1: RAN2 to align with CT1 on the maximum number of other UEs (UE2-UEn) in SLPP specification as 63.

Proposal 2: Adding a note in the SLPP spec that ‘the Max number of Tx UEs or Rx UEs is 63 in this version of specification’. Adopt TP in Annex 1.

Proposal 3: Adopt Draft Reply LS to CT1 in Annex 2.

- Reference

- R2-2405988	LS on the maximum devices supported in SLPP

- C1-245040	Reply LS on the maximum number of devices supported in SLPP

- TS 24.080 V18.5.0	Supplementary services specification; Formats and coding

# Annex 1 - TP for P2

## 6.4	Multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxNrOfUEs                                  INTEGER ::= 256        -- Max number of Tx UEs or Rx UEs

-- Note: the Max number of Tx UEs or Rx UEs is 63 in this version of specification.

nrMaxBands                                  INTEGER ::= 1024       -- Maximum number of supported bands in UE capability

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

# Annex 2 – Draft reply LS

3GPP TSG-RAN2 Meeting #127bis	R2-24xxxxxx

Hefei, China, Oct 14th – 18th, 2024

Title:	DRAFT Reply LS on the maximum number of devices supported in SLPP

Response to:	R2-24xxxxx(was C1-245040)

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	vivo [to be TSG RAN2]

To:	CT1

Cc:	CT4, SA2

Contact Person:

Name:

E-mail Address:

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks CT1 for the Reply LS on the maximum number of devices supported in SLPP.

RAN2 will align with CT1 on the maximum number of devices in SLPP specification as 63.

2. Actions:

To RAN1

ACTION: 	RAN2 kindly asks CT1 to take the above information into consideration.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #128	18 – 22 November 2024		Orlando, US